HUD Survey Instructions and Report

U.S. Department of Housing	                                  OMB Approval No. 0000-0000

and Urban Development				     (Exp. 00/00/0000)

Office of Housing

Public Reporting Burden for this collection of information is estimated to average 0.5 hour per response, including the time for reviewing instructions, searching existing data sources, gathering and maintaining the data needed, and completing and reviewing the collection of information.  Send comments regarding this burden estimate or any other aspect of this collection of information, including suggestions for reducing this burden, to the Reports Management Officer, Office of Information Policies and Systems, U.S. Department of Housing and Urban Development, Washington, DC 20410-3600 and to the Office of Management and Budget, Paperwork Reduction Project (2502-0468), Washington, DC  20503.  Do not send this completed form to either of the above addresses.

This survey is to be used in a multifamily housing loan transaction submitted to HUD.

Its uses will include:

Land title recordation (all cases).

Site grading plan preparation (item 1 below).

Plot plan design/redesign (item 2 below).

Special Project Features:

C  Condominium/Air-rights, and/or

Other: (Specify)

Standards of Performance: In every instance the survey and map(s) and/or plat(s) must be made in accordance with the requirements for an “ALTA/ACSM Land Title Survey” and in compliance with the:

▪    2011 Minimum Standard Detail Requirements for ALTA/ACSM Land Title Surveys, jointly established and adapted by the American Land Title Association and the National Society of Professional Surveyors;

▪   Table A, Optional Survey Responsibilities and Specifications,

thereof, items 1, 2, 3, 4, 6a, 6b, 7a, 8, 9, 10a, 10b, 11b, 12, 13, 16, 17, 18, 19, and 20a;

▪   And the following requirements as applicable:

1. Site Grading Involved: Comply with table A, item 5. Contours

may not exceed 1-foot vertical intervals, except that 2-foot and

5-foot vertical intervals may be used where the mean site gradient exceeds 5 percent and 10 percent respectively. Where curbs and/ or gutters exist, show top of curb and flow line elevations.

2. Plot Plan Design/Redesign Involved: Comply with Table A,

Item 6.

3.

4.

5.

Condominium/Air-rights Involved: The surveyor must provide a survey made in accordance with any Property Jurisdiction requirements or, in the absence of such requirements, professionally recognized standards.

Flood Hazard Involved: Where any portion of the site is subject to flood hazard, show the 100-year return frequency flood hazard elevation and flood zone for all projects plus the 500 year return frequency flood hazard elevation and flood zone for Section 811 housing program.  For existing projects show the site elevation at the building entrances, lowest habitable finished floor, and basement for each primary building, and the vehicular parking area that serves each primary building. Take return frequency flood hazard elevations from the applicable Federal Flood Insurance Rate Map. Where such is not available, take the elevations from available state or local equivalent data, or  when not available, work in conjunction with owner's engineer.

Blanket Easement Involved. Show on the map/plat the location of any facility that is located within or traverses the property under provisions of a blanket easement.

Additional Owner Requirements: The following requirements are not intended to void any other part of this instruction.

________________________________________________________________________________________________________

___________________

Owner's Representative / Contact:

Name & Phone No: ___________________________________________________________________________________________________________

Address: ___________________________________________________________________________________________________________________

Surveyor's Report: A current Surveyor's Report (not more than 120 days old) must be included with the survey map(s)/plat(s) submitted to HUD for: project design review, construction contract document sets, as required during construction, upon project completion; and with the map(s)/plat(s) used at initial and final closing.

Certification: The survey map/plat must bear the following certification:

“I hereby certify to HUD, (Borrower), (Sponsor), (Lender), (Title Insurance Underwriter), (Other), and to their successors and assigns, that:

“I made an on the ground survey per record description of the land shown hereon located in (city or town, county, township,

etc.), on (date); and that it and this (these) map(s) was (were) made in accordance with the HUD Survey Instructions and Report, HUD 92457M, and the requirements for an ALTA/ACSM Land Title Survey, as defined in the 2011 Minimum Standard Detail Requirements for ALTA/ACSM Land Title Surveys.

“To the best of my knowledge, belief and information, except as shown hereon: There are no encroachments either way across property lines; title lines and lines of actual possession are the same; and the premises are [free of any] [subject to a] 100/500 year return frequency flood hazard, and such [flood free] [flood] condition is shown on the Federal Flood Insurance Rate Map, Community Panel No. ( if none, so state).”