OMB Control Number: 0690-0030

Expiration Date: 04/30/2014

Examples of Long-Range Probabilistic Hydrologic Outlooks

Long-range hydrologic outlooks contain probability values that are calculated using current conditions of the river with multiple season scenarios from 30 or more years of historical data.  Current conditions include height or flow of the river, estimated water content of any existing snow cover and soil moisture.  Additionally, 30 to 90 day long-range outlooks of temperature and precipitation may also be considered. By providing the complete range of probabilities, the level of risk associated with long-range planning decisions can be determined. These probabilistic outlooks are part of the National Weather Service's Advanced Hydrologic Prediction Service (AHPS).

The flood categories used by the National Weather Service are as follows:

Minor flooding - when the river reaches flood stage and causes minimal or no property damage, but possibly some public threat.

Moderate flooding – when river levels cause some inundation of structures and roads near stream with some evacuations of people and/or transfer of property to higher elevations.

Major flooding – when river levels cause extensive inundation of structures and roads with significant evacuations of people and/or transfer of property to higher elevations.

Three examples follow that use various customer input received over the last several years.  Graphical representations follow the text examples.*

In Example 1, the probabilistic hydrologic outlook contains two tables, the first table provides information on the probability of reaching minor, moderate and major flooding, and the second table provides information on the probability (at 10 % intervals) of exceeding a given stage at a given location over the next 90 days.

Example 1

THIS OUTLOOK IS VALID FROM FEB 1 2011 – MAY 2 2011

THE FOLLOWING TABLE GIVES THE CHANCE OF REACHING DIFFERENT

FLOOD LEVELS AT THE GAGES DURING THE VALID PERIOD.

...TABLE 1--PROBABILITIES FOR MINOR...MODERATE AND MAJOR

FLOODING...

APPROXIMATE CHANCE OF

FLOOD LEVELS (FT)       REACHING FLOOD LEVELS

LOCATION          MINOR  MOD  MAJOR       MINOR   MOD   MAJOR

--------          ----- ----- -----       -----  -----  -----

MISSISSIPPI RIVER

DUBUQUE LD11 IA  16.0  17.0  20.5        98%    93%    67%

DUBUQUE IA       17.0  18.0  21.5        98%    98%    72%

BELLEVUE LD12 IA 17.0  18.0  20.0        88%    86%    65%

IN THE TABLE BELOW...THE 90 THROUGH 10 PERCENT COLUMNS INDICATE THE

CHANCE THE RIVER COULD RISE ABOVE THE LISTED STAGE LEVELS IN THE

NEXT 90 DAYS.

...TABLE 2--EXCEEDANCE PROBABILITIES...

CHANCE OF EXCEEDING STAGES AT SPECIFIC LOCATIONS

LOCATION        FS(FT) 90%  80%  70%  60%  50%  40%  30%  20%  10%

-----------     ----   ---  ---  ---  ---  ---  ---  ---  ---  ---

MISSISSIPPI RIVER

DUBUQUE LD11    16.0  17.4 19.1 20.3 21.5 22.2 23.2 24.0 24.7 26.2

DUBUQUE         17.0  19.0 20.7 21.9 22.9 23.6 24.7 25.4 26.3 29.3

BELLEVUE LD12   17.0  16.9 18.6 19.8 20.7 21.3 22.0 22.6 22.8 23.8

In Example 2, the probabilistic hydrologic outlook contains three tables, the first table (1A) provides information on the probability of reaching minor, moderate and major flooding in a slightly different format than used in Example 1; the second table (1B) provides additional information on the departure from normal of reaching minor, moderate, and major flooding; and the third table (2) provides the probability (at 10 % intervals) of exceeding a given stage at a given location, similar to Example 1.

The main difference between this example and the previous example is the information in the second table (1B), which shows how the flood threat deviates from normal for minor, moderate, and major flooding.

Example 2

...TABLE 1A--PROBABILITIES FOR MINOR...MODERATE AND MAJOR FLOODING...

VALID  2/1/2011 - 5/2/2011

-----------FLOOD STAGE-----------

LOCATION             MINOR          MODERATE        MAJOR

STG   PCT      STG   PCT      STG   PCT

MISSISSIPPI RIVER

DUBUQUE LD11       16.0  98%      17.0  93%      20.5  67%

DUBUQUE            17.0  98%      18.0  98%      21.5  72%

BELLEVUE LD12      17.0  88%      18.0  86%      20.0  65%

...TABLE 1B--DEPARTURES FROM NORMAL OF REACHING MINOR...MODERATE

AND MAJOR FLOOD STAGE...

VALID  2/1/2011 - 5/2/2011

-----------FLOOD STAGE-----------

LOCATION              MINOR            MODERATE         MAJOR

STG   DEPARTURE  STG   DEPARTURE  STG   DEPARTURE

MISSISSIPPI RIVER

DUBUQUE LD11       16.0  53% ABOVE  17.0  56% ABOVE  20.5  60% ABOVE

DUBUQUE            17.0  51% ABOVE  18.0  58% ABOVE  21.5  65% ABOVE

BELLEVUE LD12      17.0  58% ABOVE  18.0  67% ABOVE  20.0  59% ABOVE

NR NORMAL = WITHIN 5% OF NORMAL

...TABLE 2--EXCEEDANCE PROBABILITIES... (same as Ex. 1 Table 2)

CHANCE OF EXCEEDING STAGES AT SPECIFIC LOCATIONS

VALID  2/1/2011 - 5/2/2011

LOCATION          FS(FT) 90%  80%  70%  60%  50%  40%  30%  20%  10%

MISSISSIPPI RIVER

DUBUQUE LD11     16.0  17.4 19.1 20.3 21.5 22.2 23.2 24.0 24.7 26.2

DUBUQUE          17.0  19.0 20.7 21.9 22.9 23.6 24.7 25.4 26.3 29.3

BELLEVUE LD12    17.0  16.9 18.6 19.8 20.7 21.3 22.0 22.6 22.8 23.8

In Example 3 the probabilistic hydrologic outlook contains only two tables, the first table (1) combines information on the probability of reaching minor, moderate and major flooding in a similar format to what was used in example 2(1A) above, but with only information on the departure from normal of reaching flood stage; and the second table (2) provides the probability of exceeding a given stage at a given location but uses different and fewer percentage intervals than what was used in examples 1 and 2.

The main difference in this example is that it only gives you departure from normal for reaching flood stage (not for moderate or major flooding), and this example uses different, and fewer, percentage intervals for exceedance probabilities.

Example 3

CHANCE OF EXCEEDING MINOR TO MAJOR FLOOD STAGE

DEVIATION FROM NORMAL FLOOD RISK

VALID PERIOD: 2/1/2011 - 5/2/2011

...TABLE 1--PROBABILITIES FOR MINOR...MODERATE AND MAJOR

FLOODING...

DEP FROM

---------FLOOD STAGES---------      NORMAL

MINOR     MODERATE    MAJOR     OF REACHING

LOCATION               STG  PCT   STG  PCT   STG  PCT   FLOOD STAGE

----------------------------------------------------------------------

MISSISSIPPI RIVER

DUBUQUE LD11           16   98%   17   93%   21   67%   ABOVE NORMAL

DUBUQUE                17   98%   18   98%   22   72%   ABOVE NORMAL

BELLEVUE LD12          17   88%   18   86%   20   65%   ABOVE NORMAL

...TABLE 2--EXCEEDANCE PROBABILITIES...

CHANCE OF EXCEEDING STAGES AT SPECIFIC LOCATIONS

VALID PERIOD: 2/1/2011 - 5/2/2011

95%    90%    75%    50%    25%    10%     5%

---    ---    ---    ---    ---    ---    ---

MISSISSIPPI RIVER

DUBUQUE LD11    16.5   17.4   19.1   22.2   24.5   26.2   27.3

DUBUQUE         18.1   19.0   21.1   23.6   26.0   29.3   31.5

BELLEVUE LD12   16.3   16.9   18.7   21.3   22.8   23.8   24.7

LEGEND

STG = STAGE (FEET)

PCT = PERCENT

--- = DISTRIBUTION COMPLETELY BELOW THIS STAGE

DEP = DEPARTURE

NEAR NORMAL = 0.5 - 1.5 times normal

BELOW NORMAL = 0.25 - 0.50 times normal

MUCH BELOW NORMAL = <0.25 times

ABOVE NORMAL = >1.5 - 3.0 times normal

MUCH ABOVE NORMAL = > 3.0 times normal

* Hydrographs for reference: