Infrastructure Survey

Central America Water and Sanitation Program Sustainability Evaluation:

Infrastructure Sanitary Survey (Feb 2012)

Central America – El Salvador, Guatemala, Honduras, Nicaragua

Water System Performance/Design

1. Is the system working? (i.e., water coming out of taps or pump?)		YES		NO

2. If working, how many hours per day is water delivered to taps:

O	16 - 24 hours			O  other: ________________

O	8 –16 hours

O	8 –16 hours

O	less than 8 hours

O	NA (on demand hand pump system)

3. Functionality of components:

a. Catchment structure or well:

All valves function well? (test them)		YES		NO		NA

Cracks or leaks in structures?			YES		NO		NA

Clean inside spring box or behind dam?	YES		NO		NA

Catchment or well structures fenced in?	YES		NO		NA

Are there latrines or other sources of 		YES		NO		NA

contamination (livestock, cesspools, etc.) within 100 feet of the catchment or well?

If so, what and how far away?

For springs and dams:

What is upstream of the catchment structure? (forest, grazing land, houses, roads, etc.)

For wells:

Hand pump function well? (test it)		YES		NO		NA

Well casing extend 18” above ground or	YES		NO		NA

normal flood level?

Top of well casing sealed from surface	YES		NO		NA

water, rain water, or contaminants?

Is the well sealed at the ground surface?	YES		NO		NA

(i.e, can surface water, rain water, or contaminants enter the well at the ground surface?

b. Conduction line from source to tank:

Leaks in pipes or joints?			YES		NO		NA

Exposed PVC pipe in line?			YES		NO		NA

Clean out valves and air valves working?	YES		NO		NA

c. Storage Tank:

All valves function well?			YES		NO		NA

Cracks or leaks in structure?			YES		NO		NA

Clean inside?					YES		NO		NA

Hatch covers in good shape?			YES		NO		NA

Covers locked?				YES		NO		NA

Tank fenced in?				YES		NO		NA

d. Treatment System:

Is there any treatment system?		YES		NO		NA

If yes, what type:

O	sedimentation tank

O	chlorination

O	other (specify): _________________

Is it working?					YES		NO

e. Distribution Network:

Are there leaks in distribution network?	YES		NO		NA

Are there leaks in domestic connections?	YES		NO		NA

Does water arrive at all taps?			YES		NO		NA

4. Does the design of the system match the design information given to you by the watsan delegate?

YES		NO

If no, describe the differences and their significance.

Sanitation Facility Performance/Design

(to be answered by visiting a sample of the latrines constructed in each community)

5. Given the type of latrine constructed, was it built properly?	YES 		N0

Type_____________________________

comments (especially if improperly constructed):

6. Is it being operated properly?					YES 		NO

comments (especially if improperly operated):

7. Does the design match the design information given to you by the watsan delegate?	YES	NO

If no, describe the differences and their significance.

8. Is the water system new (constructed after Mitch) or re-built (existing water system, repaired after Mitch?									New 		Re-built