1

Federal Railroad Administration

Office of Safety

1

HIGHWAY-RAIL CROSSING INVENTORY

INSTRUCTIONS AND PROCEDURES MANUAL

December 1996

2

REGISTRATION FORM

Please register my name as a holder of the "Highway-Rail Crossing Inventory - Instructions and Procedures Manual."

By returning this Registration Form, I will receive all future revisions to this manual.

(PLEASE TYPE OR PRINT)

NAME

RAILROAD/STATE AGENCY

DEPT./UNIT/BUREAU

ADDRESS

P.O. BOX

CITY, STATE, ZIP CODE

BUSINESS TELEPHONE NO.

(Area Code)

Please complete and return this form to:

Federal Railroad Administration

Office of Safety (RRS-23)

Highway-Rail Crossing and Trespasser Programs Division

400 7th Street, S.W.

Washington, D.C.  20590

HIGHWAY-RAIL CROSSING INVENTORY

INSTRUCTIONS AND PROCEDURES MANUAL

for the

Federal Railroad Administration

Highway-Rail Crossing Inventory

Data Maintenance Program

December 1996

prepared for:

U.S. Department of Transportation

Federal Railroad Administration

Office of Safety

Highway-Rail Crossing and Trespasser Programs Division

400 Seventh St. S.W.

Washington, D.C. 20590

by:

AMB Associates, Inc.

818 Roeder Road, Suite 500

Silver Spring, Maryland  20910

CONTENTS

Page

1.0  INTRODUCTION	1-1

1.1  Purpose	1-1

1.2  Goal	1-1

1.3  Project History	1-1

1.4  Data Files	1-3

1.5  Definitions of Highway-Rail Intersections	1-4

2.0  HIGHWAY-RAIL CROSSING INVENTORY NUMBER	2-1

2.1  Background	2-1

2.2  Uniqueness and Calculation	2-2

2.3  Assignment of Numbers	2-5

2.4  Number Boards	2-7

2.4.1  Specifications	2-7

2.4.2  Installation and Display	2-9

2.4.3  Maintenance	2-10

3.0  U.S. DOT-AAR CROSSING INVENTORY FORM

RECORDING INSTRUCTIONS	3-1

3.1  Recording Instructions	3-1

3.2  U.S. DOT-AAR Crossing Inventory Form Heading	3-1

3.3  Part I  Location and Classification of All Crossings	3-2

3.4  Part II  Detailed Information for Public Vehicular at Grade Crossing	3-6

3.5  Part III  Physical Data	3-10

3.6  Part IV  Highway Department Information	3-12

4.0  INVENTORY UPDATE PROCEDURES	4-1

4.1  General	4-1

4.2  Inventory Form	4-3

4.3  Data Items	4-3

4.4  Railroad and State Agency Update Submission Procedures	4-6

4.5  Summary of Completed Inventory Form Handling Procedures	4-10

4.6  One-party Submissions	4-11

4.7  Inventory Computer Printout Mark-up	4-12

4.8  Overview of Update Processing at FRA	4-12

5.0  MASS UPDATING	5-1

5.1  Fill-in-the-Blanks Computer Printout	5-1

5.1.1  Train Counts and Train Speeds Updating	5-4

5.1.2  Signs, Markings, and Part IV State Highway Information	5-7

5.1.3  Other Examples	5-9

5.2  Mass Update Requirements	5-9

5.3  Special Procedures for Certain Items	5-11

5.4  Feedback of Updated Records from the National File	5-11

5.5  Continuous Feed Update Forms	5-13

5.6  Common Errors in Updating	5-13

5.7  Handling of Errors on Updates	5-15

6.0  MAGNETIC TAPE	6-1

7.0  GX DATA MANAGEMENT SYSTEM	7-1

7.1  Installation Program	7-2

7.2  Browse Facility	7-3

7.3  Update Facility	7-3

7.4  Report Facility	7-3

7.5  Data Extraction Facility	7-3

7.6  Backup and Restore Facility	7-4

7.7  Documentation	7-4

8.0  SYSTEM OUTPUTS	8-1

9.0  DOT HIGHWAY-RAIL CROSSING ACCIDENT PREDICTION

AND RESOURCE ALLOCATION PROCEDURE	9-1

9.1  Background	9-1

9.2  DOT Accident Prediction Formula	9-2

9.3  Resource Allocation Model	9-6

9.4  Data Provided to States and Railroads	9-8

9.5  Performance Compared to Other Models	9-10

10.0  ADDITIONAL INFORMATION AND REFERENCES	10-1

10.1  Highway-Rail Grade Crossing Handbook	10-1

10.2  Worldwide Geographic Location Codes	10-1

10.3  Federal-Aid Policy Guide	10-2

APPENDIX A - STATE CONTACTS	A-1

APPENDIX B - COMPUTER PRINTOUT FORMAT SAMPLES	B-1

APPENDIX C - FRA MAGNETIC TAPE FORMAT	C-1

APPENDIX D - GX DATA FILE STRUCTURE	D-1

APPENDIX E - RAILROAD CODES	E-1

1.0  INTRODUCTION

1.1  Purpose

The purpose of this manual is to set forth the instructions and procedures to provide a useful, up-to-date and accurate data base for the National Highway-Rail Crossing Inventory Data File maintained by the Federal Railroad Administration (FRA) for use by States and railroads.

The procedures for updating the National Highway-Rail Crossing Inventory Data File in this manual are applicable upon completion of the basic inventory, they are to be used for providing data to the FRA and they may be used by States and railroads for maintaining separate files.

This manual is a combination of all previous procedures and update manuals published since 1974 and other instructions periodically prepared.

1.2  Goal

The major goal of the National Highway-Rail Crossing Inventory Program is to provide information to Federal, State, and local governments as well as the railroad industry for the improvement of safety at highway-rail crossings.  Good management practices necessitate maintaining the data base on a current basis.  The data will continue to be useful only if maintained and updated as inventory changes occur.

The Federal-Aid Highway Act of 1973 (Section 203) required that each State highway agency maintain an inventory of all crossings.  According to the implementing instructions contained in the Federal-Aid Policy Guide (FAPG), maintaining the National Inventory will satisfy the legislative requirement for a State inventory (23 CFR Part 924 (a) (1)).  A primary purpose of the National Inventory is to provide for the existence of a uniform inventory data base which can be merged with accident files and used to analyze information for planning and implementation of crossing improvement programs by public and private agencies responsible for highway-rail crossing safety.

1.3  Project History

In August, 1972, the U.S. Department of Transportation submitted a report to Congress entitled: Railroad-Highway Safety Part II: Recommendations For Resolving The Problem.  The primary goal of this report was to provide recommendations for alternative courses of action which would lead to a significant reduction in accidents, fatalities, personal injuries and property damage at highway-rail crossings.

The report recommended the development of an adequate information system.  Although various local, State, and Federal agencies had collected and maintained information about highway-rail crossings, most crossing information systems were fragmented and incomplete.

Certain site-specific information was necessary to provide for a systematic approach to the planning and evaluation of programs for the improvement of highway-rail crossing safety at both the State and Federal level.

The report further recommended that :

a.	The Federal Railroad Administration issue requirements for the railroads to assign and display identification numbers at all highway-rail crossings based upon a uniform national standard to be prescribed by the Department of Transportation.  Further, it required FRA to contract with all railroads to provide site-specific inventory data for all crossings on their respective lines, and to annually provide information updating this inventory following inventory standards established jointly by the Federal Highway Administration and the Federal Railroad Administration and working with appropriate railroad and State representatives.

b.	The Federal Railroad Administration expand the current highway-rail crossing accident reporting by the railroads to include all train-involved public and private crossing incidents.

NOTE:	The terms "accident" and "incident" are used interchangeably in this manual.  The current preferred term for "accidents" is often "collisions" or "crashes."

3

c.	The National Highway Traffic Safety Administration (NHTSA) give early attention and emphasis to implementation of a plan to have all highway-rail crossing accidents reported through a central State agency.  Also, NHTSA should require the inclusion of the crossing identification number on the accident report form used by police officers when reporting highway-rail crossing accidents to permit correlation of railroad and police reports with the crossing inventory.

Following the submission of the report, the Federal Railroad Administration assumed principal responsibility for the development of the National Highway-Rail Crossing Information System.

The Federal Railroad Administration entered into a contract with the Association of American Railroads to develop a "Comprehensive National Highway-Rail Crossing Information and Numbering System."  The project was established as a cooperative effort between all the nation's railroads and the U.S. Department of Transportation with the cost of the project to be funded equally by the railroads and the U.S. Department of Transportation.

The railroad companies, with direction and guidance from the Association of American Railroads and the American Short Line Railroad Association, were assigned the responsibility for making a site-specific inventory of each highway-rail crossing and for installing a unique identifying number at each location.  The railroads were also identified as being responsible for periodic update of certain inventory information and maintenance of the crossing number.

The State highway departments assisted in the project by providing site-specific highway location and use data.  State public utility commissions and other State and local governmental agencies also participated in the project.  The responsibility for the updating of certain highway information data items was determined to be through the efforts of these agencies.

1.4  Data Files

There are two types of data files maintained by the FRA.  These two data files are the Inventory Data File and the Accident Data File.

The Inventory Data File is a record of grade crossing location, physical, and operational characteristics to provide information for the administration and statistical analysis of crossings.  This information is reported to the FRA on the U.S. DOT-AAR Crossing Inventory Form (see Figure 1-1).  Each State and railroad is responsible for maintaining its respective inventory file.  In order for the files to serve as an effective data base, the States and railroads maintaining their own file should immediately update them.  States can maintain the National Data File in lieu of their own file.

The Accident Data File is a record of all train-involved crossing accidents or incidents.  The Federal Railroad Administration now requires the reporting of all train-involved crossing accidents and incidents which includes the DOT-AAR Crossing Identification Number.  The form used to report all train-involved crossing accidents or incidents is the Highway Grade Crossing Incident Report (see Figure 1-2).  Effective 1/1/97, there will be a new version of this form.

Routinely, the highway-rail crossing accident data is integrated with inventory data and the information from the combination is used for the development of Federal programs, funding alternatives for crossing improvement, studies related to railroad safety programs, effectiveness of warning devices, high-speed railroad corridors, accident costs, public awareness and driver training, and other safety program development and research opportunities.

The Federal Railroad Administration's Office of Safety, Highway-Rail Crossing and Trespasser Programs Division, serves as the National Highway-Rail Crossing Information Center.  The address is:

Federal Railroad Administration

Office of Safety

Highway-Rail Crossing and Trespasser Programs Division

400 7th Street, S.W., (RRS-23)

Washington, D.C.  20590

1.5  Definitions of Highway-Rail Intersections

For the National Crossing Inventory purposes, the following definitions apply for all public, private and pedestrian crossings, with or without crossing signs or active warning devices.

A highway-rail crossing is the intersection (at grade or grade separated) of a roadway (including associated sidewalks and pathways) and one or more railroad tracks.  A crossing at a dual or multi-lane roadway is reported as a single crossing.  Also, a crossing is reported as a "single crossing" even where there are multiple tracks within the limits of a single set of crossing signs or warning devices and even if the individual tracks belong to more than one railroad company or track owner.  (For further discussion of this latter point, see Section 2.3 of this manual.)  As a minimum, all at-grade crossings of public and private roads and streets with railroad tracks across are assigned an inventory number if any railroad operations are conducted.

As a general rule, and if no other agreement exists, a crossing that is located (usually equally) on a State, county and/or city boundary line should be considered to be geographically located in the jurisdiction that is South or East of the crossing.

NOTE:

49 CFR Part 234.5(a) provides the following definition:

"Highway-rail grade crossing means a location where a public highway, road, street, or private roadway, including associated sidewalks and pathways, crosses one or more railroad tracks at grade."

4

For the purposes of the Inventory only, the following definitions apply:

A.	Public Crossing.

A public crossing is the location where railroad tracks intersect a roadway which is part of the general system of public streets and highways, and is under the jurisdiction of and maintained by a public authority and open to the general traveling public.

Public crossings can be at-grade or grade separated.  If they are at-grade, usually both highway approaches are maintained by a public authority, or the public authority accepts the responsibility for the roadway maintenance.  (Highway is a word used here to include highways, streets and roads into a single word.)

NOTE:

23 CFR Part 460.2 provides the following definitions:

"Public road means any road under the jurisdiction of and maintained by a public authority and open to public travel."

"Public authority means a Federal, State, county, town or township, Indian tribe, municipal or other local government or instrumentality thereof, with authority to finance, build, operate or maintain toll or toll-free highway facilities."

"Open to public travel means that the road section is available, except during scheduled periods, extreme weather or emergency conditions, passable by four-wheel standard passenger cars, and open to the general public for use without restrictive gates, prohibitive signs, or regulation other than restrictions based on size, weight, or class of registration.  Toll plazas of public toll roads are not considered restrictive gates."

"Maintenance means the preservation of the entire highway, including surfaces, shoulders, roadsides, structures, and such traffic control devices as are necessary for its safe and efficient utilization."

5

1.	Discussion.

A crossing shall be classified as public if, and only if, the roadway is deemed a public road in accordance with 23 CFR Part 460.2.  In general, a roadway across railroad track for which both approaches are maintained by a public authority and which is open to the public is considered a "public" crossing.  These are roadways that are part of the general system of public streets and highways.  Some jurisdictions accept a crossing as "public" when only one approach is publicly maintained.  If a public authority accepts a crossing as "public," it is a public crossing.  All others are considered "private."

Therefore, public crossings are those on roadways which meet the following three conditions:

a.	The roadway is part of the general system of public streets and highways, and

b.	Under the jurisdiction of and maintained by a public authority, and

c.	Open to the general traveling public.

2.	Access to Public Facilities.

If the primary function of the road is to provide public access to a publicly owned facility for the principal purpose of on-site use by the public, then the facility may be deemed a logical terminus of a public roadway.

Thus, crossings which exist for the primary purpose of providing public access to publicly owned and operated facilities such as fairgrounds, parks, schools, libraries, hospitals, clinics, airports, bus terminals, beaches, piers, boat launching ramps, recreational facilities, etc., which permit access to or invite use by the general traveling public would satisfy the definition "open to public travel," even if the entrance thereto is equipped with gates to effect seasonal or periodic closures (such as overnight), or limit access, or require an entry fee for use.

3.	Determined by Empowerment.

In situations where a State has empowered a public agency (such as the State DOT, State Highway Department, Public Utility Commission, State Commerce Commission, etc.) to make determinations as to whether crossings are public or private, such determinations will govern for Inventory purposes.

B.	Private Crossing.

1.	Definition.

A private crossing is a highway-rail crossing which is not a public crossing.  (A public crossing is defined as the location where railroad tracks intersect a roadway which is under the jurisdiction of and maintained by a public authority and open to public travel.  See Par. 1.5.A.)

2.	Discussion.

A private crossing is one that is on a private roadway which may connect to part of the general system of public streets and highways but is not maintained by a public authority.  Usually, it is a crossing where the property on both sides or at least one side of the railroad tracks is private property.  It may also be on a roadway that is publicly owned but which is either restricted or not intended for use by the general public.  Private crossings are generally intended for the exclusive use of the adjoining property owner and the property owner's family, employees, agents, patrons and invitees.  Crossings are classified as private where the normal need or use is for residential, farm, recreation/cultural, industrial or commercial activities.

Most private crossings exist by virtue of railroad charter provisions, deed covenants, State statute or other prescriptive rights.  If none of these apply, the railroad may require an agreement with the private property owner whereby the railroad may install and maintain the crossing proper and any necessary signs or signals at the property owner's expense, and the property owner will assume liability for the crossing and provide coverage via a liability insurance policy.

In some instances, changes in land use have resulted in an expansion of crossing use to the extent that a previously private crossing has some attributes of a public crossing, whether or not any public agency has accepted responsibility for maintenance or control of the use of the roadway over the crossing.  The railroad company and highway agency should make every effort to mutually resolve and agree on the appropriate classification (either public or private) of such a questionable crossing.

3.	Private Crossings with Public Access.

A private crossing may exist with permitted or limited public access for the primary purpose of providing public access to facilities (either public or privately owned) such as shopping centers, fairgrounds, parks, golf courses, zoos, museums, schools, libraries, hospitals, clinics, airports, bus terminals, beaches, piers, boat ramps, recreational facilities, etc.  Such crossings permit access or invite use by the general public, but usually restrict or discourage general public use by requiring permits, or charging admission or other fees to gain entry or use of the facility.

For such crossings, the primary roadway use is to gain entry to the facility.  The entrance may even be equipped with gates to effect seasonal or periodic closures.  These crossings generally do not qualify as being "open to the general traveling public" and should not be deemed as a public crossing.

Crossings which exist primarily to provide access to publicly owned facilities for "authorized personnel only", such as military bases, ports, equipment yards, maintenance/storage facilities, water or sewerage treatment plants, landfills, levees, service and/or maintenance only entrances, or other facilities, are not normally intended for on-site use by the general public and should be deemed as a private crossing.

4.	Private Crossing with No Public Access.

A private crossing with no public access would include, for instance, the crossing within a secured industrial complex or between farm fields where public access to the complex or fields is precluded.

5.	Responsibilities.

The railroad should ensure that each crossing is listed in the National Inventory.

C.	Pedestrian Crossing.

A pedestrian crossing is a separate designated intersection where pedestrians, but not vehicles, cross a track.  Sidewalk crossings contiguous with, or separate but adjacent to, public road crossings, and in the public road right-of-way, are presumed to be part of the public roadway crossing and are not assigned a separate crossing number.

An area where pedestrians trespass is not considered a crossing.  The designation of a crossing may be made by a sign, device, or filled materials between the rails.

Pedestrian crossings may also be classified as public pedestrian crossings or private pedestrian crossings, similar to vehicle crossings.  The determination is based on whether or not a public agency has jurisdiction over and maintains the sidewalk (or walkway) on either side of the track and whether the crossing is intended for use by the general public.  The distinction may be shown on the Inventory Form by inserting "PUB" for public or "PVT" for private in the field for Item 10. - Street or Road Name.

D.	Special Situations.

An area where vehicles or pedestrians trespass is not considered a crossing.  Vehicles or persons that cross railroad tracks without railroad permission at other than defined crossings are considered trespassers.

Crossings used only by railroads (such as in a railroad yard or terminal) do not need to be reported, but it is best to assign at least one crossing number to the entire location so that a crossing accident, if it occurs, can be properly identified to that location.  Also, where multiple private industrial crossings exist within the same restricted-access industrial plant or facility, the railroad should report all such crossings but may report them under a single crossing number.  Any accidents, if they occur, would be reported under that single crossing number.

Crossings created to serve specific temporary (six months or less) activities, such as construction sites, do not need to be assigned a crossing number and reported.

E.	Open Crossing.

An active or open crossing is one where railroad operations and highway traffic occur or could occur on a regular or irregular basis.

F.	Closed Crossing.

A closed crossing is one where the crossing has been physically removed or where railroad operations or highway traffic are not possible.

Examples are where the crossing has been barricaded and highway crossing surface material removed, or where the railroad tracks have been cut or barricaded, physically removed, or a connecting turnout has been removed, or where rail operations are not possible because the railroad tracks are paved over, etc.  Crossings along such inactive railroad lines should be reported as closed.  (Crossing records reported as closed remain in the National Inventory File for at least five years.)

G.	Crossings on Abandoned Track.

Crossings along railroad lines that have been placed in a railroad "abandoned" category, are seasonal in usage, or might be considered temporarily out-of-service (no railroad operations occur) should remain in an open status as long as track remains in place and there is a reasonable possibility that the line will be used again.  This condition may exist even if the particular line is physically separated from the balance of the railroad operating system.  For example, if a railroad abandons a line which may very possibly be turned into a commuter line as soon as the appropriate political and financial conditions are resolved, the crossings along the line do not need to be reported as closed in the Inventory.  While these crossings remain active and open in the Inventory, the railroad traffic numbers should be reduced to zero in the Inventory until the line becomes active again.  The ownership will also probably change to the new property owner.  If progress toward reactivating the line does not proceed within a reasonable period of time (about 2 years), then the crossings should be reported as closed and re-opened at a later date using the same crossing numbers.  "Rail-banked" crossings, those which may be on-hold by a State or other governmental body for possible future use, would be in this category.

H.	Standards and Regulations.

The current Federal regulation that applies to highway-rail crossings is the Code of Federal Regulations, Title 49 CFR Part 234, Grade Crossing Signal System Safety, effective January 1, 1995.  This regulation imposes minimum maintenance, inspection and testing standards for highway-rail grade crossing warning systems.  This Regulation (Part) also prescribes standards for the reporting of failures of such systems and prescribes minimum actions that railroads must take when such warning systems malfunction.  This Part does not restrict a railroad from adopting and enforcing additional or more stringent requirements not inconsistent with this Part.  Effective August 19, 1996, FRA amended 49 CFR Part 234 to clarified the Rule as originally published on September 30, 1994.

Additionally, there are Regulations (FHWA) to prescribe policies and procedures for advancing Federal-aid projects involving railroad facilities including projects for elimination of hazards, adjustments required by highway construction, and other financial reimbursement and audit procedures.  These are contained in Title 23 CFR Part 646.2, Subpart B - Railroad-Highway Projects.  Specifically, Part 646.214 Design refers to the conditions where installations are to include automatic gates with flashing light signals.

Sign and warning device standards are delineated in the Manual on Uniform Traffic Control Devices (MUTCD) published by the Federal Highway Administration.  Part VIII of the MUTCD deals with highway-rail crossings.

Also, the Railroad-Highway Grade Crossing Handbook provides general information on the physical and operational conditions of crossings for safe and efficient use by both highway and rail traffic.

Figure 1-1.  U.S. DOT-AAR Crossing Inventory Form

Figure 1-2.  Highway Grade Crossing Incident Report

2.0  HIGHWAY-RAIL CROSSING INVENTORY NUMBER

2.1  Background

In 1974, an effort was undertaken to inventory and assign a unique number to all public and private highway-railroad intersections and pedestrian crossings in the United States.  As a part of the original inventory, data were collected for all public, private, and pedestrian crossings, both at grade and grade separated, including location, operational, physical and classification information.

A National Advisory Committee having representation from all involved parties was appointed to provide technical guidelines for the implementation of the inventory.  The Advisory Committee determined the type and extent of the data to be collected.  In general, the data elements to be included in the inventory were selected on the basis of their significance to the computation of a priority index for grade crossing improvements.

The railroad industry and each of the States participated in the initial inventory.  Following an agreed procedure, the States and railroads continue to submit new and updated crossing information to the Federal Railroad Administration.  The FRA, through its contractor, updates and maintains the National Data File.  This information is available for public use and may be obtained through the FRA Office of Safety.

In addition to these data, some States and some railroads maintain additional crossing information in their data files.  However, these data are not generally available to the public and may be obtained only through the State or railroad maintaining the supplemental information.

In many ways the National Highway-Rail Crossing Inventory number, placed at all public, private, and pedestrian crossings, is similar to a credit card or bank account number.  Important information is assigned to the number by State agencies and railroads.  Police, accident investigators, project engineers, utilities, States and railroads are but a few of those who refer to these numbers and the connecting data regularly.

The need for accurate information assigned to the appropriate crossing is important in any decision to upgrade existing warning devices.  The number of accidents, motor vehicles and trains using the crossing, and the type of warning device are but a few of the data elements that are critical in the computation of a "hazard index" for individual grade crossings.  Not only is it important that these data be kept current, but it is also critical that the information be assigned to the proper crossing via the identification number.

In addition to the assignment of data regarding the physical and operational characteristics of a crossing, the inventory number is used on all FRA grade crossing accident reports and warning device malfunction reports.  Many States and local jurisdictions use the crossing number on reports of accidents at or near crossings even when not involving a rail vehicle.  All railroads and States use the inventory number on crossing improvement project documents, and railroad crews report near misses and other information regarding a crossing by the inventory number.  Some utility companies even use the number to locate rail  crossings.  All of these factors require the need for displaying the number at the crossing to insure that the information is being assigned to the correct location.

2.2  Uniqueness and Calculation

The crossing inventory numbering system was designed to reduce the possibility of error by insuring that crossing data is recorded for the correct location.  The crossing identification number, which consists of a maximum of six numeric digits with a single alpha check character, is placed at crossings on number boards along with the "U.S. DOT-AAR" designation.  The number assigned to each highway-rail intersection is unique.  It is important for proper identification to have the crossing number permanently displayed and mounted on a number board (Figure 2.1) and it is strongly recommended that it be displayed on both sides of the track at each and every crossing.  By referencing this number, all inventory and accident data on file, including data collected by State and local agencies and railroad companies, will have a common link.  The identification number serves as a communication reference between railroad companies and public agencies, as well as between individual railroad companies regarding specific crossings.

A simple numeric system requiring the use of a maximum of six digits was adopted by the National Advisory Committee.  Some exceptions were made in the numbering system.  For example, numbers having the same digit repeated consecutively three or more times were eliminated (e.g., 7777).  Also, numbers having three digits or less were not used, and some may have leading zeros.  The crossing identification number, with its six numeric digits, has a single alpha check character at the end of the number sequence.

This alpha check character is another feature of the National Inventory number that makes it similar to a credit card.  When the inventory numbers are generated, they are accompanied by the check character.  Therefore, every time a number is used, it can be validated by the check character.  The check is performed as follows:

a.	Add the six individual numbers which result from the products of each of the first six digits times the digit's position in the number stream, with position one being the left-most digit (see Step 1 below).

b.	Subtract multiples of 22 from this total until the remainder is less than 22 (see Step 2 below).

c.	The remainder is then compared against Table 2-1 to find or verify the alpha code.

2

Figure 2-1.  National Highway-Rail Crossing Inventory Number and Number Board

EXAMPLE #1

The procedure can be illustrated by validating the inventory number from Figure 2-1 (836 597 H).  The validation is done as follows:

Step 1.  Compute Numeric Code.

= [(8x1) + (3x2) + (6x3) + (5x4) + (9x5) + (7x6)]

= (8 + 6 + 18 + 20 + 45 + 42)

= 139

Step 2.  Determine Remainder for Alpha Code.

= 139 - (subtract multiples of 22 until you get a number that is less than 22)

= 139 - (22 x 6)

= 139 - 132

= 7

Step 3. Verify the Alpha Code

The alpha character represented by the number 7 is the letter H (from Table  2-1).

Therefore, the inventory number (836 597 H) has been validated.

EXAMPLE # 2

A second example is shown below:

| Crossing Number: |  | 0 | 7 | 6 | 5 | 2 | 1 | C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Multiplication: | x | 1 | 2 | 3 | 4 | 5 | 6 |  |  |
| Sum: |  | 0 | +14 | +18 | +20 | +10 | +6 | = | 68 |

Remainder:		68/22 = 3 and remainder of 2

(NOTE:  Use long division or subtract multiples of 22.)

Alpha Code:		2 = C from Table 2-1

| Remainder | Alpha Code | Remainder | Alpha Code | Remainder | Alpha Code | Remainder | Alpha Code |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | A | 6 | G | 12 | N | 18 | V |
| 1 | B | 7 | H | 13 | P | 19 | W |
| 2 | C | 8 | J | 14 | R | 20 | X |
| 3 | D | 9 | K | 15 | S | 21 | Y |
| 4 | E | 10 | L | 16 | T |  |  |
| 5 | F | 11 | M | 17 | U |  |  |

Table 2-1.  Remainder vs. Alpha Code

2.3  Assignment of Numbers

Every crossing in the United States, including public, private and pedestrian, both at grade and grade separated shall have a crossing inventory number assigned and recorded in the National File.  The only exceptions are crossings (1) created to serve specific temporary (six months or less) activities such as construction, and (2) those used only by railroad employees within a railroad yard on railroad property and not available to the general public.  In a plant complex or for yard-type locations where there are numerous crossings that are not always distinguishable (e.g., a port or dock area), one number can be assigned to include all crossing areas within the property limits.  Such number should be clearly posted at the railroad point of entry.

NOTE:

There should only be one crossing number assigned to a single crossing (defined as the tracks between a pair of warning devices), no matter how many railroads own track that traverses the crossing.  There may be cases where two mainline tracks, owned and maintained by two different railroads, traverse a crossing, with each of these railroads having assigned a separate crossing number for the crossing.  If this situation exists, one of the numbers should be deleted (closed) and one of the railroads involved should claim the crossing and list the other railroad as "operating across the same crossing."

To identify the owning railroad for a crossing, use the following procedure:

	identify the operating railroad or the railroad that owns the property, or

	identify the railroad that performs maintenance on the crossing, or

	have the railroads jointly decide who will carry the crossing on its inventory.

6

Any remaining original number "tags" from the 1973-1975 National Inventory Project which have not been assigned to a crossing may be assigned to and installed at new crossings.  When this supply is exhausted, the FRA National Highway-Rail Information Center will furnish, upon request, a series of unique numbers to be assigned to newly opened crossings or to crossings that are identified without a number.  A railroad or State should request enough numbers to meet their estimated yearly need, including any increase in new crossings.  (Tags or number boards are not provided by FRA.)

Railroads and States may address their requests for crossing inventory numbers to:

Federal Railroad Administration

Office of Safety

Highway-Rail Crossing and Trespasser Programs Division

400 7th Street, S.W., (RRS-23)

Washington, D.C.  20590

New numbers are to be used for any new crossings or for any crossings that have been identified as not having an assigned number (a careful and detailed check should be made before assignment of a new number to insure that a prior number was not already assigned).  While FRA provides valid usable crossing numbers, it is the railroad or State that actually assigns the number.  The actual assignment of a number to a crossing occurs when the number is placed on a completed four-part Inventory Form and the Form returned to FRA for processing into the National File (which takes about three months).  It is important that this occur as quickly as possible for any existing crossing that does not have a number.  Preparation and submittal of a four-part U.S. DOT-AAR Crossing Inventory Form for such crossings should be expedited.

For a public at-grade crossing, the railroad needs to complete Parts I, II & III.  For private crossings, grade-separated crossings (including public) and all pedestrian crossings, only Part I information is required.  The railroad retains the fourth sheet (orange) and sends the top three sheets to the appropriate "State Crossing Inventory Contact" (see Appendix A) for completion of Part IV (if the crossing is public at-grade).  We suggest that the railroad copy the FRA on the transmittal correspondence.  The State will complete Part IV, retain the yellow copy, return the pink copy to the railroad, and forward the green copy to FRA for processing into the National Inventory File.  New public at-grade crossing records cannot be entered into the National Inventory File unless all items in Parts I-IV are completely filled in.

A crossing that is moved may or may not retain the same crossing inventory number.  The crossing number is unique to a specific crossing and location.  If the physical crossing is moved a short distance (usually within several hundred feet) and the operational characteristics do not materially change, the same number may be used at the new location.  If this is not the case, the old number should be closed and a new number assigned to the new location.

If a crossing is closed, the closure must be reported to the FRA.  Although the crossing will no longer be in use, the number assigned will be permanently associated with the closed crossing in the National Inventory File.  It is imperative that the number not be re-assigned to another new crossing.  However, if the crossing is re-opened at a later date, the same number would be re-activated and used.

All crossings (public, private, and pedestrian, including grade-separated) must be inventoried and the inventory records should be updated at least once every five years to verify that the crossing still exists.

When there is no record of a crossing number in the U.S. DOT-AAR National Highway-Rail Crossing Inventory, the usual cause is that the crossing was never inventoried or that the original inventory record was never submitted to FRA for input into the National File.  It is also possible that the crossing was recorded as closed at some time in the past.  Whatever the reason, it is very important that the situation be corrected as rapidly as possible by preparation and submission of a four-part U.S. DOT-AAR Crossing Inventory Form.

When a crossing is located on a county or State line, it is suggested that the crossing be inventoried by and in the county or State that is south or east geographically.

If you have any questions about this process, contact FRA at (202) 632-3312.

2.4  Number Boards

The National Crossing Inventory Number, which consists of six numeric digits and an alpha check character, shall be permanently displayed at all crossings, recommended on both sides of the railroad right-of-way, on number boards in accordance with specifications outlined herein.  The FRA does not provide tags or number boards.  These may be purchased from suppliers or manufactured by the railroad.  In some cases, States have supplied number boards as part of an overall program.

2.4.1  Specifications

The specifications for permanent number boards for the DOT-AAR Highway-Railroad Crossing Inventory System were adopted in 1978 by the National Advisory Committee.

The number board shall be of light-gauge (.032") aluminum, 4" x 9" in size, with eight slots for mounting, and left unpainted so as not to be attractive to vandals.  The crossing number shall be embossed in the center with 1-1/2" numeric-alpha characters.  The railroad code shall be embossed above the crossing number with 1/2" characters and the Inventory System designation "U.S. DOT-AAR" shall be embossed with 1/2" characters below the crossing number.  (See Figure 2-1 for an example.)  This type of sign can be considered as an unpainted "license plate" and blanks used for motorcycle tags could be used.

The method of manufacture or procurement of the permanent number board is left to the discretion of the railroads.  However, the resulting sign or number board must comply with the general specifications (see Table 2-2).  The railroad may fabricate or purchase signs of metal, plastic, or other suitable materials, or may emboss, stencil, paint, or otherwise inscribe the number in the proper location.

 4 inches high by 9 inches wide

 Made of .032 inch thick aluminum, unpainted

 Pierced with eight slots, four on each side

 1/2 inch embossed across the top: the railroad code (example: BN)

 1-1/2 inch embossed across the center: the crossing number (example: 836 597 H)

 1/2 inch embossed across the bottom: U.S. DOT-AAR

 2 plates for each crossing are recommended, plus any desired spares

Table 2-2.  Number Board Specifications

Crossing number boards can be purchased from manufacturers and fabricators of signs and railroad materials.  However, as of January 1994, only one manufacturer was known to supply such number boards as part of their offered products.  This manufacturer is identified below for the convenience of short line railroads and others who may not have fabricating facilities, or for those who do not wish to fabricate their own plates.  This is not an endorsement of this manufacturer in any way by the FRA.

Keyes-Davis Company

Box 1557, 74 Fourteenth Street

Battle Creek, Michigan 49016

Telephone:	(616) 962-7505

Facsimile:	(616) 962-4411

Approximate price quotes, as of 10/18/95, for individually numbered plates conforming to the specifications described above are contained in the following table (Table 2-3 ).

| In Quantities of | Price per Board |
| --- | --- |
| 1-9 | $22.00 |
| 10-24 | 16.50 |
| 25-49 | 12.25 |
| 50-99 | 7.25 |
| 100-249 | 5.75 |
| 250-499 | 4.25 |
| 500-999 | 3.35 |
| 1000-2499 | 2.50 |
| 2500-up | 1.60 |

Table 2-3.  Number Board Price Quote

2.4.2  Installation and Display

At least one sign shall be located on a signal mast or crossbuck post for crossing and must be clearly visible from the roadway.  Ideally, it should also be visible from the rail right-of-way, if possible.  While only one sign is required, two signs are recommended, one for each side of the crossing.  If a mast or post is not present, the number should be mounted to any type of fixture or structure present, even a wall or the ties.  In lieu of a sign, the number could be clearly painted (stenciled) on the masts or posts.  As a temporary or alternate measure, spray painting or stenciling can be used on signal cabinets or any other suitable location.  The key point to remember, DISPLAY THE NUMBER AT BOTH SIDES OF THE CROSSING FOR EACH AND EVERY CROSSING.

There are three important considerations when installing number boards,

a.	The attachment of the number board should not interfere with the operation of the crossing warning device;

b.	The number board should be placed (where possible) above the reach of possible vandals; and,

c.	When attached to posts or poles, the number board should be mounted, whenever possible, so that it is facing the roadway.

There is only one number for a crossing, but it is recommended that two number boards be installed, one on each side of the crossing.  Some examples of installation are shown in Figure 2-2.

When number boards are to be mounted on metal poles, a banding tool and metal strap are required.  To provide a secure attachment, care must be taken in threading the strapping material through fastening slots in the number board.

Where number boards are to be mounted on wooden poles, galvanized nails or heavy duty staples are required.  The minimum number of nails or staples should be four.

The number boards can be mounted on the different fixtures in the following manner:

a.	Crossbucks.  The number board should be installed just above eye level using either nails or long staples, or strapping for metal poles.

b.	Mast Mounted or Cantilevered Flashing Lights and/or Gates.  The number board should be installed just above eye level using strapping material.

c.	Stop Sign or Other Sign.  Where crossbucks or other signals are not present, mount the number board on a metal or wood pole supporting the sign.  The number board should be installed above eye level or just below the stop sign.

d.	Post Mount where there are no Signs.  Mount the number board on a post at least 5 feet above the ground.  Installation on a post is recommended at locations where signs or signals are not installed, such as at private crossings and grade separations.  When the sign cannot be installed on a post, the railroad should spray paint or stencil the number on a suitable location and store the number board for later installation, or the number board may be mounted on the nearest railroad-owned signal or communication pole.

2.4.3  Maintenance

To insure identification and verification of a crossing, the display of number boards must be maintained by the railroads at all crossings, especially at public-at-grade crossings.  Display and maintenance of signs at private, pedestrian, and grade-separated crossings is also important for identification purposes and should be maintained in the same manner.  Number boards should be replaced if the originals deteriorate or are vandalized.

Care must be taken that number boards are not transported to an incorrect location via posts and poles which may be classified as surplus and then re-used at a different location.  When posts or poles are taken down, the number boards should be removed and re-installed or destroyed if the crossing is closed.  When surplussed (re-used) poles or posts are installed,

Figure 2-2.  Number Board Mountings

the installation crew should insure that these do not contain number boards from other locations.

3.0  U.S. DOT-AAR CROSSING INVENTORY FORM

RECORDING INSTRUCTIONS

3.1  Recording Instructions

The following section explains the process of filling out the U.S. DOT-AAR Crossing Inventory Form.  Each section is preceded by a pictorial representation of the section of the form being detailed.  Each letter (e.g., A.) or item number (e.g., Item 1.) in the explanation refers to the same lettered or numbered item on the form.  Alphanumeric items are to be entered left-justified, while numeric items will be entered right-justified.

The four-part U.S. DOT-AAR Crossing Inventory form may be used to submit crossing inventory changes.  Railroads should check submittals to insure that changes have been circled, that an effective date is shown, and that all information is correct.  If the changes are simple or minor (numbers, closings, ownership changes, etc.), corrections can be made in red pen directly on the "one-page-per crossing printout" sheets and returned to FRA through the State for processing.

3.2  U.S. DOT-AAR Crossing Inventory Form Heading

A.  Initiating Agency.

Enter a check mark in the appropriate box (for either railroad or State) to indicate who is initiating the update form.

B.  Crossing Number.

Enter the crossing inventory number here and under "I. D. Number" at the bottom of the form.

C.  Reason for Update.

Enter a check mark in the appropriate box to indicate that the reason for the form is a change in existing data, a new crossing, or a closing is being reported.

D.  Effective Date.

Enter the date the change was completed or put into effect.  Ideally, all public, private and pedestrian crossings, including grade-separated, should be updated to at least verify that the crossings still exist.  A current effective date should be indicated.  If it is verified that there are no changes in the data and the crossing still exists, and the most recent record is over 5 years old, an effective date of January 1 of the current year (e.g., 01/01/96) should be indicated in red pen on the inventory report and the old date crossed out.

3.3  Part I  Location and Classification of All Crossings

Item 1.  Railroad Operating Company.

Enter the U.S. DOT-AAR railroad code (reference Appendix E) of the "operating" railroad company, i.e., the railroad that operates train movements over the crossing.  The operating railroad will normally also be the reporting company but may or may not own and maintain the roadbed, tracks, and signal system controlling the crossing.  If the operating company is not the owner of the track, and the track owner would generally be considered as a "non-railroad," it is suggested to enter the owner's name  in Item 13, Branch or Line Name.

NOTE:	Crossings are to be assigned to the operating railroad, that is, the identity of the railroad company that operates over the trackage where the crossing is located and not necessarily to the owner of the track or property itself, unless it is an operating railroad.  Thus, designations such as  "XYZ Corporation" should be changed to the name of the railroad that is actually operating on the specific line since they are the operating railroad.

7

Item 2.  Railroad Division or Region.

Enter the name of the division, region, or major district, if the railroad system is divided into such groups.

Item 3.  Railroad Subdivision or District.

Enter the name of the sub-division or other classification, if the railroad system is divided into such groups.

Item 4.  State.

Enter the name of the State where the crossing is located.  If the crossing is located on a State boundary so that parts of the crossing lie in two or more States, agreement must be made between the two States as to which shall "claim" it for inventory record purposes.

Item 5.  County.

Enter the name of the county where the crossing is located.  If the crossing is on a county line so that parts of the crossing lie in two or more counties, a decision must be made to place it in one county only.

Item 6.  County Map. Ref. No.

Enter the county map identification or other reference number provided by the highway agency to specifically identify the crossing on the street and road system.  If it is not available, leave this entry blank.

Item 7.  City.

Enter the name of the incorporated city where the crossing is located.  If the crossing is on a city line so that parts of the crossing lie in two or more cities, identify only one city.  If not within a city, omit this item and complete item 8.

Item 8.  Nearest City.

If the crossing is not within an incorporated city, town, or village, enter the name of the unincorporated city, town, or village or the nearest city, town, or village, whether or not on the railroad lines.

Item 9.  Highway Type and No.

Enter the type of highway such as Interstate (I), U.S. numbered (US), State (ST), county (C), local (L), etc., and number of the highway.  Please abbreviate, as I-95, US-1, ST-234, C-2096, etc.  The number of the highway should be posted on the highway or found on State or county maps.  If there is more than one number, enter the most important route or all the numbers.

Item 10.  Street or Road Name.

Enter the name of the highway or street, if the highway or street has a name.  If it is a private roadway and it has a name, enter the name of the road or the owner's name, otherwise just enter "private."

Item 11.  RR I.D. No.

If a crossing has an identification number other than the DOT-AAR number, such as a State agency number (e.g., a Public Utility Commission (PUC) assigned number) or a railroad number and it is posted at or assigned to the crossing, enter that number.

Item 12.  Nearest RR Timetable Station.

Enter the name of the nearest timetable station of the operating company.

Item 13.  Branch or Line Name.

Enter the name of the line or branch as used by the railroad to describe this segment of track.  If the track is an industry lead, industry spur, yard lead, wye, etc., enter the name of the track or industry.

Item 14.  Railroad Mile Post.

Enter the railroad milepost number in miles and hundredths of miles.  (53 feet is approximately 1/100 mile.)  Enter the number with the decimal point.

NOTE:	Because of data-retrieval anomalies, alphabetical letters in the milepost field should be avoided.

8

Item 15.  Pedestrian Crossing (Position).

Enter a check for the appropriate position of the railroad relative to the pedestrian crossing.

Item 16A. Private Vehicle Crossing (Type).

Enter a check in the box which best describes the usage of a private crossing based on the following categories:

1.  Farm.  A farm crossing is any crossing used for the movement of farm motor vehicles, farm machinery or livestock in connection with agricultural pursuits, forestry, or other land-productive purposes.

2.  Residential.  A residential crossing is any crossing used to provide vehicular access for occupants and their invitees to a private residence or residences.

3.  Recreational.  A recreational crossing is any crossing used to provide access to otherwise isolated recreational areas.

4.  Industrial.  An industrial crossing is any crossing used to provide access between industrial plant facilities or to an industrial or other commercial area.

Item 16B.  Private Vehicle Crossing (Position).

Enter a check in the appropriate box describing where the railroad crosses the highway.

Item 16C.  Private Vehicle Crossing (Warning Device).

Enter a check in the appropriate box for the type of crossing warning device.  If signs and/or signals exist, enter a brief description in the spaces provided.

Item 17.  Public Vehicle Crossing (Position).

Enter a check in the appropriate box for the position of the railroad relative to the public crossing and complete the remainder of the form for all public at-grade vehicular crossings.

NOTE:	For private, pedestrian, and grade-separated crossings, no further information needs to be provided.  Such forms are complete and should be forwarded to the State and FRA for processing.

9

3.4  Part II  Detailed Information for Public Vehicular at Grade Crossing

Item 1A1 - 1A4.

Enter the number of the train movements through the crossing and the number of switching movements at the crossing for both daylight and night time hours.  Typical number of daily train movements means the normal or average daily train movements.  Through trains are trains whose primary responsibility is to move cars over the road, and there may be a limited number of pickups and setouts along the route.  Classify all others, (i.e., locals, industrial runs, switch engine) as switching movements.  Include the total number of the train movements both for the reporting "operating" company and for any other railroad operating over the crossing.

Item 1B.  Check if Less Than One Movement Per Day.

Enter a check if train frequency is less than one train per day.

Item 2A.  Speed of Train at Crossing, Maximum Time Table Speed.

Enter the maximum timetable speed.

Item 2B.  Speed of Train at Crossing, Typical Speed Range Over Crossing.

Enter the typical minimum speed and maximum speed over the crossing.

Item 3.  Type and Number of Tracks.

Enter the number of main line tracks and specify the number and type of any "other" tracks.  A track is considered main if through trains operate on the track.

Item 4.  Does Another RR Operate a Separate Track at Crossing?.

Enter the U.S. DOT-AAR railroad codes of all railroads that operate a separate track within the warning devices at the crossing.  Up to four railroad codes, in blocks of four characters each, may be entered in this field.  Enter each railroad code, flush left, and the appropriate blank spaces if the code is not four characters.

Item 5.  Does Another RR Operate Over Your Track at Crossing.

Enter the U.S. DOT-AAR railroad codes of all railroads that operate trains over your track at the crossing.  Up to four railroad codes, in blocks of four characters each, may be entered in this field.  Enter each railroad code, flush left, and the appropriate blank spaces if the code is not four characters.

NOTE:	For Items 6A through 6D, that follow, if more than one type of warning device is present, fill in all applicable types of warning device(s).  Enter a "9" where the number is 9 or greater.  Provide short descriptions of "other" devices in the appropriate spaces.

10

Item 6A.  Type of Warning Device at Crossing, Signs.

Enter the number of masts with crossbucks, not a count of all crossbuck signs.  Two or more crossbucks mounted on a single mast are counted as one crossbuck.  If the crossing has a train activated device, do not count the crossbucks mounted on that device.

A standard highway stop sign is red with white letters and has eight sides.  Classify all other stop signs as "other stop signs."

Also indicate number and type of any other passive signs at crossing.

Item 6B.  Type of Warning Device at Crossing, Train Activated Devices.

9./10.  Gates:  Enter the count of gates with red and white reflectorized arms separately from the count of other colored gates.

11./12.  Cantilevered Flashing Lights:  Separate cantilevered flashers from those over traffic lanes and those not reaching the roadway or over only parking lanes, turnout lanes, or shoulders.  Count individual cantilever units; do not count the flasher head pairs mounted on the units.

13.  Mast Mounted Flashing Lights:  Count all flashers on a single mast as one flasher.  Do not count flasher heads or a pair of flashing lights separately.

14./15.  Other Flashing Lights:  Flashing lights not in accordance with the latest       Manual on Uniform Traffic Control Devices (MUTCD) should be reported as "other flashing lights."

16.  Highway Traffic Signals:  Highway signals refer only to train activated red-amber-green signals that control street traffic over the crossing.  Do not count highway signals controlling a nearby intersection even if they are interconnected with the crossing devices.

17.  Wigwags:  Count all wigwags.

18.  Bells:  Count all bells if present, either alone or in conjunction with other protection.

Item 6C.  Type of Warning Device at Crossing, Specify Special Warning Device not Train Activated.

Enter the  type of special warning device which is not train activated.  Examples of special warning devices not train activated are:

a.	Manually operated signals and/or gates

b.	Train crew flagging the crossing

c.	Watchmen

d.	Floodlights.

For watchmen and for manually operated gates, the number of hours daily in effect should also be indicated.  For floodlighting, the number of masts with lights should be reported.  Only floodlighting which is distinctive from other ordinary street lighting in the immediate area by its intensity, light distribution, focus or color is to be reported.

Item 6D.  Type of Warning Device at Crossing, No Signs or Signals.

Enter a check if no signs or signals are present.

Item 7.  Is Commercial Power Available?

Enter a check in the appropriate box if there is commercial electric power available within 500 feet of the crossing.

Item 8.  Does Crossing Signal Provide Speed Selection for Trains?

Enter a check in the appropriate box if the signal is equipped with a device to provide a constant warning time for train operation at the speed range listed in Item 2B.  Enter a check in the N/A box (Not Applicable) if there are no automatic signals at the crossing.

NOTE:	The four-part inventory form (often referred to as the "green form") asks, "Does crossing signal provide speed selection for trains?".

This question will only apply to crossings with automatic active warning devices.  All other types of crossings (especially those with passive warning devices) should have the response "N/A" indicated.  Crossings using only block or island type circuitry to activate the active warning devices should have the response "NO" indicated.  For active warning device crossings, the response "YES" should be indicated if the track circuitry uses circuitry and electronic hardware which is termed as a "constant warning time (CWT) sensor," "predictor (Grade Crossing Predictor (GCP))," etc.

11

Item 9.  Method of Signalling for Train Operation: Is Track Equipped with Signals?

Enter a check in the appropriate box for whether the track has some type of automatic signals or interlocking to control train operations.

3.5  Part III  Physical Data

Item 1.  Type of Development.

Enter a check in the appropriate box which best describes the predominant type of development in the vicinity (up to 1000 feet) of the crossing based on the following categories.

1.  Open Space.  Undeveloped or sparsely developed, very lightly populated, agricultural.

2.  Residential.  Built-up residential area.

3.  Commercial.  Retail stores and businesses, offices, personal services.

4.  Industrial.  Manufacturing, construction, heavy products, factories, and warehouses.

5.  Institutional.  Schools, churches, hospitals, parks, and other community facilities.

Item 2.  Smallest Crossing Angle.

Enter a check in the appropriate box which most closely describes the smallest angle between the highway and the track.  (The angle may be estimated by eye or with a simple device, such as a protractor.)

Item 3.  Number of Traffic Lanes Crossing Railroad.

Enter the number of through traffic lanes crossing the track.  Do not include shoulders or lanes that are typically used for parking.

Item 4.  Are Truck Pullout Lanes Present?

Enter a check in the appropriate box for whether special added lanes are provided to accommodate vehicles required to stop at crossings.

Item 5.  Is Highway Paved.

Enter a check in the "Yes" box if the highway is paved with material on which pavement markings can be effectively maintained.  Enter a check in the "No" box if the highway surface is gravel, dirt, or has a surface treatment on which markings cannot be maintained.

Item 6.  Pavement Markings.

Enter a check in the appropriate box for each type of pavement marking present that conforms to the highway traffic manual (MUTCD).  If both stop lines and RR crossing symbols are present, check both boxes.  If neither stop lines nor RR crossing symbols are present, check "None."

Item 7.  Are RR Advance Warning Signs Present?

Enter a check in the appropriate box for whether there are advance warning signs present on any of the highway approaches.

Item 8.  Crossing Surface.

Enter a check in the appropriate box which most closely fits one of the following descriptions:

1.  Sectional Treated Timber.  Prefabricated units approximately 8 feet in length of treated timber individually installed and removable for maintenance and replacement purposes.

2.  Full Wood Plank.  Full wood plank surface, other than section treated timber, covering the entire crossing area above the crossties, made out of ties, boards, bridge ties, etc.

3.  Asphalt.  Asphalt surface over the entire crossing area or in the area between planks or other material forming flangeway openings, with or without single planks, or rubber on outside of running rails.

4.  Concrete Slab.  Precast concrete sections, installed and removable, individually, for maintenance and replacement purposes.

5.  Concrete Pavement.  Concrete surface which is continuous over the track area and is not removable except by destruction of the surface.

6.  Rubber.  Preformed rubber sections, installed and removable, individually, for maintenance and replacement purposes.

7.  Metal Sections.  Preformed sections of steel or other metal, installed and removable, individually, for maintenance and replacement purposes.

8.  Other Metal.  Complete coverage of the crossing area with railroad rails or other metal materials not removable in limited sectional units.

9.  Unconsolidated.  Ballast or other unconsolidated material placed above the tops of crossties, with or without planks on one or both sides of the running rails.

0.  Other Specify.  Surfaces other than the previous surfaces (1 - 9): structural foam, plastic, "high-tech" etc.

If there are multiple tracks and the tracks have different types of surfaces, choose lower grade surface material to indicate on the form.  (It doesn't increase the overall rideability of the crossing to have one surface better than the adjacent.)  A suggested order might be: concrete, rubber, wood, asphalt, unconsolidated, other.

Item 9.  Does Track Run Down A Street?

Enter a check in the appropriate box for whether the crossing involves the track running parallel to and within a street or highway.

Item 10.  Nearby Intersecting Highway?

Enter a check in the appropriate box for whether the street or highway at this crossing is intersected by another street or highway within 75 feet of this crossing.

3.6  Part IV  Highway Department Information

Item 1.  Highway System.

Enter the correct highway system code from the following Table 3-1.

The Highway System Codes for the National Highway-Rail Crossing Inventory File were revised as a result of the 1991 Intermodal Surface Transportation Efficiency Act, (ISTEA) Section 1006.  ISTEA requireD the redefinition of the National Highway System (NHS) which Congress officially approved.  The NHS includes  approximately 160,000 miles while the total Federal-Aid Highway (FAH) is approximately 953,000 miles (which includes the 160,000 mile NHS).  There are now three classifications for highway systems which are:

RXI Code

a.	National Highway System		 1 & 2

b.	Other Federal-Aid Highway	  	    3

c.	Non-Federal-Aid			    8

The National Crossing Inventory File uses this classification, but subdivides the National Highway System into "Interstate" and "Other."  On January 1, 1996, the old codes in the National Inventory File were automatically converted to the new Highway System Codes using the following process.

OLD CODES   			   NEW CODES

1  Interstate		-->		Interstate

2  Fed-Aid		-->		Other NHS

3  Fed-Aid Urban		-\

>		Other Fed-Aid Non NHS

4  Fed-Aid Secondary	-/

8  Non Fed-Aid		-->		Non Fed-Aid

While this procedure converted most of the records to the new categories, many will need to be checked and verified by the States, especially where the status of a highway has changed.  There is no change in the Functional Classification Codes.

The Highway System Codes are listed in the following table.

| Code | Definition | Included |
| --- | --- | --- |
| 1 | Interstate National Highway System | Interstate, rural, and urban |
| 2 | Other National Highway System | Other urban and rural principal arterial, non interstate (Congressional Approval, Sept. 30, 1995) |
| 3 | Other Federal-Aid Highway-Not NHS | Rural major collector and higher category, or urban collector and higher category, not part of NHS |
| 8 | Non-Federal-Aid | Local rural roads, rural minor collectors, and local urban city streets or any other non-Federal-Aid roadway |

Table 3-1.  Highway System Codes

Item 2.  Is Crossing on State Highway System.

Enter a check in the appropriate box for whether the crossing is on a State highway system.

Item 3.  Functional Classification of Road over Crossing.

Enter the appropriate code for the functional classification of the crossing highway that the State has determined in accordance with the Federal-Aid Highway Program Definitions.  The current functional classification codes are listed in Table 3-2.

| Category | Codes | Functional Classification |
| --- | --- | --- |
|  | 01 | Interstate |
|  | 02 | Other principal arterial |
| Rural | 06 | Minor arterial |
|  | 07 | Major collector |
|  | 08 | Minor collector |
|  | 09 | Local |
|  | 11 | Interstate |
|  | 12 | Other freeway and expressway |
| Urban | 14 | Other principal arterial |
|  | 16 | Minor arterial |
|  | 17 | Collector |
|  | 19 | Local |

Table 3-2.  Functional Classification Codes

NOTE:	The tens digit for the rural codes must be "0" and for urban must be "1".

12

Item 4.  Estimate AADT.

Enter the estimated present average daily traffic (total both directions) based on available traffic information.  A reasonable estimate of the AADT will be acceptable if actual traffic counts are not readily available.

Item 5.  Estimate Percent Trucks.

Enter the estimated percentage of trucks in the traffic stream.

4.0  INVENTORY UPDATE PROCEDURES

4.1  General

The procedures for updating the National Inventory File are applicable to public, private and pedestrian crossings, whether at grade or grade separated.  These procedures are designed to insure availability and use of an up-to-date highway-rail crossing data base with uniform and consistent data collection criteria and a uniformity in the procedures used by States and railroads.

The procedures are based on the concept that the State transportation agency should be the party who forwards all data item changes for any and all crossings to the FRA.  This is consistent with the sequence of steps followed during the initial inventory.  The steps are railroad to State to FRA.  Or in situations where the State agency (rather than a railroad) initiates crossing changes, the sequence is State to railroad to State to FRA.

The process requires a continuing, cooperative effort between the States and railroads because only one may have changes to report, yet both need to review and update their respective crossing records.  Channels of communication need to be established whereby such information is provided to the appropriate individuals in the railroad companies and the State transportation agencies.

There are five types of update formats which may be submitted.  These are:

a.	U.S. DOT-AAR Crossing Inventory Form

b.	Mass Update Form

c.	Inventory Computer Printout

d.	Magnetic Tape

e.	GX Computer Program

Examples of the different types of forms upon which changes and corrections may be submitted are shown in Figure 4-1.  At the top is the "Inventory Computer Printout," on the right in the middle of the page is the "U.S. DOT-AAR Crossing Inventory Form," and the fill-in-the-blanks Mass Update Form is at the bottom.  Section 5.0 of this manual addresses the use of the Mass Update Forms. Use of the Inventory Computer Printout for mark-up is discussed and illustrated at the end of this section.  Section 6.0 of this manual explains how updates may be submitted on magnetic tape.

Changes may also be submitted on floppy disk using the GX computer program.  If the changes are submitted on the GX disks by a railroad, the railroad should send a copy of the transmittal letter to the State (contact person) and the FRA to notify them that a change has been submitted via GX disk.  This notifies the State that in 2 or 3 months they can receive a

Figure 4-1. The Procedures/Forms Used for Reporting Changes

GX update of this material after it has been added to the National File.  The same applies to a State that submits changes.  The State should notify the railroads involved and the FRA by sending a copy of the transmittal letter.  Section 7.0 of this manual describes the use of the GX Programs.

This section, Section 4 of this manual, emphasizes the procedures involved for submitting updates via the U.S. DOT-AAR Crossing Inventory Form.

4.2  Inventory Form

The U.S. DOT-AAR Crossing Inventory Form FRA F6180.71 (OMB-004-R4039) (see Figure 1-1) is used for providing data to initiate new crossings or changes to the Highway-Rail Crossing Inventory.  The inventory forms are four-part forms with a self-carbon feature.  This form is used for reporting all types of changes, including the establishment of a new crossing, closing of an existing crossing, or changes in the characteristics of a crossing.  Detailed instructions for completing the form are given in Section 3.0.  The form does not provide space for comments.  Should comments or explanation regarding a crossing be considered necessary or useful, a separate sheet should be used and attached to the form.

While changes and corrections may be submitted using other formats, new crossings must always be submitted on the Inventory Form.  When Parts I, II and III have been completed by the railroad, the top three copies must be forwarded to the State for completion of Part IV.  It is suggested that FRA be sent a copy of the transmittal correspondence.

Railroads and State highway agencies may obtain needed forms from the FRA.  The address is:

Federal Railroad Administration

Office of Safety

Highway-Rail Crossing and Trespasser Programs Division

400 7th Street, S.W. (RRS-23)

Washington, D.C.  20590

4.3  Data Items

Each data element contained on the inventory form is considered to be one of three categories: administrative, physical, or operational.  The following tables contain the data elements comprising the three categories.  The tables also indicate the agency that is expected to be most aware of any changes to those data elements and which would normally initiate the update process.

Table 4-1 lists the administrative data elements, which pertain to the management and jurisdiction of the crossing.  Changes in administrative data elements (such as division, subdivision names, etc.) usually occur because of an administrative action by a railroad.  A State agency may also make decisions that would result in changes in certain administrative elements.  Thus, the appropriate agency should initiate the update process when changes occur.

| Item No. | Element Name | Agency |
| --- | --- | --- |
| I-4 | State | State Highway or Railroad |
| I-5 | County | State Highway or Railroad |
| I-6 | County Map Reference | State Highway or Railroad |
| I-7 | City | State Highway or Railroad |
| I-8 | Nearest City | State Highway or Railroad |
| I-9 | Highway Number | State Highway or Railroad |
| I-10 | Street or Road Name | State Highway or Railroad |
| IV-1 | Highway System | State Highway |
| IV-2 | Crossing on State System | State Highway |
| IV-3 | Functional Class | State Highway |
| I-1 | Railroad Company | Railroad |
| I-2 | Railroad Division | Railroad |
| I-3 | Railroad Subdivision | Railroad |
| I-11 | Railroad I.D. Number | Railroad |
| I-12 | Timetable Station | Railroad |
| I-13 | Branch or Line Name | Railroad |
| I-14 | Railroad Milepost | Railroad |
| II-5 | Another Railroad? | Railroad |
| I-15 | Pedestrian Crossing | State Highway or Railroad |
| I-16 | Private Vehicle Crossing | State Highway or Railroad |
| I-17 | Public Vehicle Crossing | State Highway or Railroad |

Table 4-1.  Administrative Data Elements

The physical items describe the crossing configuration.  Changes to physical characteristics generally occur as a result of construction activity by a railroad or State.  The authority for the work usually is in the form of a contract, work order, etc.  An update must be submitted by the proper agency when any of these data elements change.  Table 4-2 lists the physical data elements.

| Item No. | Element Name | Agency |
| --- | --- | --- |
| II-3 | Type, Number Tracks | Railroad |
| II-4 | Separate Track/Other Railroad | Railroad |
| II-6 | Type of Warning Device | Railroad or State Highway |
| II-7 | Commercial Power? | Railroad or State Highway |
| II-8 | Speed Selection Provided | Railroad |
| II-9 | Signals for Train Control | Railroad |
| III-8 | Crossing Surface | Railroad or State Highway |
| III-1 | Development | State Highway |
| III-2 | Crossing Angle | State Highway or Railroad |
| III-3 | Number Traffic Lanes | State Highway or Railroad |
| III-4 | Truck Pullout Lanes | State Highway |
| III-5 | Is Highway Paved? | State Highway or Railroad |
| III-6 | Pavement Markings | State Highway or Railroad |
| III-7 | Advance Warning Signs | State Highway |

Table 4-2.  Physical Data Elements

The operational items pertain to the use of the crossing by railroads.  It is recognized that the values of the operational data elements (e.g., number of trains, speeds, etc.) may change over a period of time.  Whenever the changes are made or become known, the data elements should be updated.  Table 4-3 lists the operational data elements.

| Item No. | Element Name | Agency |
| --- | --- | --- |
| II-1 | Daily Train Movements | Railroad |
| II-2 | Maximum Train Speed/Crossing | Railroad |
| IV-4 | Estimated AADT | State Highway |
| IV-5 | Estimated Percent Trucks | State Highway |

Table 4-3.  Operational Data Elements

4.4  Railroad and State Agency Update Submission Procedures

There are three situations which require the reporting of changes by a railroad or State highway agency.  These situations are as follows:

a.	When one or more of the physical, operational, or administrative characteristics of an existing crossing change,

b.	When a new crossing is opened, and

c.	When an existing crossing is closed.

IMPORTANT NOTE:	In all cases when an update form is prepared, the items in Table 4-4 must be provided in addition to the items being updated.

13

| Section | Item No. | Item |
| --- | --- | --- |
| Heading | A | Initiating agency |
| Heading | B | Crossing number |
| Heading | C | Type of update |
| Heading | D | Effective date |
| Part I | 1 | Railroad operating company |
| Part I | 4 | State |
| Part I | 5 | County |

Table 4-4.  Required Update Items

Only the data items being updated, i.e., those items for which a value is being changed from the existing records, are to be entered in the appropriate place on the form.  These items should then be circled.

The steps necessary to process an update are shown in Figures 4-2 and 4-3.  The primary responsibility for submitting the data changes to the FRA lies with the State agencies; however, the railroad has responsibility for submitting updates to the State.  The sequence for submitting updates is shown in Figures 4-2 and 4-3, depending upon whether the update is initiated by the railroad or the State agency.

RAILROAD INITIATED

Figure 4-2.  Railroad Initiated Update

STATE INITIATED

Figure 4-3.  State Agency Initiated Update

Depending upon the data element(s) involved (see Tables 4-1, 4-2, and 4-3), either a State or a railroad should initiate the update form.  Unless otherwise mutually agreed by the State and the railroad, the party initiating an update should be in accordance with these tables.

As has previously been explained, an update form should be initiated when one or more of the physical, operational, or administrative characteristics change (see Tables 4-1, 4-2, and 4-3).  Physical characteristic changes generally occur as a result of a construction activity.  Changes in administrative data elements usually occur because of an administrative action on the part of the State highway agency or a local jurisdiction.  Operational data elements may change significantly annually or over a period of time (e.g., traffic counts, percent trucks, pavement markings, etc.).  The procedures of the State should be such that these items are updated at least every 3 years.  However, whenever the changes become known, the data elements should be immediately updated.

When a new crossing is opened, Form FRA F 6180.71 must always be used to report the inventory information.  The process is the same as for the process just described for reporting changes.  The railroad/State needs to complete an update form for the new crossing and assign a valid crossing number.  Crossing numbers can be obtained from the FRA (see Section 2.0).  The railroad/State must also install the crossing identification number at the crossing and it is strongly recommended that it be installed on both sides of the crossing.  If the crossing is public, the form must be completely filled out.  If the crossing is private or grade separated, only Part I of the form must be completed.  The railroad/State should initiate the update by completing a form and routing it according to Figure 4-2 or 4-3, whichever applies.

The sequence of steps to be followed when a crossing is reported closed is the same as for the submissions for changes and additions.

When a railroad initiates an update submission, the flow process is as pictured in Figure 4-2.  The railroad will complete a four-part form, retaining the bottom (orange) copy, and forwarding the top three copies to the appropriate State agency (Step 1).  The State agency will review the form, adding any changes necessary, and return a copy (pink) to the railroad for use in updating its records (Step 4).  The top copy (green) will be forwarded by the State agency to FRA for processing into the National File.

The State agency should carefully review the forms to insure that the location information is correct and that the State concurs in the railroad's classification of the crossing as public or private.   The State agency and the railroad must reach agreement regarding the crossing classification, prior to forwarding the inventory form to the FRA.

The State highway agency should review other parts of the completed form for a new public crossing to insure that the data shown agrees with its records.

After the railroad has received the final copy (pink) of the completed four-part form set from the State highway agency, for all types of submissions (i.e., add, change, or delete), the railroad should update its records.  This may consist of placing the pink update form in its proper place in a file cabinet, and/or entering the information into a computerized data base.

When the State is responsible for initiating the update submission, the flow will be as shown in Figure 4-3.  The State will send three copies of the form to the railroad.  Whenever a form set (three copies) for a crossing is received by the railroad from a State agency, existing data elements for the crossing should be reviewed, particularly the railroad operational items, although other items also should be checked.  If it is known that the values for these items have changed, efforts should be made to determine the current values and they should be entered on the form.  Two copies (green and yellow) of the form will then be returned to the State agency, with one copy (pink) being retained for the railroad's permanent records.

This process provides the opportunity to update all data elements of a crossing that may have changed.  The items involved in these situations probably will be the operational items, but could include other administrative or physical items.  If there is any doubt about whether a change has been reported previously, it should be included at this time.

Routing of the four-part forms will be the same as just described for adds, changes, and deletions.

NOTE:	The State agency has the primary responsibility for submittal of forms to FRA, including the top (green) copy of the four-part form.

14

The top (green) copy of the four-part form should be sent to:

Federal Railroad Administration

Office of Safety

Highway-Rail Crossing and Trespasser Programs Division

400 7th Street, S.W. (RRS-23)

Washington, D.C.  20590

4.5  Summary of Completed Inventory Form Handling Procedures

Forms submitted by railroads and States need to have the changes circled in accordance with the instructions (reference Section 4.4 of this manual).  Please double check submittals to insure that any and all changed items are circled.

When the forms are completed and reviewed, including resolution of any discrepancies between railroad and State agency information, they are to be handled in the following manner:

a.	Initially, for railroad completed inventory forms, the railroad shall mail the top three copies to the State contact (see Appendix A).  If possible, include county maps with the crossing identified by location and number.  The railroad should keep the fourth (orange) copy for its interim records.

b.	Initially, when the State agency has completed the inventory form, they will mail the top three copies to the railroad.

c.	The final distribution of the color copies of the inventory form is as follows:

1.	Green Copy.  This copy is to be forwarded by the State agency to the FRA.

2.	Yellow Copy.  This copy is to be retained by the State DOT.

3.	Pink Copy.  This copy is to be retained by the railroad.

4.	Orange Copy.  This copy is to be retained temporarily by the railroad or State agency, whichever is initiating the update form, and destroyed when the permanent pink or yellow copy is received.

It is suggested that a cover letter accompany each group of update forms shipped.  This letter should include the total number of forms included in the mailing and broken down according to private, pedestrian, grade separation, and public at-grade crossings, and the crossing numbers submitted.

Completed inventory forms for transmittal to the FRA (green copies only) should be sent to the following address:

Federal Railroad Administration

Office of Safety

Highway-Rail Crossing and Trespasser Programs Division

400 7th Street, S.W. (RRS-23)

Washington, D.C.  20590

4.6  One-party Submissions

If a State or railroad initiates update forms and the other party does not review the forms and provide its update information within a reasonable time, the initiating party may make a one-party submission to the FRA.  Instead of sending the green, yellow, and pink copies of the update form to the other party, the green copy can be sent directly to the FRA and the yellow or pink copy, as appropriate, would be sent to the other party.  The letter of transmittal to the FRA should explain that it is a one-party submission and that the other party has been sent the appropriate copy of the update form.  However, this procedure should only be used if, after repeated attempts, the other party does not review and return forms within a reasonable time (usually 3 months maximum).

4.7  Inventory Computer Printout Mark-up

When there are a few simple corrections that need to be made, such corrections can be indicated directly on the Computer Inventory Printout, circled in red pen (or high-lighted), effective date indicated, and returned to the FRA for processing.  A copy needs to be sent to the appropriate State or railroad.

This procedure should only be used for direct numerical or word replacements and when only a few updates are involved.  An example of this process and the procedure is shown in Figure 4-3.

Figure 4-4 is a sample of what one State uses as a checklist of guidelines for their inspectors to use when marking up the computer printouts or completing the Inventory Forms.

4.8  Overview of Update Processing at FRA

All Inventory File update submissions are received by FRA, where a cursory review of the submissions is performed.  After the review is accomplished, FRA forwards the submissions by courier to the contractor for processing.  A flow chart showing an overview of the current GCIS processing is contained in Figure 4.5.  The major steps comprising the update processes are as follows:

	Updates to the Inventory are received either on hard copy format (inventory forms or fill-in-the-blanks) or magnetic media (9-track tape or GX format on diskettes).

	Hard copy updates are sorted and batched manually.  Updates on magnetic media are sorted and assigned batches programmatically.

	A rapid scan of the hard copy documents is performed to identify records which are ineligible for updating because of missing or invalid mandatory source data.

	Updates on inventory forms and printouts are key entered into the system.

	All source data is fully edited and validated and exceptions are "Suspensed" for resolution.

Figure 4-3.  Inventory Computer Printout Mark-Up Procedure

Figure 4-4.  Sample Guidelines and Checklist for Field Inspectors

	All manually input data is subjected to 100% sight verification to insure accuracy.

	Master data is uploaded to NIH for further processing.

	The data is submitted to the edit job stream.  Valid updates are posted to the Inventory and error records are "Suspensed" for resolution.

Figure 4-5.  Overview of Current GCIS Processing

3

5.0  MASS UPDATING

This section explains the process for submitting crossing changes via fill-in-the-blank computer printouts.

When a large number of crossings have undergone changes affecting the same few data elements for each crossing, computer generated mass update forms will be easier for supplying the information. Examples of these are:

a.	Changes in railroad operating company or railroad identification names

b.	Updates resulting from a statewide signing and marking program

c.	Changes in highway traffic counts

d.	Changes in train movements

e.	Changes in train speeds

f.	Crossing closures.

Examples of the types of printouts and data displays of highway-rail crossing data which are available to States, railroads, and others are located in Appendix B.  Included in the examples are several fill-in-the-blanks lists for a variety of data elements.

While there are many ways in which the data can be presented, this document provides certain standard formats which can be readily identified and requested by users.  Such requests can be expeditiously fulfilled because computer software programs exist to simplify the process.  Appendix B shows samples of the format displays with an identifying program name which can be used for making requests.  By requesting the program name, the users can be assured that they will receive the data in a form most useful to their needs.  The requestor should call the FRA to discuss various options before requesting data.

5.1  Fill-in-the-Blanks Computer Printout

A State or railroad desiring to use this method should contact or write the FRA to provide a fill-in-the-blanks printout containing the data elements which the State or railroad desires to update.  The printout shows the data currently contained in the National File for specific elements and has space to fill in data that have changed.  Another type of printout, the index, can be provided which will include identification elements such as the street name, county, etc.  Instructions for using the fill-in-the-blanks printout will also be provided.  The fill-in-the-blanks printout may be selected on and sorted in any manner desired.  An abbreviation key for most fill-in-the-blanks computer printouts is listed in Table 5-1.

Table 5-1.  Fill-in-the-Blanks Abbreviation Key

| Abbreviation | Explanation |
| --- | --- |
| CROSSING or ID NO. | U.S. DOT-AAR crossing number |
| EFFECTIVE DATE | Enter date when the crossing status changed |
| STATE or ST | State abbreviation |
| ST CD | FIPS State code |
| CNTY | FIPS county code |
| RR | Railroad operating company |
| HWY# | Highway type and number |
| STATUS | Crossing status (open or closed) |
| XBR | Number of crossbucks, reflectorized |
| XBNR | Number of crossbucks, nonreflectorized |
| FL OVR | Number of cantilevered flashing lights over traffic lanes |
| FL NOV | Number of cantilevered flashing lights not over traffic lanes |
| FL MST | Number of mast mounted flashing lights |
| GT RW | Number of gates, red and white reflectorized |
| GT OTH | Number of gates, other colored |
| STP STD | Number of standard highway stop signs |
| DAY-THRU | Typical number of daily daylight (6 a.m. to 6 p.m.) thru train movements |
| DAY-SWT | Typical number of daily daylight (6 a.m. to 6 p.m.) switching train movements |
| NGT-THRU | Typical number of daily night (6 p.m. to 6 a.m.) thru train movements |
| NGT-SWT | Typical number of daily night (6 p.m. to 6 a.m.) switching train movements |
| LT-1-MOV | Less than one train movement per day (Yes or No) |
| NX-TT-SPD | Maximum timetable speed |
| MIN-SPD | Minimum typical train speed |
| MAX-SPD | Maximum typical train speed |
| SPD-SEL | Crossing signal speed selection provided (Yes, No, or N/A) |
| TYPE or TYP-POS | Crossing type and position 1st number: 1-pedestrian, 2-private, or 3-public 2nd number: 1-at-grade, 2-RR under, 3-RR over The following is the key for the crossing types: 11 - Pedestrian at grade 12 - Pedestrian RR under 13 - Pedestrian RR over 21 - Private at grade 22 - Private RR under 23 - Private RR over 31 - Public at grade 32 - Public RR under 33 - Public RR over |
| WDCODE | Highway warning device class at crossing (Highest type is to be indicated) 8 - Gates 7 - Flashing lights 6 - Highway signals, wigwags, or bells 5 - Special protection 4 - Crossbucks 3 - Stop signs 2 - Other signs or signals 1 - None of the above |
| ******* under MILEPOST | Milepost has an alphabetical character and will not print out |

Table 5-1.  Fill-in-the-Blanks Abbreviation Key (Continued)

Examples of the "Fill-in-the-Blanks" printout, the index, and their instructions for updating crossbucks, pavement markings, advance warning signs, and the highway information are located in Sections 5.1.1, 5.1.2 and 5.1.3.  Any data element may be requested to be included or selected for the fill-in-the-blanks printout.

After completion, a photocopy of the printout should be provided to the other party, be it a State or a railroad, so that they will be informed of the updates made to their crossings.  It should also be indicated in the letter of transmittal to the FRA that the other party has been furnished with a copy.  After the updates are processed into the National File, a new updated list may be requested from the FRA to verify the corrections after three months from the date of the transmittal letter.

5.1.1  Train Counts and Train Speeds Updating

This example shows the process for updating the Train Counts and Train Speeds using the fill-in-the-blanks format.  The instructions for filling out the fill-in-the-blank form are listed in Table 5-2.  A copy of the Train Counts and Train Speeds update format is located in Figures 5-1 and 5-2.

| Column Heading | Information |
| --- | --- |
| CROSSING | Enter the DOT-AAR crossing identification number. |
| EFFECTIVE DATE | Enter the effective date of the changed information. |
| CNTY | Enter the county code. |
| ST | Enter the State code. |
| DAY THRU | Enter the number of day thru trains if this has changed. The data currently in the inventory is shown in the column "DT". |
| DAY SWT | Enter the number of day switching trains if this has changed. The data currently in the inventory is shown in the column "DS". |
| NGT THRU | Enter the number of night thru trains if this has changed. The data currently in the inventory is shown in the column "NT". |
| NGT SWT | Enter the number of night switching trains if this has changed. The data currently in the inventory is shown in the column "NS". |
| LT1 | This column is to be used in two circumstances. If there has been an increase from less than one train movement per day to one or more movements per day enter a "0". If there has been a decrease from one or more train movements per day to less than one movement per day enter a "1". The data currently in the inventory is shown in "LT1". |
| MAX TT | Enter the maximum timetable speed if there has been a change. The data currently in the inventory is shown in the column "MAX TT". |
| TYP MIN | Enter the typical minimum train speed if there has been a change. The data currently in the inventory is shown in the column "TYP MIN". |
| TYP MAX | Enter the typical maximum train speed if there has been a change. The data currently in the inventory is shown in the column "TYP MAX". |

Table 5-2.  Train Counts and Train Speeds Updating Form Instructions

DULUTH MISSABE AND IRON RANGE RAILWAY COMPANY                                      PAGE    1

FILL-IN-THE-BLANKS LIST                                                  12/22/93

FOR RAILROAD, CROSSING STATUS, WARNING DEVICES, TRAIN COUNTS, AND SPEEDS

CROSSING            STATE CNTY                                                                              LT1   MAX

COUNTY    EFFECTIVE BRANCH           MILEPOST    XB   FL   FL   FL   GT   GT   STP  DAY   DAY   NGT   NGT   PER   TT    MIN   MAX

ROAD      DATE       HWY #    RR          XBR  NR   OVR  NOV  MST  RW   OTH  STD  THRU  SWT   THRU  SWT   DAY   SPD   SPD   SPD

------------------------------------------------------------------------------------------------------------------------------------

ON Duluth, Missabe & Iron Range Railway Com

252245L   _________  MN   075  DMIR  000495  2    0    0    0    0    0    0    0     2     0     2     0   NO     22    10    22

LAKE                LOOP

____  ______ ___  ___  ___  ___  ___  ___  ___  ___  ____  ____  ____  ____  ____  ____  ____  ____

252247A   _________  MN   075  DMIR  000547  2    0    0    0    0    0    0    2     3     0     3     0   NO     22    10    22

LAKE                LOOP

CO 33    ____  ______ ___  ___  ___  ___  ___  ___  ___  ___  ____  ____  ____  ____  ____  ____  ____  ____

252248G   _________  MN   075  DMIR  002903  2    0    0    0    0    0    0    2     3     0     3     0   NO     35     5    35

LAKE                MAIN

UT 33    ____  ______ ___  ___  ___  ___  ___  ___  ___  ___  ____  ____  ____  ____  ____  ____  ____  ____

252249N   _________  MN   075  DMIR  002960  0    0    0    0    2    2    0    0     3     0     3     0   NO     35     5    35

LAKE                MAIN

CO 122   ____  ______ ___  ___  ___  ___  ___  ___  ___  ___  ____  ____  ____  ____  ____  ____  ____  ____

252250H   _________  MN   075  DMIR  003078  2    0    0    0    0    0    0    2     6     0     6     0   NO     35    20    35

LAKE                MAIN

CO 12    ____  ______ ___  ___  ___  ___  ___  ___  ___  ___  ____  ____  ____  ____  ____  ____  ____  ____

252251P   _________  MN   075  DMIR  003221  2    0    0    0    0    0    0    2     6     0     6     0   NO     35    20    35

LAKE                MAIN

CO 13    ____  ______ ___  ___  ___  ___  ___  ___  ___  ___  ____  ____  ____  ____  ____  ____  ____  ____

252252W   _________  MN   075  DMIR  003304  2    0    0    0    0    0    0    2     6     0     6     0   NO     35    20    35

LAKE                MAIN

CO 38    ____  ______ ___  ___  ___  ___  ___  ___  ___  ___  ____  ____  ____  ____  ____  ____  ____  ____

Figure 5-1.  Train Counts and Train Speeds,

Plus Warning Device and Crossing Status Format (Sample)

DULUTH MISSABE AND IRON RANGE RAILWAY COMPANY                                      PAGE    1

FILL-IN-THE-BLANKS LIST                                                  01/07/94

FOR TRAIN MOVEMENTS AND SPEEDS THRU PUBLIC-AT-GRADE CROSSINGS

CROSSING  EFFECTIVE  STATE  CNTY   RR   DIVISION       WDCODE     DAY-THRU  DAY-SWT  NGT-THRU   NGT-SWT   LT-1-MOV        NOTES

DATE                        SUBDIVISION    MILEPOST                                                            AND

BRANCH                    MX-TT-SP  MIN-SPD  MAX-SPD    SPD-SEL                 COMMENTS

------------------------------------------------------------------------------------------------------------------------------------

252239H   _________    27    075  DMIR                    7       3  ____   0  ____   3  ____   0  ____   NO  ____

MN               IRON RANGE         2.40

LOOP                     20  ____  10  ____  20  ____  NO  ____

252241J   _________    27    075  DMIR                    7       3  ____   0  ____   3  ____   0  ____   NO  ____

MN               IRON RANGE         3.14

LOOP                     20  ____  10  ____  20  ____  NO  ____

252242R   _________    27    075  DMIR                    4       3  ____   0  ____   3  ____   0  ____   NO  ____

MN               IRON RANGE         4.19

LOOP                     22  ____  10  ____  22  ____  N/A ____

252245L   _________    27    075  DMIR                    4       2  ____   0  ____   2  ____   0  ____   NO  ____

MN               IRON RANGE         4.95

LOOP                     22  ____  10  ____  22  ____  N/A ____

252247A   _________    27    075  DMIR                    4       3  ____   0  ____   3  ____   0  ____   NO  ____

MN               IRON RANGE         5.47

LOOP                     22  ____  10  ____  22  ____  N/A ____

252234Y   _________    27    075  DMIR                    4       0  ____   0  ____   0  ____   0  ____  YES  ____

MN               IRON RANGE      ********

MAIN                      5  ____   1  ____   5  ____  N/A ____

252236M   _________    27    075  DMIR                    4       0  ____   0  ____   0  ____   0  ____  YES  ____

MN               IRON RANGE      ********

MAIN                      5  ____   1  ____   5  ____  N/A ____

Figure 5-2.  Train Counts and Train Speeds Format (Sample)

5.1.2  Signs, Markings, and Part IV State Highway Information

This example shows the process for updating Highway Signs, Markings, and other Highway Department Information using the fill-in-the-blanks form.  The instructions for filling out the fill-in-the-blanks form are listed in Table 5-3.  A sample of the "Fill-in-the-Blanks List for Updating Highway Information" is located in Figure 5-3.

| Column Heading | Information |
| --- | --- |
| CROSSING | The DOT-AAR crossing identification number. |
| EFFECTIVE DATE | Enter the effective date of the changed information. |
| ST | The State code. |
| CNTY | The county code. |
| RAILROAD OLD | The railroad code as it existed in the Inventory File when the report was run. |
| RAILROAD NEW/ OR CLOSED | Enter the railroad code if it has changed, or enter "Closed" if the crossing no longer exists. |
| TYPE | The type of crossing based on Table 5-1. |
| COUNTY CITY STREET | The values for County, City, and Street as reflected in the Inventory File. |
| WD | Enter the appropriate highway warning device class, in accordance with Table 5-1, if there has been a change. The code currently in the Inventory File is shown in this column. |
| TRAF LANE | Enter the number of through traffic lanes crossing the track, if there has been a change. The number currently contained in the Inventory File is shown in this column. |
| HWY PAVED | Enter whether or not the highway is paved, according to Section 3 of this manual, if there has been a change. The value of this item currently in the Inventory is shown in this column. |
| PAVE MARK | Enter the appropriate code for pavement markings, from the codes on the bottom of the report, if there has been a change. The number currently in the Inventory is shown in the this column. |
| ADV WARN | Enter the appropriate code, or "YES"/"NO" for advance warning signs if there has been a change. The number or decode name currently in the Inventory is shown in this column. The codes are as follows: 1 - Advance warning signs present on either highway approach, and 2 - No advance warning signs present. |
| XING SURF | Enter the appropriate code for the highway crossing surface, if there has been a change. The code currently contained for this data element in the Inventory is shown in this column. |
| HWY SYS | Enter the appropriate highway system code, in accordance with Section 3 of this manual, if there has been a change. The number currently in the Inventory is shown in this column. |
| ON ST HWY | Enter the appropriate State highway indicator code if there has been a change. The number currently in the Inventory is shown in this column. |
| HWY CLASS | Enter the suitable highway functional classification code, according to Section 3 of this manual, if there has been a change. The number currently in the Inventory is shown in this column. |
| AADT | Enter the traffic count if there has been a change. The value currently in the Inventory is shown in the column "AADT". |
| PCT TRUK | Enter the estimated percentage of trucks in the traffic stream, if there has been a change. The value currently in the Inventory is shown in this column. |

Table 5-3.  Instructions for Updating Highway Information

STATE OF ALASKA                                                      PAGE    1

FILL-IN-THE-BLANKS LIST FOR UPDATING HIGHWAY INFORMATION                                 01/07/94

ON THE Alaska Railroad Corporation

EFFECTIVE            RAILROAD       COUNTY

CROSSING   DATE     ST CNTY  OLD NEW/OR TYPE CITY                  WD  TRAF  HWY   PAVE  ADV   XING  HWY  ON ST  HWY         PCT

CLOSED      STREET                    LANE PAVED  MARK  WARN  SURF  SYS   HWY  CLASS  AADT  TRUK

------------------------------------------------------------------------------------------------------------------------------------

868224X  _________  AK  122  ARR         31  KENAI PENINSULA        4    2   YES     3   YES     4     04  YES     07   1415   20

_______      IN   SEWARD

DOCK RD              ____ ____  ____  ____  ____  ____  ____  ____  ____  _____  ____

868225E  _________  AK  122  ARR         31  KENAI PENINSULA        4    2   YES     3   YES     4     04  YES     07   1415   20

_______      IN   SEWARD

DOCK RD              ____ ____  ____  ____  ____  ____  ____  ____  ____  _____  ____

868226L  _________  AK  122  ARR         31  KENAI PENINSULA        4    2   NO      3   YES     2     08  NO      09    200   20

_______      IN   SEWARD

SKILL CENTER         ____ ____  ____  ____  ____  ____  ____  ____  ____  _____  ____

NONE

868228A  _________  AK  122  ARR         31  KENAI PENINSULA        4    2   NO      3   YES     2     08  YES     08    200    2

_______      NEAR SEWARD

AIRPORT ROAD         ____ ____  ____  ____  ____  ____  ____  ____  ____  _____  ____

868229G  _________  AK  122  ARR         31  KENAI PENINSULA        7    2   YES     2   YES     6     04  YES     07   1800    2

_______      NEAR SEWARD

NASH RD              ____ ____  ____  ____  ____  ____  ____  ____  ____  _____  ____

868231H  _________  AK  122  ARR         31  KENAI PENINSULA        4    2   NO      3   NO      2     08  NO      09    100    2

_______      NEAR SEWARD

LAKE DRIVE           ____ ____  ____  ____  ____  ____  ____  ____  ____  _____  ____

------------------------------------------------------------------------------------------------------------------------------------

* THIS DATA WAS PRODUCED BY THE FEDERAL RAILROAD ADMINISTRATION WITH INFORMATION SUPPLIED BY FEBRUARY 28, 1992

* KEY FOR PAVE-MARK: (1) STOPLINES, (2) RR XING SYM, (3) NONE,  (4) BOTH

* KEY FOR WD (WARNING DEVICE) AND XING-SURF: REFER TO MANUAL OR FORM

Figure 5-3.  Signs, Markings, and Part IV State Update Form

5.1.3  Other Examples

On the "Fill-in-the-Blanks List for Indicating Sold Crossings," (see example in Appendix B) the railroad needs to insert the acquisition date under the column "DATE SOLD" and the abbreviation for the railroad under the column "RAILROAD NEW" for each crossing that has changed ownership.  Return the copy to FRA and send a copy to the State and the former or new railroad.  Make additional changes or updates by using the standard four-part U.S. DOT-AAR Crossing Inventory forms.  A copy of the detailed inventory information for each crossing may be requested if assistance is needed to identify specific crossings.

The "Fill-in-the-Blanks List for Indicating Closed Crossings" (see example in Appendix B) should be used to show the closure of a crossing by entering the effective date under the column "DATE CLOSED."  The objective is to identify the status (closed or open) of any such crossings belonging to the reporting railroad.  Crossings along a specific line should be together since the data is organized by railroad, division, subdivision, branch, and milepost.  If there are any names that are incorrect or not listed consistently the same, correct them on the line below the name.

5.2  Mass Update Requirements

The following Steps are requirements if a State or railroad prepares their own Mass Update form.

a.	Skip one line between each line of printed data.

b.	The first five left-hand columns in order must be as follows:

1.	DOT-AAR crossing number

2.	Effective date

3.	State code (FIPS)

4.	County code (FIPS)

5.	Railroad code.

NOTE:	If all data submitted is for the same State, railroad, or date, then that particular column of data can be omitted by indicating it at the top of the page.

15

c.	The remainder of the old and new data should be evenly distributed in columns across the page.

d.	The "effective date" of the changed information is imperative.  This date can refer to either the date that the change became effective or the date that the change was first reported.  It is undesirable to use the same effective date for submittals for crossing changes and data which may have been accumulated over several months or even a year.  It is the date when the data changed for the crossing, i.e., if flashing lights were installed at the crossing on December 1, 1995, the effective date of the change is 12/1/95 and written 120195.  The date must be in month-day-year (MMDDYY) format, e.g., 030895 for March 3, 1995.  Enter the effective date in the blank under the column "Effective Date."

e.	Fields are limited to those fields with write-in changes provided for each crossing being updated using a particular mass update printout.  Changes to fields other than those on a mass update printout must be submitted in another manner, i.e., on a four-part form, another mass update printout with provisions for those fields needing updating, magnetic tape, letter, etc.

f.	When sorting and listing crossings on a mass update printout, crossings should be arranged at least by State and railroad.

g.	If the crossing is not on the railroad which is indicated, enter the name or code of the correct railroad on the blank under the column heading "NEW RR," if provided.  Otherwise, submit a four-part form.

h.	Fill in the remainder of the blanks with the new data and cross out the old data (preferably with red pencil) only where changes are required.  If the present data is correct, there is no need to indicate this information.

i.	If the inventory data on the report varies from what exists in the field, please provide the correct information for updating the National Inventory File.  Mass update computer printouts may be requested for the categories and formats contained in "A Sample of Computer Printout Formats Available."  Changes may also be submitted on the U.S. DOT-AAR Crossing Inventory forms (often referred to as the "green forms").  All such submittals should be checked to insure that changes have been circled and that all other information is correct.

j.	New crossings must always be submitted on the four-part Inventory forms.  When Parts I, II and III have been completed by the railroad, the top three copies must be forwarded to the State for completion of Part IV.  The FRA should be sent a copy of the transmittal correspondence.

k.	After the list is completed and reviewed, a copy of the sheets containing changes needs to be sent to the State or railroad, and a copy to the Federal Railroad Administration for processing the changes into the National Inventory File.  Be sure to retain a copy for your records.

5.3  Special Procedures for Certain Items

A list containing the crossing number, effective date, railroad, State, county and the changed data will be sufficient to update certain data elements, as specified in the following:

a.	Railroads with changes for a large number of crossings involving any or all of the following:

1.	Railroad operating company

2.	Railroad division

3.	Railroad subdivision

4.	Railroad ID number

5.	Branch or line name

6.	Railroad mile post.

b.	States with changes for a large number of crossings, involving any or all of the following data items:

1.	County map reference number

2.	Street or road name

3.	Highway type and number

4.	AADT.

A State or railroad making such a submission needs to also provide the other party, State or railroad, with a list of the changes.

5.4  Feedback of Updated Records from the National File

Once the updates have been applied to the National File, the State or railroad may request the updated records from the FRA.  The updated records can be provided approximately 3 months after transmittal in one of the following four ways:

a.	Computer-Generated Form.  Upon request, the FRA can make information available on a one-page (white) computer-generated update form which is identical to the standard update form.

b.	One Page Per Crossing Printout.  This printout (Figure 5-4) is used to provide the complete detailed inventory information about a crossing from the National File.

c.	Computer Tape.  A reel-to-reel magnetic tape can be provided of all desired crossings.

Figure 5-4.  Inventory Crossing Printout (Sample) ("One-page-per-crossing")

d.	GX.  GX is a self-contained database package which allows the user to retrieve and display records, update them, print both comprehensive and summary reports and produce magnetic updates for submission to the FRA.

5.5  Continuous Feed Update Forms

The FRA can make available upon request a continuous feed single copy (white) or four-copy update form for those States and railroads desiring such.  This form is similar to the standard four-part update form and consists of either a single sheet white copy or the usual four colored copies (green, yellow, pink, and orange) in order to allow for standard processing.

5.6  Common Errors in Updating

The following paragraphs describe the most common types of errors in updating the inventory which result in processing delays.

a.	Problem: Incomplete "NEW CROSSING" forms.  This is a very common problem with "new" Public-at-Grade crossings and changes from private or grade separated crossings to Public-at-Grade.  Often a railroad will submit the inventory form with only Parts I through III completed.  Sometimes a State will forward a "new crossing" without Part IV information.  These situations often result in the crossing record being held in "suspense" until the information is received.

Resolution.  "New" Public-at-Grade crossings or re-classifications to Public-at-Grade must always be handled as two-party transactions where both the States and railroads provide the information each is responsible for.  An exception would be if the party completing the form has all of the information required.  Part IV information MUST be completed before the new crossing can be entered into the National File.

b.	Problem: Incomplete information for changes or closings.  This problem occurs when the updating agency does not provide sufficient administrative data regarding the crossing to either close or update it.

Resolution.  Regardless of whether the update is for changing crossing characteristics or for closing a particular crossing, the update must have, at a minimum, the following in order for FRA to process the information:

1.	Initiating agency

2.	Crossing number

3.	Reason for update

4.	Effective date

5.	State

6.	County

7.	Railroad.

To insure data integrity, these elements are compared against the current inventory record for the crossing.

When changing State, Railroad, or County information, be sure to circle the data in these fields.  Indicate the "old" data for these fields outside the circle.

c.	Problem: More than one type of crossing identified.  Sometimes the submitting agency will check more than one type of crossing (Public, Private, Pedestrian).

Resolution. There can only be one type of crossing.  Refer to Section 1.5 of this manual for the correct definitions.

d.	Problem: Invalid Crossing I.D. number reported.  The check character in the Crossing I.D. does not correspond to the numeric portion of the I.D.

Resolution.  There is only one valid alphabetic check character for each sequence of numbers for the crossing I.D.  Refer to Section 2.2 of this manual for the algorithm to compute the valid check character.

e.	Problem: Data fails validation and edit checks.  To insure data integrity, certain information in the update records are validated against established tables.  These include:

1.	State, County, City - The information provided for these fields is validated against the Federal Information Processing Standard (FIPS) for Worldwide Geographic Location Codes.  Refer to Section 10.2 of this manual for information on obtaining this reference data.

2.	Nearest Timetable Station - The data for this field is validated against the DOT-AAR Standard Point Location Code inventory of railroad timetable stations.

3.	Railroad Operating Company, Other Railroads Operating Separate Track and Other Railroads Operating Over Same Track - The codes provided for these fields are validated against the U.S. DOT-AAR table of railroad codes.

Aside from above validations, FRA performs data-to-data logic checks within each record.  These include:

1.	If the Sum of all Daily Train Movements is greater than 1, it is invalid for the Less than One Movement Per Day field to be checked.

2.	If Signs and Train Activated Devices are identified in Part II-6A through 6B, it is invalid for the No Signs or Signals field to be checked.

3.	The Typical Minimum Speed can not exceed the Typical Maximum Speed over the crossing.  The Typical Maximum Speed over the crossing can not exceed the Maximum Timetable Speed.

4.	Train Activated Devices must be identified in Part II-6B in order for crossing to provide Speed Selection for Trains in Part II-8.

5.	The total number of Tracks must be greater than zero.

6.	The total number of Traffic Lanes must be greater than zero.

7.	For Pavement Markings (Railroad Crossing Symbols and/or Stoplines) to be valid, Is Highway Paved? must be "Yes".

5.7  Handling of Errors on Updates

In cases where the update fails the edit and the data reported is either inconsistent or incomplete, the form will be returned to the State for correction.  If the correction involves railroad initiated data, it is requested that the State contact the railroad for the correction.  This will insure that the updating process remains within the bounds which the FRA and its contractor can effectively administer.

If requested corrections have not been received within 30 days, telephone contact will be made.  If corrections still have not been received in 30 more days, the update will be placed in a permanent hold status.  Both the State and the railroad will be notified of this action. If the corrections are then received, the update will be reactivated and processed.

6.0  MAGNETIC TAPE

A State or railroad with computer facilities may elect to provide updates on reel-to-reel magnetic tape if they develop a computer program to format these updates in the FRA prescribed format.  This format is presented and explained in Appendix C of this document.  Before starting, the State or railroad should contact the FRA to make the necessary arrangements.  A State or railroad which is preparing its own tape and submitting updates to the FRA in the prescribed format should also provide a printout of the updated records to the other party, either State or railroad.

The following is a list of guidelines for submission of updates to the National Highway-Rail Grade Crossing Inventory on reel-to-reel magnetic tape:

a.	Submit the updates on a nine-track tape.  Densities of 1600 BPI and 3200 BPI are recommended although a 6250 BPI tape may also be submitted.  The data format may be ASCII or EBCDIC, and the tapes may be labelled or unlabelled.

b.	Enclose a cover letter describing the tape's characteristics, including the number of 80-character records it contains and the number of crossings to be  updated.  If possible, include a printed dump of the first block of data on the tape.

c.	Use as much of each 80-character record as possible.  The format of DATA ELEMENT NUMBER, COMMA, DATA, SLASH allows several data fields to be strung together within the same record.  If it is necessary to use multiple lines for a crossing, duplicate the identification data (columns 1 through 25) into the next line and continue with the data beginning in column 26.

d.	There can be only one update per grade crossing per submission.  As many fields as possible may be changed within a given update for a crossing but the system only allows for a crossing to be updated once in a given run.  For example, two updates for the same crossing: one dated 1/1/95 and the other dated 1/1/96, should not be submitted simultaneously even if they are updating different fields.  The Effective Date is part of the identification data.

e.	Contact AMB Associates, Inc. at (301) 587-9439, extension 222 regarding any questions related to this process, or FRA if the telephone number is no longer in operation.

The field specifications for each data element within the inventory record are contained in Appendix C.  The tables in Appendix C describe the length of each field as well as the valid range of values for the data.  Sample update records are also provided in Appendix C.

7.0  GX DATA MANAGEMENT SYSTEM

GX (which stands for Grade crossing or Xing) is FRA's highway-rail crossing data maintenance system.  It is intended for use by States and railroads to maintain their crossing inventory as well as to send updates to the FRA.  Use of this program makes updating very simple and reduces the need for large quantities of paper updates.

GX is a self-contained database package which allows the user to retrieve and display records, update them, print both comprehensive and summary reports, produce updates on magnetic diskette for submission to the FRA and perform administrative functions such as backups of the data.  GX is written and compiled in Clipper and does not require a run-time database package (like dBASE III+) to execute.  In simple terms, all that is needed is a PC running DOS 3.3 or higher in order to run GX.

GX was developed to enhance reporting and updating of crossing information for the National Inventory File.  FRA is making GX available at no cost only to interested railroads and State agencies.  GX can be used to update or close existing crossings or re-open closed crossings.  However, new crossing records must always be submitted on the standard DOT-AAR four-part form.  If a railroad is updating more than 30 crossing records, it will probably be more efficient to use this program.  If less than 30 crossing records are to be updated, using the four-part (green) forms is probably easier, although the railroad may still obtain the program for reference.

Each GX package includes a custom database containing only the requesting agency's crossings, reference files (cities, counties, railroads, etc.), the necessary programs to run the system, and GX documentation.  The database structure is listed in Appendix D.  GX users can make updates to their databases as often as they desire.  Periodically (at least once every quarter), the user should extract all changes for submission to the FRA to update the National Inventory.  In addition to the extensive edit and validation checks which are incorporated into GX, the incoming updates will also be edited against the National Inventory to insure its accuracy.  Updates which do not pass the edits will be returned to the submitting agency for review and reprocessing.

Also, GX will only accept changes in railroad ownership where the current railroad alpha code is in the package reference files. Some newly created railroads may not have their code in the reference file, and therefore GX will not accept a change in ownership.  These changes must then be submitted manually or the user will need to wait until a new package can be obtained with the newly created railroad's code contained therein.  GX users who wish to add new codes which are not presently in their Railroad file should contact AMB Associates to request a replacement file.

To install and run GX on a PC, the following is needed:

a.	An IBM PC or compatible computer, preferably with an 80286 or higher processor

b.	A color card/monitor (CGA, EGA, VGA, or SVGA)

c.	A minimum of 4 Mb of free hard disk space (storage requirements vary with the size of the user's files)

d.	640 Kb RAM minimum

e.	A high density diskette drive (either 3.5 inch, 1.44 Mb or 5.25 inch, 1.2 Mb)

f.	MS-DOS version 3.3 or later.

Although GX has been installed and tested on an IBM XT with a monochrome monitor, it is strongly recommended that the user have at least an AT-class computer with a color card/monitor to gain full use of the program's capabilities.

If interested in obtaining a custom GX package, please contact the GX Program Manager, at (301) 587-9439, extension 222; Fax telephone (301) 587-9442; or write (requests must be on letterhead stationery) to:

AMB Associates, Inc.

FRA Project/GX Program

818 Roeder Road, Suite 500

Silver Spring, Maryland  20910

NOTE:	While GX is provided to interested States and railroads free of charge, use of this product is purely voluntary.  No warranties relating to the use of this product are expressed or implied.  Neither the Government nor AMB Associates, Inc. is responsible for any damage to data or hardware which may arise from the use of this product.

16

The following sections explain some of the functions of the GX program.

7.1  Installation Program

GX provides an easy-to-use installation routine which will automatically update the computer's config.sys and autoexec.bat files to include the necessary environment variables to run the program.  The GX programs and data files are shipped in compressed format and are expanded during the installation process.  As many as 10,000 crossings can be provided to the user on a single high density diskette.

7.2  Browse Facility

GX incorporates a convenient Browse facility which allows the user to view groups of crossings in tabular form.  This feature is useful in identifying crossings by other characteristics such as Street Name when the Crossing ID is not known.  GX also includes a versatile interface to this facility which lets the user preselect the criteria for the crossings to be displayed.  The user can view all crossings or just a subset (public-at-grade only, active crossings only, etc., or various combinations) and can dictate the order in which the records will be displayed (in crossing order, in railroad order, or in State/county order).  The Browse facility also provides "Hot-Key" functions which allow the user to print reports and to perform searches by simply positioning the cursor to the appropriate field and pressing a combination of two keys.

7.3  Update Facility

GX provides a means for the user to view, update, or close a full crossing record at a time.  A total of four screens display the complete, most recent inventory record for the requested crossing.  GX incorporates the same extensive edit and validation checks used in processing the hard copy documents.  Table look-ups are performed to validate Railroad Codes (see Appendix E for a complete list of railroad codes), City and County Names, and Timetable Station Names.  In addition, the program executes several logic checks in comparing two or more fields within each record.

The latest version of GX provides a mass update facility which allows the user to make global changes to a group of selected crossings.

7.4  Report Facility

GX allows the user to select from several report formats.  A detailed full crossing profile, which can also be generated from both the Browse and Update facilities, is available.  Summary report formats providing counts of crossings by type or by railroad can also be generated by the user.

7.5  Data Extraction Facility

GX provides an automated routine which allows the users to extract all updates performed on their database and store them on diskette for submission to the FRA.

A future capability of the GX package will be the generation of read-only copies of the database by any combination and sort sequence of State, county, city, railroad, or any subset of the database for distribution to the recipient.

7.6  Backup and Restore Facility

GX provides administrative programs which let the user backup all changes made to the database and, in the event of a catastrophic loss of data, restore the database to its state at the time of the last backup.

7.7  Documentation

A User's Manual is included with the GX package when it is distributed for use.

8.0  SYSTEM OUTPUTS

Appendix B contains examples of the types of printouts and data displays of highway-rail crossing data which are available to States, railroads, and others.  Included in the samples are the following:

a.	Outputs from the Accident Prediction Programs.

b.	Outputs from the Resource Allocation Programs.

c.	Several examples from programs which count the number of crossings for various purposes.  These outputs may be produced for a State, railroad, or county and sorted as required.

d.	Listings of Inventory data.  These reports may be generated for a State or a railroad and sorted as desired.  Specific selection criteria may vary and include such as "all crossings with no warning devices," "all crossings with gates," "all private crossings," "all crossings with no pavement markings," etc.

e.	An Accident/Incident Report.

f.	A number of examples of fill-in-the-blanks lists for mass updates.

g.	Special information reports.

NOTE:	In addition to the examples presented in this document, other specially tailored reports may be obtained by direct request from FRA.

17

9.0  DOT HIGHWAY-RAIL CROSSING ACCIDENT PREDICTION

AND RESOURCE ALLOCATION PROCEDURE

The DOT Highway-Rail Crossing accident prediction formula is an accident and severity prediction calculation which computes the expected number of accidents at a crossing, based on information from the U.S. DOT-AAR National Highway-Rail Crossing Inventory and the Railroad Accident Reporting System (RAIRS) data files.

The DOT Highway-Rail Crossing resource allocation procedure is a computer model designed to nominate crossings for improvement consideration on a cost-effective basis and to suggest the type of warning device to be installed, given the cost of crossing improvements and an available budget level.

A number of crossing hazard formulas have been developed and used extensively in dealing with solutions to highway-rail crossing safety problems.  The DOT accident prediction formula is based on the extensive data in the DOT Crossing Inventory and Accident data files, and is an improvement over other hazard formulas.

9.1  Background

The Highway Safety Acts of 1973 and 1976, the Surface Transportation Assistance Acts of 1978 and 1982, the Surface Transportation and Uniform Relocation Assistance Act of 1987, and the Intermodal Surface Transportation Efficiency Act of 1991 provide funding authorizations to individual States to improve safety at public highway-rail crossings.  The installation of active motorist warning devices, such as flashing lights or flashing lights with gates, is an important part of crossing safety improvements.  The U.S. Department of Transportation (DOT) assists States and railroads in determining effective allocations of Federal funds for highway-rail crossing safety improvements by use of the resource allocation procedure developed to assist in the allocation of funds among crossings to achieve maximum crossing safety benefits for a given level of funding.

The procedure consists of two parts.  The first is an accident and severity prediction formula which computes the expected number of accidents at each crossing, based on information from the U.S. DOT-AAR National Highway-Rail Crossing Inventory and the Railroad Accident/Incident Reporting System (RAIRS).  The second part is a resource allocation model designed to nominate crossings for improvement consideration on a cost-effective basis and to suggest the type of warning device to be installed.

The DOT Highway-Rail Crossing accident prediction formula and resource allocation model were developed at the Transportation Systems Center (TSC) under the sponsorship of the Federal Railroad Administration (FRA) Office of Safety Analysis and the Federal Highway Administration (FHWA) Office of Research.  When used together, these procedures provide a systematic means of assisting in making a preliminary, optimum allocation of funds among individual crossings, considering available improvement options.  These procedures provide a ranked listing of crossings which can then be used as a guide for selecting crossings for on-site visits by diagnostic teams.

The formula and procedures were reviewed and slightly revised in 1986 and 1987.  While some improvements were implemented, the basic formulas remain the same as those originally developed in 1976.  A subsequent review is planned for the 1996-1997 period.

9.2  DOT Accident Prediction Formula

The availability of both inventory and accident data for crossings influenced the development of the DOT accident prediction formula.  This formula calculates the expected annual number of accidents at a crossing on the basis of physical and operational characteristics of the crossing as described in the Inventory and the most recent five year accident experience at that crossing as contained in the FRA Railroad Accident/Incident data file.

Three formulas are used to calculate predicted accidents:  a basic formula which contains factors from the Crossing Inventory, a second formula which incorporates accident history as an explicit factor, and a third formula which involves a normalizing constant.  The three formulas, given in a general form, are shown in equations [1], [2], and [3], respectively.  The output of equation [1] is an input to equation [2].  The output of equation [2] is the input to equation [3].  The output of equation [3] is the predicted accidents per year for the crossing of interest.

a = K x EI x DT x MS x MT x HP x HL	[1]

B = ToTo + T(a) + TTo + TNTTo10.05 + a	[2]

A = 3 4

The DOT formula is of the absolute type, since it estimates the number of accidents, as opposed to providing a "relative" index (often referred to as a hazard rating index).  The formula combines two independent calculations of the number of accidents for a crossing to produce the final absolute accident prediction.  The two independent calculations are obtained from the first two formulas described in the next paragraphs.

K1 x B (for passive devices)

K2 x B (for flashing lights)	[3]

K3 x B (for gates)

3

1.	A "basic" formula provides an initial prediction of the accidents on the basis of the physical and operational characteristics of the crossing as described in the Inventory.  This formula predicts crossing accidents through a calculation similar to that used in other common formulae, such as the Peabody-Dimmick and New Hampshire, and can be considered as a "hazard rating index."

The basic formula is as follows:

a = K x EI x DT x MS x MT x HP x HL	[1]

where

a  = initial accident prediction index, (accidents per year at the crossing),

K  = constant for initialization of factor values at 1.00,

EI = factor for exposure index based on product of highway and train traffic,

DT = factor for number of thru trains per day during daylight,

MS = factor for maximum timetable speed,

MT = factor for number of main tracks,

HP = factor for highway paved (yes or no), and

HL = factor for number of highway lanes.

The basic formula was developed by applying nonlinear multiple regression techniques to crossing characteristics stored in the 1976 Inventory and Accident data files.  Half of the file was used to determine the formula coefficients by regression and iteration, and the other half for testing the formula.  The data sets were disjoint, of equal size, and comprised of a random sample of records from the inventory, including all records for which accident data existed.  Each data set was categorized into two groups of accident and non-accident crossings.  The result can be expressed as a series of factors which, when multiplied together, yield the initial predicted accidents per year at a crossing.

The basic formula consists of a number of multiplicative factors, with each factor representing a characteristic of the crossing described in the DOT Crossing Inventory.  The numerical value of each factor is related to the statistical influence which the specific crossing characteristic has on the predicted number of accidents.  The values of (a) calculated from equation [1] could be considered an accident prediction, but (a) has not been normalized properly.  Three sets of equations are used to determine the values of each factor, corresponding to the following categories of warning devices:  passive warning devices, flashing lights, and flashing lights with automatic gates.  Specific equations for the crossing characteristic factors by the three warning device categories are contained in the publications listed in Section 9.3.  Each set of factor equations should only be used for crossings with the warning device category for which it was designed.  To calculate the value of (a) at a crossing with crossbucks, only the passive set of equations should be used.  The same applies for crossings with flashing lights and crossings with gates.

2.	The predictive capacity of the basic formula is limited because certain important crossing characteristics, such as sight distance at the crossing, are not included in the DOT Crossing Inventory.  Inclusion of actual accident history at a crossing is done in equation [2], which dramatically improves the predictive capabilities of the formula.  Equation [2] calculates a value (B) which is a weighted average of two separately derived predictions.  The value of (B) is determined by combining the value (a) with the crossing's accident history, using equation [2] or a table by extrapolation as contained in other publications referenced in this Section.

The intermediate prediction (B) thus includes the observed accident history (over a five year period) at a crossing.  It assumes that future accidents per year will be the same as the average historical accident rate.  It is referred to as the accident history of the crossing, and is equal to the total observed accidents divided by the number of years over which the observations were made.  (Note:  The formula allows any number of years of accident history data to be used.  However, a five year period is more commonly recognized and used.)

The DOT accident prediction formula is then expressed as

B = ToTo + T(a) + TTo + TNT, 	[2]

where

B3=4

intermediate accident prediction, accidents per year at the crossing,

4

To5=6  = 10.05 + a ,7

formula for weighting factor

5

a8=9

NT10=11

initial accident prediction index (accidents per year) from formula [1], and

6

accident history prediction, accidents per year, where N is the number of observed accidents in T years at the crossing.

7

The DOT formula calculates a weighted average of the predicted accidents at a crossing from the basic formula "a" and accident history "N/T".  The two formula weights, ToTo + T12 and TTo + T13, add to the value of 1.

The intermediate prediction (B) is the value (a) from equation [1], which provides an initial prediction on the basis of a crossing's characteristics (as described in the DOT Crossing Inventory), and the actual accident history at a crossing where NT14 is equal to the number of previous accidents (N) divided by the number of years of data (T).  The value of (T) is usually taken to be five.  The most recent five years of accident history data should be used to insure good performance from the formula.  Accident history information older than five years may be misleading because of changes in crossing characteristics.

3.	To get the final predicted accidents (A), (B) is multiplied by one of three constants as indicated by equation [3].

A = 15 16

K1 x B (for passive devices)

K2 x B (for flashing lights)	[3]

K3 x B (for gates)

8

The particular constants, K1, K2, and K3, depend on whether the crossing has passive devices (e.g., crossbucks), flashing lights, or gates.  These constants adjust the predictions to reflect more recent levels of accident experience.  They are recalculated periodically and published annually in FRA's Highway-Rail Crossing Accident/Incident and Inventory Bulletin.

A flow diagram of the DOT accident and severity prediction formulas, showing the data bases employed, is shown in Figure 9-1.  The abbreviations used for the Accident Prediction lists produced by FRA are contained in Appendix B.

Figure 9-1.  DOT Highway-Rail Crossing Accident and Severity Prediction Formulas

9.3  Resource Allocation Model

The availability of the U.S. DOT-AAR National Highway-Rail Crossing Inventory and Accident data permitted the development of a resource allocation model.  Development of accident prediction formulas was a necessary intermediate step.  The U.S. DOT Highway-Rail Crossing accident prediction formulas were created utilizing nonlinear, multiple regression techniques applied to the crossing characteristics in the National Inventory and the Accident databases compiled by FRA.  The model calculates the expected annual accident rate at a crossing.

It is important to note that the U.S. DOT's accident prediction formulas produce an absolute prediction which is different from a hazard index (e.g., the New Hampshire formula).  The hazard index only produces a relative index for each crossing based on available physical characteristic data and does not include any accident history information.  A hazard index has value only in relatively comparing one crossing with another with very similar characteristics.  The U.S. DOT accident prediction formulas provide an absolute prediction process which can compare all crossings and one that is needed for the resource allocation model.

The U.S. DOT resource allocation model determines which crossings should have motorist warning devices installed so as to achieve the maximum crossing safety benefit for a given level of funding.  The net result is a list of the most cost-effective improvement decisions.  Possible grade crossing improvements include:  (1) passive devices to flashing lights, (2) passive devices to gates, and (3) flashing lights to gates.

Inputs to the resource allocation model include the predicted accident rate of the crossing, costs and effectiveness of the different improvement options, and the budget level available.  Cost data required are the installation costs for each of the possible upgrade options.  Effectiveness is defined as the percentage by which accidents are reduced after installation of a warning device at a crossing.

The resource allocation model provides a ranked list based on benefit/cost ratios.  Benefit is expressed as predicted accidents prevented per year and cost is the life-cycle cost of the equipment.  The algorithm considers the benefit/cost ratios beginning with the largest ratio and continuing in decreasing order.  The process continues until the monies spent (costs of recommended warning devices) equal or exceed the available budget.  Thus, an optimal list of recommended improvements is obtained.

The primary function of the resource allocation procedure is to assist States and railroads in preparing Statewide grade crossing improvement programs.  Because of the magnitude of the Inventory and Accident data bases, use of the model has required a mainframe computer.  Data and computer printout list are available by directing a request to FRA.

Information on the formulas and procedures may be found in the "Rail-Highway Crossing Resource Allocation Procedure, User's Guide, Third Edition," U.S. Department of Transportation, Federal Railroad Administration, August 1987, Transportation Systems Center, Cambridge, MA, 02142, Report Numbers DOT/FRA/OS-87/10 and DOT-TSC-FRA-87-1 (both for the same report).  This document is available to the public through the National Technical Information Service, Springfield, Virginia, 22161.

Further information on the formulas and procedures is contained in "Summary of the DOT Rail-Highway Crossing Resource Allocation Procedure - Revised," U.S. Department of Transportation, Federal Railroad Administration, June 1987, Transportation Systems Center, Cambridge, MA, 02142, Report Numbers DOT/FRA/OS-87/05 and DOT-TSC-FRA-86-2 (both for the same report).  This summary contains the formulas which calculate a severity prediction, extended warning device effectiveness data, and inclusion of the stop sign option in the resource allocation model.  This document also is available to the public through the National Technical Information Service, Springfield, Virginia, 22161.

The theory underlying the formulas is contained in P. Mengert, "Rail-Highway Crossing Hazard Prediction Research Results," U.S. Department of Transportation, Transportation Systems Center, Washington, DC, March 1980, FRA-RRS-80-02, which is available as a reference only.

9.4  Data Provided to States and Railroads

The U.S. DOT accident prediction computer printouts list public highway-rail crossings ranked by predicted accidents per year.  The printouts show the ordered ranking in a State, county, city, railroad or any combination thereof and include the accident history along with other crossing data.  They also list the crossings in ascending order, by crossing number, and provide location information.

The U.S. DOT resource allocation procedure consists of three computer printouts listing public highway-rail crossings.  One printout lists crossings according to the number of accidents predicted annually.  The highest prediction is listed first.  The second printout shows proposed crossing projects in a benefit-cost sequence determined by simultaneous consideration of accident predictions, alternative costs and benefits and budget levels.  The proposed project with the highest benefit-cost ratio is listed first.  A third printout indexes all the crossings considered in this process, by crossing number, accident prediction, and rank.

The Department of Transportation accident prediction formula combines two independent calculations:  (1) a basic formula that predicts accidents based on the Inventory's physical and operating characteristics, and (2) the initial prediction combined with another prediction derived from the reported accident history at the crossing.

The values and data are derived from the Federal Railroad Administration's Inventory and Accident files and are subject to the processing contractor's keypunch errors and input data submission errors from both railroads and States.  Efforts have been made to find and correct errors, but there remains a possibility that some errors still exist.  For this reason, States and/or railroads should verify the data by conducting on-site inspections of those crossings whose prediction ranking indicates a relatively high value.  Erroneous data may significantly alter accident prediction and resource allocation values.  It must also be recognized that this is only one model and that other models may give different results.  As with all models, there are certain characteristics that are not or cannot be included in arriving at a prediction value.  These characteristics include the sight distance at the crossing, highway congestion, and the volume of hazardous materials traffic.

These data are produced by using accident prediction formulas developed to aid in planning highway-rail crossing safety programs.  The resource allocation procedure uses these accident prediction formulas together with cost evaluation data to produce a ranking of those crossings that can achieve maximum improvement benefits given a specific level of funding.  This model is designed to nominate crossings for improvements on a cost-effective basis and

suggests the type of warning device to be installed.  The cost-effective data used for producing the enclosed material appear at the beginning of the printout.

Figure 9-2.  Highway-Rail Crossing Resource Allocation Procedure

When requesting a resource allocation printout, four data elements are required:

(1)	The average cost of warning device upgrade from passive devices (crossbucks) to flashing lights.

(2)	The average cost of warning device upgrade from passive devices (crossbucks) to gates (with flashing lights).

(3)	The average cost of warning device upgrade from flashing lights to gates (with flashing lights).

(4)	The total budget level of available funds, or a higher value for planning purposes.

States and/or railroads desiring to install the formula and models on their own computer should contact FRA.  The current computer programs used by FRA can be provided on a customer supplied reel-to-reel magnetic tape for use on a mainframe computer.  Because of the size of the supporting data bases, use of the model has required a mainframe computer.  However, for smaller data bases, an individual State or railroad may avail themselves of programs developed by non-government sponsored researches that will operate on a personal computer (PC) or microcomputer.

9.5  Performance Compared to Other Models

In a report prepared for the 1986 annual meeting of the Transportation Research Board (TRB), researchers at the University of Virginia revealed that the U.S. DOT accident prediction formula is a better procedure for establishing priorities for grade crossing safety improvement projects than other models tested.  A total of five formulas were evaluated using the State of Virginia Department of Highways and Transportation grade crossing inventory.  In addition to the U.S. DOT formula, the other formulas were:  Peabody-Dimmick; NCHRP No. 50; Coleman-Stewart; and the New Hampshire.  According to the researchers, the DOT formula outperformed the other models in both the evaluative and comparative analyses.

The authors of the Virginia study caution the reader that although the U.S. DOT accident prediction formula outperformed the other four nationally recognized models, the following facts remain:

"The DOT accident prediction formula takes into account the most important variables that are statistically significant in predicting accidents at rail-highway crossings.  However, it must be noted that there is no general consensus as to which of the site characteristics are the most important ones.  As a result, the priority list that is produced by using this formula must serve as only one of the criteria for improving conditions at any crossing.  This information must be supplemented by regular site inspections and other qualitative issues that can not be feasibly incorporated into a mathematical formula."

To determine the availability of this report, contact:

Rail Transportation Division

Virginia Department of Highways

and Transportation

1221 East Broad Street

Richmond, Virginia 23219

To obtain a copy of the TRB paper presented at the 1986 annual meeting, contact:

Department of Civil Engineering

University of Virginia

Thornton Hall

Charlottesville, VA 22901

During development and review of the accident prediction formula, comparisons were made with other highway-rail crossing accident prediction models.  Statistical tests which compared these models indicated that the accuracy of DOT's formula is superior for ranking crossings by predicted accident levels.  Since the DOT formula is based on the DOT Crossing Inventory, a common data base of crossing characteristics is available to formula users.  As the DOT Crossing Inventory is updated, the DOT accident prediction formula will reflect the latest information.

10.0  ADDITIONAL INFORMATION AND REFERENCES

10.1  Highway-Rail Grade Crossing Handbook

The "Railroad-Highway Grade Crossing Handbook - Second Edition," 1986 provides general information on highway-rail crossings, including characteristics of the crossing environment and users, and the physical and operational improvement for safe and efficient use for both highway and rail traffic.  The handbook will be of interest to Federal, State, and local highway agency personnel, railroad officials, consulting engineers and educators involved with highway-rail grade crossing installation, safety and operation.

A standard distribution of the handbook was made to the Federal Highway Administration (FHWA) Region and Division offices and the State highway agencies in 1986.  Copies of the handbook were also provided to the Federal Railroad Administration and the Association of American Railroads.  A limited number of copies are available to States and railroads from the Federal-Aid Program Branch of the Federal Highway Administration.  The handbooks can be ordered from the following addresses:

Federal Highway Administration

Federal-Aid Program Branch

400 7th Street S.W., HNG-12

Washington D.C.  20590

Telephone:  (202) 366-4656

National Technical Information Service

5285 Port Royal Road

Springfield, Virginia, 22161

(703) 487-4700 or (703) 487-4650

For "Rush Handling," call (800) 553-6847

10.2  Worldwide Geographic Location Codes

The "Worldwide Geographic Location Codes," U.S. General Services Administration, Office of Finance, 1987 lists the standard numeric (number) and alpha (letter) codes that some Federal agencies use in designating geographic locations in automatic data processing programs.  These FIPS codes are those used for processing crossing inventory data.  Use of standard codes facilitates the interchange of machine-sensible data from agency to agency within the Federal community and to State and local groups who request or contribute to Federal programs.

Three sections of codes are given in the publication.  Part 1 indexes numeric codes for counties and cities within the 50 States of the United States.  Part 2 catalogs alpha and numeric codes for U.S. possessions and areas administered by the United States.  Part 3 lists similar combinations for foreign countries.

The codes for the States, counties, and cities listed in Part 1 are based on two Federal Information Processing Standards Publications (FIPS 5-1 and FIPS 6-1) issued by the National Bureau of Standards in accordance with the provisions of Public Law 89-306 (commonly known as the Brooks Act) and the Office of Management and Budget, Circular A-86.

Government departments or agencies using the codes within this publication may request the assignment of additional codes for populated areas having recognized boundaries.  Such requests should be submitted in writing to the Public Building Service (PGG), General Services Administration, Washington, D.C., 20405.  Telephone requests may be made by dialing (202) 501-1426.  The criteria for assigning additional codes, except for military installations, are established by the above named office.  Additional codes for military installations are assigned or verified through the Department of Defense.

Copies of this publication are available from the Superintendent of Documents, U.S. Government Printing Office, Washington, D.C., 20402.  The codes are also available on diskette or magnetic tape for $50.00 from the General Services Administration (PGG), Washington, D.C., 20405, telephone: (202) 501-1426.

10.3  Federal-Aid Policy Guide

The Federal-Aid Policy Guide (FAPG) 23 CFR, Part 646 Railroads, Subpart B, Railroad-Highway Projects, contains the regulations which prescribe policies and procedures for advancing Federal-Aid projects involving railroad facilities.

This directive contains the following:

a.	Information which applies to Federal-Aid projects involving railroad facilities, including projects for the elimination of hazards of highway-rail crossings, and other projects which use railroad properties or which involve adjustments required by highway construction to either railroad facilities or facilities that are jointly owned or used by railroad and utility companies.

b.	Additional instructions for projects involving the elimination of hazards of highway-rail grade crossings pursuant to 23 U.S.C. 130 are set forth in 23 CFR Part 924 of the FAPG.

c.	Procedures on reimbursement for projects undertaken pursuant to this directive are set forth in 23 CFR Part 140, Subpart I of the FAPG.

d.	Procedures on insurance required of contractors working on or about railroad right-of-way are set forth in 23 CFR Part 646, Subpart A of the FAPG.

Copies of the regulations are available from the FHWA State Division office located in the capital city of each State or by contacting the Federal Highway Administration at the following address:

Federal Highway Administration

Federal-Aid Program Branch

400 7th Street S.W., HNG-12

Washington D.C.  20590

Telephone:  (202) 366-4656

APPENDIX A

STATE CONTACTS

STATE CONTACTS

as of December, 1996

US DOT/AAR HIGHWAY-RAIL CROSSING INVENTORY PROGRAM

ALABAMA

Jerry L. Peters

Bureau, Multimodal Transportation

Alabama Department of Transportation

1409 Coliseum Boulevard

Montgomery, Alabama 36130-3050

Telephone - (334) 242-6080

ALASKA

Bruce Freitag, P.E.

State Traffic Engineer

Alaska Dept. of Transportation & PF

3132 Channel Drive

Juneau, Alaska 99811

Telephone - (907) 465-2968

ARIZONA

David M. Gibson

Railroad Inventory Crossing Coordinator

Arizona Department of Transportation

1739 W. Jackson St.   064R

Phoenix, Arizona 85250

Telephone - (602) 255-7134

ARKANSAS

Jim Barnett, Attn: Mike Selig

Engineer, Traffic Division

Arkansas Transportation Department

P.O. Box 2261

Little Rock, Arkansas 72203

Telephone - (501) 569-2231

CALIFORNIA

Kevin Elcock

Structures, Spe. Proj. Agreements

California Dept. of Transportation

Sacramento, California 94274-0001

Telephone - (916) 227-8024

COLORADO

Charles Petersen

Intermodal Unit Manager

Colorado Dept. of Highways

4201 East Arkansas Ave., Room 212

Denver, Colorado 80222

Telephone - (303) 757-9822

CONNECTICUT

Robert Seaman

Transportation Planner

Connecticut Dept. of Transportation

P.O. Drawer A

Wethersfield, Connecticut 06109

Telephone - (203) 667-7340

DELAWARE

Joseph F. Walder

Manager, Office of Rail Operations

Delaware Railroad Administration

100 South French Street

Wilmington, DE 19801

Telephone - (302) 577-3278 x3452

Fax - (302) 577-6066

DISTRICT OF COLUMBIA

Abdul Rashid Sleemi

Chief, Traffic Safety & Data Analysis

D.C. Department of Public Work

2000 14th Street, N.W., 7th Floor

Washington, D.C. 20009

Telephone - (202) 939-8089

FLORIDA

Gordon Morgan

Transportation Statistics Office

Florida Department of Transportation

605 Suwannee Street, MS-27

Tallahassee, Florida 32399-0450

Telephone - (904) 488-4111

GEORGIA

Marion G. Waters, III

Office of Traffic Operations

Georgia Dept. of Transportation

935 E. Confederate Ave.,

Bldg 24, Room 307

Atlanta, Georgia 30316

Telephone - (404) 635-8120

HAWAII

William Lake

Federal Highway Administration

Hawaii Motor Carrier Safety Div.

300 Ala Moana Blvd., Room 4119

Honolulu, Hawaii 96850

Telephone - (808) 541-2700

IDAHO

Lee Wilson

Transportation Staff Engineer

Idaho Dept. of Transportation

P.O. Box 7129

Boise, Idaho 83707-1129

Telephone - (208) 334-8561

ILLINOIS

Don Lingle/Deanna Smith

Chief, Data Management

Illinois Dept. of Transportation

2300 South Dirksen Parkway

Springfield, Illinois 62764

Telephone - (217) 785-2353

INDIANA

Mr. Mike McGathey

Railroad Division Code Enforcement

Indiana Department of Transportation

100 N. Senate Ave., Suite N-901

Indianapolis, Indiana 46204-2208

Telephone - (317) 232-4786

IOWA

Richard N. Torman

Transportation Data

Iowa Dept. of Transportation

800 Lincoln Way

Ames, Iowa 50010

Telephone - (515) 239-1504

KANSAS

Ron Balsters

Bureau of Transportation Planning

Kansas Dept. of Transportation

State Office Building

Topeka, Kansas 66612

Telephone - (913) 296-3841

KENTUCKY

Bruce S. Siria

Director, Division of Planning

Kentucky Transportation Cabinet

419 Ann Street

Frankfort, Kentucky 40622

Telephone - (502) 564-7183

LOUISIANA

William Shrewsberry

Road Maintenance Engineer

Louisiana Dept. of Transportation

Capitol Station, Room 504C

P.O. Box 94245

Baton Rouge, Louisiana 70804-9245

Telephone - (504) 379-1543

MAINE

Allan H. Bartlett

Rail Transportation Division

Maine Department of Transportation Transportation Building

Augusta, Maine 04333

Telephone - (207) 287-2841

MARYLAND

Robert J. Herstein

Chief, Traffic Programs Section

Maryland Department of Transportation

7491 Connelley Drive

Hanover, Maryland 21076

Telephone - (410) 787-5867

Fax: (410) 553-6399

MASSACHUSETTS

Jeremiah Murphy

Highway Safety Engineer

Massachusetts Highway Department

10 Park Plaza, Room 7143

Boston, Massachusetts 02116-3973

Telephone - (617) 973-7346

MICHIGAN

Sandy Gregory

UPTRAN - Freight Services & Safety Division

Michigan Dept. of Transportation

P.O. Box 30050

Lansing, Michigan 48909

Telephone - (517) 373-0874

MINNESOTA

Tom Gellerman

Senior Highway Technician

Minnesota Office of Railroads and Waterways

Suite 925, Kelly Annex, Trans. Building

St. Paul, Minnesota 55155

Telephone - (612) 296-1677

Fax: (612) 297-1887

MISSISSIPPI

Lawrence D. Lockhart

Crossing Safety Engineer

Mississippi State Highway Dept.

P.O. Box 1850

Jackson, Mississippi 39205

Telephone - (601) 359-7685

Fax: (601) 359-7110

MISSOURI

Leroy Meisel

Department of Economic Development

Missouri Division of Transportation

P. O. Box 1216

Jefferson City, Missouri 65102

Telephone - (314) 751-7123

MONTANA

John W. Lewis

Transportation Planning Division

Montana Department of Transportation

P.O. Box 201001

Helena, Montana 59620-1001

Telephone - (406) 444-4377

NEBRASKA

Ken Siekmeyer

Transportation Planning Manager

Nebraska Department of Roads

P.O. Box 94759,

Lincoln, Nebraska 68509

Telephone - (402) 479-4519

NEVADA

Robert Hilderbrand

Manager, Safety Engineering

Nevada Dept. of Transportation

1263 South Stewart Street

Carson City, Nevada 89712

Telephone - (702) 687-3468

NEW HAMPSHIRE

Kelton E. Garfield

Bureau of Transportation Planning

New Hampshire Dept. of Transportation

1 Hazen Drive

Concord, New Hampshire 03302-0483

Telephone - (603) 271-3708

Fax: (603) 271-3914

NEW JERSEY

Donna Troiano, P.E.

Bureau of Utility & Railroad Engr.

New Jersey Dept. of Transportation

1035 Parkway Avenue

Trenton, New Jersey 08625

Telephone - (609) 530-5686

NEW MEXICO

Ernie Vigil

Data Collection Bureau

New Mexico Dept. of Transportation

P.O. Box 1149

Santa Fe, New Mexico 87504

Telephone - (505) 827-5561

NEW YORK

John Bell

Grade Crossing Program Mang. Section

New York Department of Transportation

Building 7A, Room 304

1220 Washington Avenue, State Campus

Albany, New York 12232

Telephone - (518) 457-1046

NORTH CAROLINA

James M. Lynch

Division of Highways, Traffic Engr.

North Carolina Dept. of Transportation

P.O. Box 25201

Raleigh, North Carolina 27611-5201

Telephone - (919) 733-3161

NORTH DAKOTA

Don Laschkewitsch

Rail and Signal Planning

North Dakota Dept. of Transportation

State Highway Bldg., Capitol Grounds

608 E. Boulvard Avenue

Bismarck, North Dakota 58505-0700

Telephone - (701) 328-4409

Fax: (701) 328-4545

OHIO

Fran A. Netting

Transportation Department

Ohio Public Utilities Commission

180 East Broad Street

Columbus, Ohio 43216

Telephone - (614) 466-0407

OKLAHOMA

Max Carter

Rail Branch Manager

Oklahoma Dept. of Transportation

200 N. E. 21st Street

Oklahoma City, Oklahoma 73105

Telephone - (405) 521-2771

OREGON

Matthew Caswell, P.E.

Railroad Utility Engineer

Oregon Dept. of Transportation

State Highway Building, Room 408

Salem, Oregon 97310

Telephone - (503) 986-3658

PENNSYLVANIA

Harold Nanovic

Statewide Transportation Planner

Pennsylvania Dept. of Transportation

Transportation & Safety Bldg., Room 917

Harrisburg, Pennsylvania  17120

Telephone - (717) 787-1251

PUERTO-RICO

Juan O. Cruz

Federal Highway Administration

Puerto Rico Motor Carrier Safety Div.

U.S. Courthouse Clemte Ruiz Nazario

Carlos Chardon Street

San Juan, Puerto Rico 00918

RHODE ISLAND

J. Michael Bennett

Attn: John Vorro

Rhode Island Dept. of Transportation

State Office Building, Room 231D

Providence, Rhode Island 02930

Telephone - (401) 277-2023

SOUTH CAROLINA

Richard B. Werts

Director of Traffic Engineering

South Carolina Dept. of Transportation

P.O. Box 191

Columbia, South Carolina 29202

Telephone - (803) 737-1455

Fax: (803) 737-0271

SOUTH DAKOTA

Larry Schoenhard

Data Inventory - Div. of Planning

South Dakota Dept. of Transportation

700 South Broadway Avenue, E.

Pierre, South Dakota 57501-2586

Telephone - (605) 773-3278

TENNESSEE

Terry Cantrell

Railroad Inventory

Tennessee Dept. of Transportation

James K. Polk Building, Suite 900

Nashville, Tennessee 37243-0334

Telephone - (615) 741-0971

TEXAS

Alvin R. Luedecke

Director, Transportation Planning

Texas Department of Transportation

P.O. Box 5051

Austin, Texas 78763-5051

Telephone - (512) 465-7346

UTAH

Lillian Witkowski

Division of Railroad Safety

Utah Department of Transportation

4501 South 2700 West

Salt Lake City, Utah 84119

Telephone - (801) 965-4286

VERMONT

Tony Reddington

Transportation Planner

Vermont Agency of Transportation

133 State Street

Montpelier, Vermont 05633

Telephone - (802) 828-2710

VIRGINIA

Earl Stitzer

Transportation Engineer

Virginia Department of Transportation

1401 East Broad Street

Richmond, Virginia 23219

Telephone (804) 786-8037

WASHINGTON

Calvin Smith

Travel Data Office, Br. Mgr.

Washington Dept. of Transportation

PO Box 47380

Olympia, Washington 98504-7380

Telephone - (206) 753-5386

WEST VIRGINIA

David E. Pennington

Statewide Planning Division

West Virginia Dept. of Highways

1900 Washington Street, East

Charleston, West Virginia 25305

Telephone - (304) 348-3134

WISCONSIN

Rex A. Hinrichs

Bureau of Program Management

Wisconsin Department of Transportation

PO Box 7916

Madison, Wisconsin 53707-7916

Telephone - (608) 267-9617

WYOMING

Marvin Cobb

Planning/Inventory Supervisor

Wyoming Transportation Department

P.O. Box 1708

Cheyenne, Wyoming 82003-1708

Telephone - (307) 777-4186

APPENDIX B

COMPUTER PRINTOUT FORMAT SAMPLES

SUMMARY

Accident Prediction Programs Output

Resource Allocation Programs Output

Output from Programs Counting the Number of Crossings

Listings of Inventory Type Data

Inventory Report (One-page-per-crossing)

Accident/Incident Report

Computer Generated Fill-in-the-Blanks Lists for Mass Updates

Special Information Reports

ACCIDENT PREDICTION PROGRAMS OUTPUT

This sample printout can be produced for:

(1)  Predicted Accidents,

(2)  Predicted Fatal Accidents, or

(3)  Predicted Casualty Index.

The Location Information printout is automatically generated with any of the above three conditions

9

PUBLIC HIGHWAY-RAIL CROSSINGS

RANKED BY PREDICTED ACCIDENTS PER YEAR

RANK   PRED      XING    ST  RR  -->  NUM  OF   ACC  <--  DATE   WD  TOTL  DAY   TOTL  TOTL  MAIN  TTBL  HWY  HWY  URBN   AADT

ACCDS     ID              ___  ___  ___  ___  ___  OF     CL  SWIT  THRU  THRU  TRKS  TRKS  SPD   PVD  LNS  RURL

#               88   89   90   91   92   CHG        TRNS  TRNS  TRNS

937   0.072752  064950W  WY  BN   1    0    0    0    0          XB    0    23    45    1     1     49   NO    2    R       43

938   0.072667  597265C  TX  BN   0    0    0    0    0   90-02  FL    0     6    10    1     1     30   YES   2    U     8330

939   0.072643  079831G  WI  BN   0    0    0    1    0          XB    4     0     0    2     0     10   YES   2    U     2120

940   0.072631  665980Y  AR  BN   0    0    1    0    0          GT    4    16    32    2     2     55   YES   2    U      570

941   0.072613  072820V  IL  BN   0    0    1    0    0          XB    0     7    12    1     1     79   NO    1    R      150

942   0.072589  064567G  NE  BN   0    1    0    0    0          XB    0    20    40    2     2     50   NO    2    R       50

943   0.072574  096510U  WA  BN   0    0    0    0    1          XB   10     3     4    1     1     40   YES   2    R       85

944   0.072538  064307N  NE  BN   0    0    1    0    0          FL    0     2     2    1     1     20   YES   2    R     5080

945   0.072536  083690G  NE  BN   0    1    0    0    0          GT    0    11    21    2     1     45   YES   2    R     2025

946   0.072464  597273U  TX  BN   1    0    0    0    0          FL    0     6    10    2     1     50   YES   2    R      760

947   0.072426  079768S  IL  BN   0    1    0    0    0          XB    0    10    20    2     2     79   NO    2    R       75

948   0.072218  091683S  WA  BN   1    0    0    0    0          XB    2     0     0    1     0     10   YES   4    U     4100

949   0.072213  245124W  CO  BN   0    0    1    0    0          XB    0     2     2    1     1     20   YES   2    R     1050

950   0.072205  663802F  OK  BN   0    0    0    0    0          FL   30    11    18    2     2     25   YES   2    U     4200

951   0.072145  070167C  MO  BN   0    1    0    0    0          FL    0    28    43    1     1     60   YES   2    R      114

952   0.072097  083311E  NE  BN   0    1    0    0    0          XB    0    11    21    1     1     45   NO    2    R      135

953   0.072076  092549J  WA  BN   0    0    1    0    0          XB    6     2     4    2     1     40   YES   2    U      137

954   0.072058  663417C  TN  BN   0    0    0    0    0          FL   50    11    22    1     1     20   YES   2    U     4420

955   0.071890  672031D  OK  BN   0    0    1    0    0          XB    0     6    12    1     1     55   YES   2    R       50

956   0.071807  093612C  ND  BN   1    0    0    0    0          XB    4    28    48    1     1     60   NO    2    R       25

957   0.071779  671991A  OK  BN   0    0    0    0    0          FL    4     4    10    1     1     20   YES   4    R    12000

958   0.071738  061524D  MN  BN   1    0    0    0    0          FL    0     5    10    1     1     50   YES   2    R      760

959   0.071704  071938G  ND  BN   0    1    0    0    0          XB    5    30    51    2     2     79   NO    1    R       15

960   0.071561  057224H  CO  BN   0    0    0    1    0          XB    0    10    28    1     1     79   NO    2    R       50

961   0.071504  665275V  AL  BN   0    1    0    0    0          XB    0     1     2    1     1     35   YES   2    U      972

962   0.071475  672020R  OK  BN   0    0    1    0    0          XB    4     4    10    1     1     55   YES   2    R       50

963   0.071463  668605J  KS  BN   0    0    1    0    0          XB    0    10    20    2     2     50   NO    2    R      127

964   0.071451  083389Y  NE  BN   0    0    0    0    1          XB    0    19    38    2     2     60   NO    2    R       40

965   0.071420  664861L  MS  BN   0    0    0    0    1          XB    0     3     6    1     1     49   YES   2    R      150

966   0.071381  668656U  KS  BN   1    0    0    0    0          XB    0    12    20    1     1     60   NO    1    R       94

967   0.071331  070839F  ND  BN   0    0    1    0    0          GT    6     6    10    1     1     20   YES   2    U     3400

968   0.071272  672930M  TX  BN   0    0    0    0    0          FL   15     0     0    1     1     40   YES   4    U    25000

969   0.071225  083173T  NE  BN   0    1    0    0    0          GT    4     7    11    3     1     30   YES   2    R     3280

970   0.071224  597191M  TX  BN   0    0    1    0    0          XB   20     6    10    3     1     30   NO    1    R      160

971   0.071223  096356Y  WA  BN   0    1    0    0    0          XB    4     1     2    2     1     25   YES   2    R      390

972   0.071108  101821J  WA  BN   0    0    0    0    1          FL    6     0     0    1     0      5   YES   2    U     4750

973   0.071082  274633W  TX  BN   0    0    0    0    1          XB    0    10    20    1     1     60   NO    2    R      100

974   0.071079  396916G  WA  BN   0    1    0    0    0          XB    0     1     2    3     1     10   YES   2    R     1580

975   0.071063  664101V  MO  BN   0    0    0    0    0          FL    4    17    29    1     1     35   YES   2    U     7990

976   0.071035  084734H  WA  BN   0    0    0    1    0          GT    4     5    10    1     1     50   YES   2    R     4204

977   0.070987  667300H  MS  BN   0    0    0    1    0          FL    0    10    20    1     1     55   YES   2    U      300

978   0.070978  072891S  IL  BN   1    0    0    0    0          FL    0     7    12    1     1     79   YES   2    U      550

PUBLIC HIGHWAY-RAIL CROSSINGS  RESOURCE ALLOCATION RESULTS

BASED ON PREDICTED ACCIDENTS PER YEAR (SORTED BY CRSSING IDs)

OBS    CROSSING    BEN/COST    STATE    COUNTY       CITY            RAILROAD    ROAD                 RRID    MILEPOST

ID          RATIO

1     058712P       7.6811     ID      BONNER       SANDPOINT          BN       SAMUELS RD.          0036     139107

2     063104F       8.6758     MO      BUCHANAN     ST JOSEPH          BN       ILLINOIS&CHEEROKE    0016     005895

3     085584F      15.9419     WA      KING         SEATTLE            BN       LANDER ST            0051     000128

4     244642N      14.2102     CO      LARIMER      FORT COLLINS       BN       CHERRY ST-E OF MA    476      007452

5     664912U       6.5847     MS      LOWNDES      COLUMBUS           BN       CO RD                         R64350

6     668548X      10.5541     OK      ROGERS       CLAREMORE          BN       WILL ROGERS BLVD              039753

7     670406G       8.1509     OK      OTTAWA       MIAMI              BN       STEVE OWENS BLVD              017270

8     672151U       8.1372     TX      DALLAS       CARROLLTON         BN       IH 35 FRONTAGE RD             070057

9     672543V       6.6038     AR      CRAIGHEAD    JONESBORO          BN                                     042433

RESOURCE ALLOCATION PROGRAMS OUTPUT

DEMO1                             12:07 MONDAY, JANUARY 10, 1994    1

HIGHWAY-RAIL CROSSING RESOURCE ALLOCATION RESULTS

BASED ON PREDICTED ACCIDENTS PER YEAR

OBS   XING    PREDICTED  BEN/COST  RECOMMD  PRESENT  TOTAL   TOTAL    CUMULATIVE  CUMULATIVE    DC1    DC2  DC3    DC4  STOP

ID      ACCIDENTS  RATIO     WARNING  WARNING  TRACKS  TRAINS   COST        REDUCED                               SIGN

#       PER YEAR             DEVICE   DEVICE           PER DAY              ACCIDENTS                             REQMNT

1   085584F  1.356217   15.9419    GATE     FLASH     4       44        58700     0.935790       .      .   .   0.413    NO

2   244642N  0.889150   14.2102    FLASH    PASS      1       10       102500     1.558195   0.463  1.224   .       .    NO

3   668548X  0.897861   10.5541    GATE     FLASH     1       22       161200     2.177719       .      .   .   0.623    NO

4   063104F  0.738071    8.6758    GATE     FLASH     5       77       219900     2.686989       .      .   .   0.758    NO

5   670406G  0.693420    8.1509    GATE     FLASH     1        8       278600     3.165449       .      .   .   0.807    NO

6   672151U  0.692253    8.1372    GATE     FLASH     1        8       337300     3.643104       .      .   .   0.809    NO

7   058712P  0.480617    7.6811    FLASH    PASS      1       31       381100     3.979536   0.857  2.265   .       .    NO

8   672543V  0.561801    6.6038    GATE     FLASH     1       26       439800     4.367179       .      .   .   0.997    NO

9   664912U  0.412015    6.5847    FLASH    PASS      1        6       483600     4.655590   1.000  2.643   .       .    NO

OUTPUT FROM PROGRAMS

COUNTING THE NUMBER OF CROSSINGS

FOR VARIOUS PURPOSES

NOTE:	These reports may be produced by State, railroad, or county and sorted accordingly

10

COUNT OF ALL RAIL-HIGHWAY CROSSINGS                                           PAGE    1

BY TYPE, POSITION AND RAILROAD                                              01/07/94

FOR THE STATE OF MINNESOTA

PUBLIC      PUBLIC  GRADE

AT        SEPARATIONS

RAILROAD                                        GRADE   RR UNDER    RR OVER   PRIVATE  PEDESTRIAN      TOTAL

------------------------------------------------------------------------------------------------------------------------------------

Buffalo Ridge Railroad Company                      1          0          0          0          0          1

Burlington Northern Railroad Company             1996        171         99       1021         25       3312

Cedar River Railroad Company                       27          0          1         26          0         54

CANADIAN NATIONAL - NORTH AMERICA                  50          0          0         17          0         67

Chicago & North Western Transportation C          733         84         43        503         11       1374

Dakota Railroad                                    66          2          3         54          3        128

Dakota, Minnesota & Eastern Railroad              368         11         23        256          1        659

Duluth, Missabe & Iron Range Railway Com          151         24         29        136          2        342

Duluth & Northeastern Railroad Company              8          0          0          1          0          9

Duluth, Winnipeg & Pacific Railway                 80          8          3         21          0        112

LAC QUI PARLE REGIONAL RR AUTHORITY                 2          0          1          5          0          8

Minnesota, Dakota & Western Railway Comp           14          0          0          0          0         14

Minnesota Commercial Railway                      100          0          2          5          0        107

Minnesota Transportation Museum                     8          1          1          5          0         15

MNVA Railroad, Incorporated                       103          2          0        101          1        207

Otter Tail Valley Railroad Company, Inco           95          5          5         74          1        180

RESERVE MINING COMPANY                              7          0          0          8          0         15

Red River Valley & Western Railroad                 4          0          0          3          0          7

SAINT LOUIS AND LAKE COUNTIES REGIONAL R           36          1          3          9          6         55

Soo Line Railroad Company                        1364        150        122        863         24       2523

Twin City & Western Railroad                      146          4         10        108          0        268

Wisconsin Central Ltd. (also Railway)              15          3          3         27          0         48

BUFFALO RIDGE REGIONAL RAILROAD AUTHORIT           54          3          1          6          0         64

LTV STEEL CORP.                                     9          0          4          7          0         20

STATE TOTAL                                      5437        469        353       3256         74       9589

------------------------------------------------------------------------------------------------------------------------------------

* THIS DATA WAS PRODUCED BY THE FEDERAL RAILROAD ADMINISTRATION

* DATA IS CURRENT WITH INVENTORY UPDATE INFORMATION SUPPLIED BY DECENBER 31, 1993

NUMBER OF PUBLIC-AT-GRADE CROSSING RECORDS                                       PAGE    1

BY YEAR OF EFFECTIVE DATE ON RECORD                                            01/12/94

FOR THE STATE OF MINNESOTA

RAILROAD                                          ORIG  PRE-76   76-77   78-79   80-81   82-83   84-85   86-87   88-89   TOTAL

------------------------------------------------------------------------------------------------------------------------------------

Buffalo Ridge Railroad Company                 0       0       0       0       0       0       0       1       0       1

Burlington Northern Railroad Company           2       1       1       3       3       0       6    1765      94    1875

Cedar River Railroad Company                   0       0       1       0       0       0       0      26       0      27

CANADIAN NATIONAL - NORTH AMERICA              0       0       0       0       2       0       0      26       3      31

Chicago & North Western Transportation C      66       9      22      11       2       1       3     539       3     656

Dakota Railroad                                3       0       0       0       0       0       0      63       0      66

Dakota, Minnesota & Eastern Railroad           8       2       1       1       0       0       0     351       2     365

Duluth, Missabe & Iron Range Railway Com      12       2       1       3       1       0       1     119       2     141

Duluth & Northeastern Railroad Company         0       0       0       0       0       0       0       7       0       7

Duluth, Winnipeg & Pacific Railway             9       0       2       0       1       1       0      64       1      78

Minnesota, Dakota & Western Railway Comp       2       0       1       0       0       0       0      11       0      14

Minnesota Commercial Railway                   0       1       1       0       4       0       0      70       0      76

Minnesota Transportation Museum                0       0       0       0       0       0       0       4       0       4

MNVA Railroad, Incorporated                    2       0       1       0       0       0       0      99       0     102

Otter Tail Valley Railroad Company, Inco       2       1       2       0       0       1       1      85       0      92

RESERVE MINING COMPANY                         0       0       1       1       0       0       0       5       0       7

SAINT LOUIS AND LAKE COUNTIES REGIONAL R       6       0       6       9       2       0       0      13       0      36

Soo Line Railroad Company                     23       2       7      12      17       0       3    1246       9    1319

Twin City & Western Railroad                   1       0       0       0       0       0       0       5       0       6

Wisconsin Central Ltd. (also Railway)          0       0       0       0       0       0       0      11       0      11

BUFFALO RIDGE REGIONAL RAILROAD AUTHORIT       0       0       0       1       0       0       0      53       0      54

LTV STEEL CORP.                                0       0       0       0       1       0       0       8       0       9

TOTAL     136      18      47      41      33       3      14    4571     114    4977

STATE TOTAL

136      18      47      41      33       3      14    4571     114    4977

ITEMS RETRIEVED     4977

JULY 31, 1993

STATE                                                          PAGE    1

PUBLIC-AT-GRADE RAIL-HIGHWAY CROSSINGS                                          01/07/94

BY RAILROAD AND TYPE OF WARNING DEVICE

FOR THE STATE OF LOUISIANA

FLASHING  HWY SIG             CROSS     STOP    OTHER              RR

RAILROAD                      GATES   LIGHTS  WW-BELL  SPECIAL    BUCKS     SIGN     SIGN     NONE    TOTAL

------------------------------------------------------------------------------------------------------------------------------------

Ashley, Drew & Northern Railway Company           0        0        0        0        1        0        0        0        1

Acadiana Railway Company                          0       18        0        0       88        0        0        4      110

Arkansas Louisiana & Mississippi Railroad         0        8        0        0       51        0        0        0       59

Atchison, Topeka & Santa Fe Railway Comp          2        1        0        0       23        0        0        0       26

Central Louisiana & Gulf Railroad Company         0        6        0        0       25        0        0        0       31

CSX Transportation                                7        5        0        8        4        1        0        5       30

Delta Southern Railroad Company                   0        5        0        0       42        0        0        0       47

Gloster Southern Railroad                         0        2        0        0       10        0        0        1       13

GULF STATES POWER                                 0        0        0        0        0        0        0        1        1

Illinois Central Railroad Company                40      101        0       16      302        6        3       53      521

Kansas City Southern Railway Company             44       35        0        0      179        0        3       15      276

Louisiana & Arkansas Railway Company             40      113        0        0      309        0        0       23      485

LAKE CHARLSE HARBOR DISTRICT                      0        3        1        0       26        0        0        1       31

Louisiana & Delta Railroad                       22       11        1        0      151        2        0       28      215

Louisiana & North West Railroad Company           1        6        0        0       36        0        0        0       43

TOTAL FOR STATE                                 368      751       17       48     2401       24       16      339     3964

ITEMS RETRIEVED     3964

------------------------------------------------------------------------------------------------------------------------------------

* THIS DATA WAS PRODUCED BY THE FEDERAL RAILROAD ADMINISTRATION

* DATA IS CURRENT WITH INVENTORY UPDATE INFORMATION SUPPLIED BY JULY 31, 1993

DULUTH MISSABE AND IRON RANGE RAILROAD                                         PAGE    1

PUBLIC-AT-GRADE RAIL-HIGHWAY CROSSINGS                                          01/07/94

BY STATE AND TYPE OF CROSSING SURFACE

FOR THE Duluth, Missabe & Iron Range Railway Com

SECTION    WOOD             CONCRETE              METAL   OTHER                   STATE

STATE                           TIMBER   PLANK ASPHALT  SLAB  PAVEMENT  RUBBER SECTION   METAL  GRAVEL   OTHER   TOTAL

------------------------------------------------------------------------------------------------------------------------------------

MINNESOTA                                   8     105      29       0       0       4       0       0       2       3     151

WISCONSIN                                   0       4       2       0       0       0       0       0       0       0       6

TOTAL FOR ROCK ISLAND                       8     109      31       0       0       4       0       0       2       3     157

ITEMS RETRIEVED      157

------------------------------------------------------------------------------------------------------------------------------------

* THIS DATA WAS PRODUCED BY THE FEDERAL RAILROAD ADMINISTRATION

* DATA IS CURRENT WITH INVENTORY UPDATE INFORMATION SUPPLIED BY MARCH 31, 1993

LISTINGS OF INVENTORY TYPE DATA

NOTE:	These reports may be produced for a State or a railroad, sorted accordingly, as required, with specific selection criteria such as "all crossings with no warning devices," "all crossings with gates," "all private crossings," "all crossings with no pavement markings," etc.

11

STATE OF PENNSYLVANIA                                                  PAGE   52

LIST FOR LOCATING CROSSINGS                                                01/11/94

ON THE ATLANTIC       DIVISION, LEHIGH         SUBDIVISION, IN THE STATE OF PENNSYLVANIA

CROSSING  TYPE   ST CNTY   RR   COUNTY          LOC. CITY            STREET          DIVISION      SUBDIVISION  BRANCH      MILEPOST

------------------------------------------------------------------------------------------------------------------------------------

264258W    31    PA  095  CR    NORTHAMPTON     NEAR NAZARETH        PENN ALLEN      ATLANTIC      LEHIGH       NAZARETH IND  109.53

264241T    31    PA  095  CR    NORTHAMPTON     IN   PEN ARGYL       "E" ST          ATLANTIC      LEHIGH       PENN ARGYL L   97.12

264243G    31    PA  095  CR    NORTHAMPTON     NEAR PEN ARGYL       TWP RD          ATLANTIC      LEHIGH       PENN ARGYL L   97.37

264244N    31    PA  095  CR    NORTHAMPTON     NEAR PEN ARGYL       HWY             ATLANTIC      LEHIGH       PENN ARGYL L   97.48

264249X    31    PA  095  CR    NORTHAMPTON     IN   WINDGAP         BROADWAY AVE    ATLANTIC      LEHIGH       PENN ARGYL L   98.86

361568G    31    PA  079  PNER  LUZERNE         IN   PLAINS          MCCULLOUGH ST.  ATLANTIC      LEHIGH       PLAINS SEC    179.86

361567A    31    PA  079  PNER  LUZERNE         IN   PLAINS          COURTRIGHT ST.  ATLANTIC      LEHIGH       PLAINS SEC    180.33

361565L    31    PA  079  PNER  LUZERNE         NEAR PLAINS                          ATLANTIC      LEHIGH       PLAINS SEC    181.05

361563X    31    PA  079  PNER  LUZERNE         NEAR PLAINS          TENANT ST.      ATLANTIC      LEHIGH       PLAINS SEC    181.68

361562R    31    PA  079  PNER  LUZERNE         NEAR PLAINS          MARKET ST       ATLANTIC      LEHIGH       PLAINS SEC    182.14

361558B    31    PA  079  PNER  LUZERNE         IN   PITTSTON        BRANDENBERG LAN ATLANTIC      LEHIGH       PLAINS SEC    184.90

264131H    31    PA  069  CR    LACKAWANNA      IN   DUNMORE         SCRANTON RD     ATLANTIC      LEHIGH       SCRANTON BR     0.85

264132P    31    PA  069  CR    LACKAWANNA      IN   DUNMORE         PARK ST         ATLANTIC      LEHIGH       SCRANTON BR     1.05

264133W    31    PA  069  CR    LACKAWANNA      IN   DUNMORE         GRAVITY ST      ATLANTIC      LEHIGH       SCRANTON BR     1.10

INDEX FOR LOUISIANA PUBLIC AT GRADE CROSSINGS                                      PAGE   77

01/12/94

CROSSING  COUNTY                                N

CITY                      STREET              HIGHWAY   RAILROAD       MILEPOST

NAME                TYPE      BRANCH

------------------------------------------------------------------------------------------------------------------------------------

335370F   EAST BATON ROUG    BATON ROUGE        0      SORRELL AVE         17        NBRD-4          000036

335371M   EAST BATON ROUG    BATON ROUGE        0      BEAUMONT AVE        19        NBRD-4          000008

335372U   EAST BATON ROUG    BATON ROUGE        0      BEAUMONT AVE        19        NBRD-5          000015

335373B   EAST BATON ROUG    BATON ROUGE        0      SORRELL AVE         17        NBRD-4          000033

335402J   EAST BATON ROUG    BAKER              1      PORT HUDSON RD.     08        D-LINE          020729

335403R   EAST BATON ROUG    BAKER              1      FLANCHER RD.        09        D-LINE          020900

335417Y   ORLEANS            NEW ORLEANS        0      S. JOHNSON ST.      19        MAIN            087036

335426X   ORLEANS            NEW ORLEANS        0      S. ROCHEBLAYE S     19        MAIN            087009

335427E   EAST BATON ROUG    BATON ROUGE        0      SHADE AVE           19        MAIN            078500

335429T   EAST BATON ROUG    BATON ROUGE        0      GULF STATES RD      19        MAIN            078581

335433H   EAST BATON ROUG    BATON ROUGE        0      CHOCTAW DR.         16        MAIN            078739

335437K   EAST BATON ROUG    BATON ROUGE        0      GAYOSO ST           19        MAIN            078846

335438S   EAST BATON ROUG    BATON ROUGE        0      NORTH ST            16        MAIN            078848

NUMERICAL ORDERED LIST OF CROSSING                                            01/11/94

CROSSING  EFF-DATETYPE   ST CNTY   RR                COUNTY

LOC  CITY               STREET          DIVISIOSUBDIV         BRANCH       MILEPOST

------------------------------------------------------------------------------------------------------------------------------------

090430F    930103  31    ID  079  UP                 SHOSHONE

IN   MULLAN             EIGHTH ST.      COLUMBISPOKANE        WALLACE BR.      6.84

A RIVER

090431M    930103  31    ID  079  UP                 SHOSHONE

NEAR MULLAN                             COLUMBISPOKANE        WALLACE BR.      6.97

A RIVER

091166D    900806  31    ID  017  MRL                BONNER

NEAR CLARK FORK         FISH HATCHERY   SYSTEM 4TH            MAIN LINE       86.45

091169Y    900807  31    ID  017  MRL                BONNER

IN   CLARK FORK         STEPHENS ST     SYSTEM 4TH            MAIN LINE       93.82

091170T    900807  31    ID  017  MRL                BONNER

IN   CLARK FORK         MAIN STREET     SYSTEM 4TH            MAIN LINE       93.90

091180Y    920101  31    ID  017  MRL                BONNER

NEAR EAST HOPE                          SYSTEM 4TH            MAIN LINE      102.65

091183U    900806  31    ID  017  MRL                BONNER

NEAR HOPE               IDAHO CO. RESOR SYSTEM 4TH            MAIN LINE      106.40

091184B    900806  31    ID  017  MRL                BONNER

NEAR HOPE               KAMLOOPS RV PAR SYSTEM 4TH            MAIN LINE      106.53

091187W    900807  31    ID  017  MRL                BONNER

NEAR HOPE               SUNNYSIDE CUTOF SYSTEM 4TH            MAIN LINE      111.71

095865S    930101  31    ID  055  BN                 KOOTENAI

IN   POST FALLS         4TH AVE.        SPOKANECOEUR D'ALENE  COUR D ALENE     3.90

INVENTORY AND ACCIDENT/INCIDENT REPORT

1.  Inventory Report (One-page-per-crossing)

2.  Accident Report

12

EXAMPLES OF SEVERAL STANDARD

COMPUTER GENERATED FILL-IN-THE-BLANKS LISTS

FOR MASS UPDATES

Examples Shown:

1.	Crossing Closure.

2.	Ownership Change.

3.	Railroad Identification Information.

4.	Identification of Railroads Operating on a Separate Track or on the Same Track.

5.	Railroad Ownership, Milepost, Warning Devices, Train Counts & Speeds.

6.	Train Counts and Speeds.

7.	State Highway Information (Railroad, Advance Warning Sign, Pavement Markings, Warning Device, AADT, Highway Name and Number, Road Name).  Note, list was produced for condition where data indicates "No Advanced Warning Signs are Present."

NOTE:	Milepost shown as "******" results when there is an alpha character in the field and will not print.  Railroads should remove all such characters and use numbers only in the Milepost field.

13

DULUTH MISSABE AND IRON RANGE RAILWAY COMPANY                                      PAGE    1

FILL-IN-THE-BLANKS LIST FOR INDICATING CLOSED CROSSINGS                                  12/22/93

ON THE                DIVISION IN THE STATE OF MINNESOTA

DATE

ID. NO.    CLOSED     ST CNTY   RR          COUNTY     LOCATED        CITY           DIVISION    SUBDIVISION      BRANCH    MILEPOST

------------------------------------------------------------------------------------------------------------------------------------

252367R   _________   27  075  DMIR  LAKE                NEAR TWO HARBORS                       IRON RANGE    WALES             0.59

252355W   _________   27  075  DMIR  LAKE                NEAR TWO HARBORS                       IRON RANGE    WALES            10.93

252356D   _________   27  075  DMIR  LAKE                NEAR TWO HARBORS                       IRON RANGE    WALES            11.78

252357K   _________   27  075  DMIR  LAKE                NEAR TWO HARBORS                       IRON RANGE    WALES            20.30

252358S   _________   27  075  DMIR  LAKE                NEAR TWO HARBORS                       IRON RANGE    WALES            30.29

252359Y   _________   27  075  DMIR  LAKE                NEAR TWO HARBORS                       IRON RANGE    WALES            32.50

252360T   _________   27  075  DMIR  LAKE                NEAR TWO HARBORS                       IRON RANGE    WALES            33.65

252361A   _________   27  075  DMIR  LAKE                NEAR TWO HARBORS                       IRON RANGE    WALES            34.09

252362G   _________   27  075  DMIR  LAKE                NEAR TWO HARBORS                       IRON RANGE    WALES            34.70

252363N   _________   27  075  DMIR  LAKE                NEAR TWO HARBORS                       IRON RANGE    WALES            35.09

252364V   _________   27  075  DMIR  LAKE                NEAR TWO HARBORS                       IRON RANGE    WALES            36.56

252365C   _________   27  075  DMIR  LAKE                NEAR TWO HARBORS                       IRON RANGE    WALES            37.43

252366J   _________   27  075  DMIR  LAKE                NEAR TWO HARBORS                       IRON RANGE    WALES            37.87

DULUTH MISSABE AND IRON RANGE RAILWAY COMPANY                                      PAGE    1

FILL-IN-THE-BLANKS LIST FOR INDICATING SOLD CROSSING                                   01/11/94

ON THE               DIVISION, IRON RANGE     SUBDIVISION, IN THE  STATE OF MINNESOTA

DATE                 RAILROAD

ID. NO.    SOLD       ST CNTY  OLD    NEW   COUNTY        LOCATED      CITY       DIVISION      SUBDIVISION   BRANCH        MILEPOST

------------------------------------------------------------------------------------------------------------------------------------

252253D   _________   27  075  DMIR  ____  LAKE             NEAR TWO HARBORS                    IRON RANGE                     39.55

252239H   _________   27  075  DMIR  ____  LAKE             NEAR TWO HARBORS                    IRON RANGE    LOOP              2.40

252240C   _________   27  075  DMIR  ____  LAKE             NEAR TWO HARBORS                    IRON RANGE    LOOP              2.52

252241J   _________   27  075  DMIR  ____  LAKE             NEAR TWO HARBORS                    IRON RANGE    LOOP              3.14

252242R   _________   27  075  DMIR  ____  LAKE             IN   TWO HARBORS                    IRON RANGE    LOOP              4.19

252243X   _________   27  075  DMIR  ____  LAKE             NEAR TWO HARBORS                    IRON RANGE    LOOP              4.40

252244E   _________   27  075  DMIR  ____  LAKE             NEAR TWO HARBORS                    IRON RANGE    LOOP              4.77

252245L   _________   27  075  DMIR  ____  LAKE             NEAR TWO HARBORS                    IRON RANGE    LOOP              4.95

252246T   _________   27  075  DMIR  ____  LAKE             NEAR TWO HARBORS                    IRON RANGE    LOOP              5.14

252247A   _________   27  075  DMIR  ____  LAKE             NEAR TWO HARBORS                    IRON RANGE    LOOP              5.47

252218P   _________   27  075  DMIR  ____  LAKE             IN   TWO HARBORS                    IRON RANGE    MAIN           *******

252219W   _________   27  075  DMIR  ____  LAKE             IN   TWO HARBORS                    IRON RANGE    MAIN           *******

252230W   _________   27  075  DMIR  ____  LAKE             IN   TWO HARBORS                    IRON RANGE    MAIN           *******

252231D   _________   27  075  DMIR  ____  LAKE             IN   TWO HARBORS                    IRON RANGE    MAIN           *******

1                                        DULUTH MISSABE AND IRON RANGE RAILWAY COMPANY                                      PAGE    3

FILL-IN-THE-BLANKS LIST FOR UPDATING IDENTIFICATION INFORMATION                              12/22/93

ON THE                DIVISION, IRON RANGE     SUBDIVISION, IN THE STATE OF MINNESOTA

EFFECTIVE            RAILROAD       COUNTY

CROSSING   DATE     ST CNTY  OLD NEW/OR TYPE CITY                 DIVISION       SUBDIVISION    BRANCH         MILEPOST  RR-ID    WD

CLOSED      STREET

------------------------------------------------------------------------------------------------------------------------------------

252328A  _________  MN  137  DMIR        21  ST LOUIS                            IRON RANGE     X               ********

_______      NEAR MCKINLEY

J&L HAUL RD          _____________  _____________  _______________  ______  ________

252329G  _________  MN  137  DMIR        21  ST LOUIS                            IRON RANGE     X               ********

_______      IN   MCKINLEY

J&L PLANT RD         _____________  _____________  _______________  ______  ________

252337Y  _________  MN  137  DMIR        21  ST LOUIS                            IRON RANGE     X                  0.65

_______      IN   FRANKLIN

MINE RD              _____________  _____________  _______________  ______  ________

252338F  _________  MN  137  DMIR        21  ST LOUIS                            IRON RANGE     X                  0.73

_______      IN   FRANKLIN

MINE RD              _____________  _____________  _______________  ______  ________

252339M  _________  MN  137  DMIR        21  ST LOUIS                            IRON RANGE     X                  0.89

_______      IN   FRANKLIN

_____________  _____________  _______________  ______  ________

252327T  _________  MN  137  DMIR        21  ST LOUIS                            IRON RANGE     X                 17.11

_______      NEAR MCKINLEY

J&L HAUL RD          _____________  _____________  _______________  ______  ________

252331H  _________  MN  137  DMIR        21  ST LOUIS                            IRON RANGE     X                 19.69

_______      IN   GILBERT

MINING CO RD         _____________  _____________  _______________  ______  ________

252332P  _________  MN  137  DMIR        32  ST LOUIS                            IRON RANGE     X                 20.10  OH 20.1 X

_______      NEAR GILBERT

_____________  _____________  _______________  ______  ________

ON ATSF IN THE STATE OF OKLAHOMA

FILL-IN-THE-BLANKS LIST FOR UPDATING IDENTIFICATION INFORMATION                              01/11/94

OPERATING ON SAME OR SEPARATE TRACK

IN THE STATE OF OKLAHOMA

COUNTRY                     DIVISION                          RAILROAD ON             RAILROAD ON

CITY                        SUBDIVISION        MILE     SEP   SEPARATE          SAME  SAME

NUMBER   DATE     TYPE  ST   STREET                RR    BRANCH             POST     IND   TRACK             IND   TRACK

------------------------------------------------------------------------------------------------------------------------------------

GARFIELD                    SOUTHERN

ENID                        ENID

011704P  _______  33    OK   GRAND AVE            ATSF                        61.42  ___   ________________  ___   ________________

GARFIELD                    SOUTHERN                    YES   SLSF              NO

ENID                        ENID

011705W  _______  31    OK   2ND ST               ATSF                        61.54  ___   ________________  ___   ________________

GARFIELD                    SOUTHERN                    YES   SLSF              NO

ENID                        ENID

011706D  _______  31    OK   ENID BLVD 4TH ST     ATSF                        61.83  ___   ________________  ___   ________________

GARFIELD                    SOUTHERN                    YES   SLSF              NO

ENID                        ENID

011707K  _______  31    OK   10TH ST              ATSF                        62.44  ___   ________________  ___   ________________

GARFIELD                    SOUTHERN                    NO                      YES   SLSF

ENID                        ENID

011708S  _______  31    OK   16TH ST              ATSF   ELEV Y TRK            0.75  ___   ________________  ___   ________________

GARFIELD                    SOUTHERN                    YES   SLSF              NO

ENID                        ENID

011709Y  _______  31    OK   16TH                 ATSF                        62.99  ___   ________________  ___   ________________

GARFIELD                    SOUTHERN                    NO                      NO

ENID                        ENID

011710T  _______  31    OK   26TH ST              ATSF                        63.68  ___   ________________  ___   ________________

GARFIELD                    SOUTHERN                    NO                      NO

ENID                        ENID

011711A  _______  31    OK   30TH ST              ATSF                        63.94  ___   ________________  ___   ________________

GARFIELD                    SOUTHERN                    NO                      NO

ENID                        ENID

011712G  _______  31    OK                        ATSF                        64.94  ___   ________________  ___   ________________

DULUTH MISSABE AND IRON RANGE RAILWAY COMPANY                                      PAGE    1

FILL-IN-THE-BLANKS LIST                                                  12/22/93

FOR RAILROAD, CROSSING STATUS, WARNING DEVICES, TRAIN COUNTS, AND SPEEDS

CROSSING            STATE CNTY                                                                              LT1   MAX

COUNTY    EFFECTIVE BRANCH           MILEPOST    XB   FL   FL   FL   GT   GT   STP  DAY   DAY   NGT   NGT   PER   TT    MIN   MAX

ROAD      DATE       HWY #    RR          XBR  NR   OVR  NOV  MST  RW   OTH  STD  THRU  SWT   THRU  SWT   DAY   SPD   SPD   SPD

------------------------------------------------------------------------------------------------------------------------------------

ON Duluth, Missabe & Iron Range Railway Com

252245L   _________  MN   075  DMIR  000495  2    0    0    0    0    0    0    0     2     0     2     0   NO     22    10    22

LAKE                LOOP

____  ______ ___  ___  ___  ___  ___  ___  ___  ___  ____  ____  ____  ____  ____  ____  ____  ____

252247A   _________  MN   075  DMIR  000547  2    0    0    0    0    0    0    2     3     0     3     0   NO     22    10    22

LAKE                LOOP

CO 33    ____  ______ ___  ___  ___  ___  ___  ___  ___  ___  ____  ____  ____  ____  ____  ____  ____  ____

252248G   _________  MN   075  DMIR  002903  2    0    0    0    0    0    0    2     3     0     3     0   NO     35     5    35

LAKE                MAIN

UT 33    ____  ______ ___  ___  ___  ___  ___  ___  ___  ___  ____  ____  ____  ____  ____  ____  ____  ____

252249N   _________  MN   075  DMIR  002960  0    0    0    0    2    2    0    0     3     0     3     0   NO     35     5    35

LAKE                MAIN

CO 122   ____  ______ ___  ___  ___  ___  ___  ___  ___  ___  ____  ____  ____  ____  ____  ____  ____  ____

252250H   _________  MN   075  DMIR  003078  2    0    0    0    0    0    0    2     6     0     6     0   NO     35    20    35

LAKE                MAIN

CO 12    ____  ______ ___  ___  ___  ___  ___  ___  ___  ___  ____  ____  ____  ____  ____  ____  ____  ____

252251P   _________  MN   075  DMIR  003221  2    0    0    0    0    0    0    2     6     0     6     0   NO     35    20    35

LAKE                MAIN

CO 13    ____  ______ ___  ___  ___  ___  ___  ___  ___  ___  ____  ____  ____  ____  ____  ____  ____  ____

252252W   _________  MN   075  DMIR  003304  2    0    0    0    0    0    0    2     6     0     6     0   NO     35    20    35

LAKE                MAIN

CO 38    ____  ______ ___  ___  ___  ___  ___  ___  ___  ___  ____  ____  ____  ____  ____  ____  ____  ____

252254K   _________  MN   075  DMIR  004151  2    0    0    0    0    0    0    2     6     0     6     0   NO     35    20    35

LAKE                MAIN

CO       ____  ______ ___  ___  ___  ___  ___  ___  ___  ___  ____  ____  ____  ____  ____  ____  ____  ____

DULUTH MISSABE AND IRON RANGE RAILWAY COMPANY                                      PAGE    1

FILL-IN-THE-BLANKS LIST                                                  01/07/94

FOR TRAIN MOVEMENTS AND SPEEDS THRU PUBLIC-AT-GRADE CROSSINGS

CROSSING  EFFECTIVE  STATE  CNTY   RR   DIVISION       WDCODE     DAY-THRU  DAY-SWT  NGT-THRU   NGT-SWT   LT-1-MOV        NOTES

DATE                        SUBDIVISION    MILEPOST                                                            AND

BRANCH                    MX-TT-SP  MIN-SPD  MAX-SPD    SPD-SEL                 COMMENTS

------------------------------------------------------------------------------------------------------------------------------------

252239H   _________    27    075  DMIR                    7       3  ____   0  ____   3  ____   0  ____   NO  ____

MN               IRON RANGE         2.40

LOOP                     20  ____  10  ____  20  ____  NO  ____

252241J   _________    27    075  DMIR                    7       3  ____   0  ____   3  ____   0  ____   NO  ____

MN               IRON RANGE         3.14

LOOP                     20  ____  10  ____  20  ____  NO  ____

252242R   _________    27    075  DMIR                    4       3  ____   0  ____   3  ____   0  ____   NO  ____

MN               IRON RANGE         4.19

LOOP                     22  ____  10  ____  22  ____  N/A ____

252245L   _________    27    075  DMIR                    4       2  ____   0  ____   2  ____   0  ____   NO  ____

MN               IRON RANGE         4.95

LOOP                     22  ____  10  ____  22  ____  N/A ____

252247A   _________    27    075  DMIR                    4       3  ____   0  ____   3  ____   0  ____   NO  ____

MN               IRON RANGE         5.47

LOOP                     22  ____  10  ____  22  ____  N/A ____

252234Y   _________    27    075  DMIR                    4       0  ____   0  ____   0  ____   0  ____  YES  ____

MN               IRON RANGE      ********

MAIN                      5  ____   1  ____   5  ____  N/A ____

252236M   _________    27    075  DMIR                    4       0  ____   0  ____   0  ____   0  ____  YES  ____

MN               IRON RANGE      ********

MAIN                      5  ____   1  ____   5  ____  N/A ____

252237U   _________    27    075  DMIR                    4       0  ____   0  ____   0  ____   0  ____  YES  ____

MN               IRON RANGE      ********

MAIN                      5  ____   1  ____   5  ____  N/A ____

1                                                       STATE OF ALASKA                                                     PAGE    1

FILL-IN-THE-BLANKS LIST FOR CROSSINGS                                           12/22/93

WITH NO ADVANCED WARNING SIGNS

ON THE                DIVISION, ANCHORAGE      SUBDIVISION, IN THE COUNTY OF ANCHORAGE

STATE CNTY       ADV

EFFECTIVE    BRANCH           WRN    PAVEMENT         XB   FL   FL   FL   GT   GT   STP         HWY TYP

CROSSING    DATE       MILEPOST    RR   SGN    MARKINGS    XBR  NR   OVR  NOV  MST  RW   OTH  STD  AADT   AND NBR        ROAD

------------------------------------------------------------------------------------------------------------------------------------

868303J   ________     AK    020  ARR   NO   NONE           2    0    0    0    0    0    0    2     150           ARTILLERY RD

MAIN

127.98      ____  ___  _____________ ___  ___  ___  ___  ___  ___  ___  ___ ______  _______  _________________

868550B   ________     AK    020  ARR   NO   NONE           2    0    0    0    0    0    0    0     200           ORCA ST.

ZONE

114.30      ____  ___  _____________ ___  ___  ___  ___  ___  ___  ___  ___ ______  _______  _________________

868551H   ________     AK    020  ARR   NO   NONE           1    0    0    0    0    0    0    0     200           SITKA ST

ZONE

114.30      ____  ___  _____________ ___  ___  ___  ___  ___  ___  ___  ___ ______  _______  _________________

868552P   ________     AK    020  ARR   NO   NONE           1    0    0    0    0    0    0    0     200           SITKA ST

ZONE

114.30      ____  ___  _____________ ___  ___  ___  ___  ___  ___  ___  ___ ______  _______  _________________

868554D   ________     AK    020  ARR   NO   NONE           1    0    0    0    0    0    0    0     100           N. WRANGELL ST

ZONE

114.30      ____  ___  _____________ ___  ___  ___  ___  ___  ___  ___  ___ ______  _______  _________________

868564J   ________     AK    020  ARR   NO   NONE           1    0    0    0    0    0    0    0     400           GULL AVE

ZONE

114.30      ____  ___  _____________ ___  ___  ___  ___  ___  ___  ___  ___ ______  _______  _________________

910257X   ________     AK    020  ARR   NO   NONE           1    0    0    0    0    0    0    0     200           SMALL BOAT HARBOR

ZONE

114.30      ____  ___  _____________ ___  ___  ___  ___  ___  ___  ___  ___ ______  _______  _________________

910258E   ________     AK    020  ARR   NO   NONE           0    0    0    0    0    0    0    0     200           WESTERN AVE

ZONE

114.30      ____  ___  _____________ ___  ___  ___  ___  ___  ___  ___  ___ ______  _______  _________________

SPECIAL INFORMATION REPORTS

The following are examples of lists which can be produced to provide specific types of information.  In general, they are the same types of lists as the fill-in-the-blanks lists used for mass updating but without provision for writing in updated information.

The examples shown are:

1.	Stop Sign Eligibility

2.	Crossing with Automated Warning Devices with Min/Max Speed greater than 30 mph.

14

PUBLIC AT-GRADE HIGHWAY-RAIL CROSSINGS                                         PAGE    1

ELIGIBLE FOR STOPSIGNS*                                                  01/07/94

FOR Burlington Northern Railroad Company     IN ILLINOIS

WARNING DEVICE     LOCAL              HWY #            TTL  TYPE                  RR DIVISION AND

CROSSING CITY                  ROAD               COUNTY     AADT  TRN  TRACK       MILEPOST    SUBDIVISION   BRANCH

------------------------------------------------------------------------------------------------------------------------------------

063774X  CROSSBUCKS            RURAL              TR472        79   12  MAIN         231.97   GALESBURG       GALES-QUINCY

AUGUSTA                                  ADAMS                                       SECOND

063775E  CROSSBUCKS            RURAL              TR35         25   12  MAIN         231.23   GALESBURG       GALES-QUINCY

AUGUSTA                                  ADAMS                                       SEVENTH

063776L  CROSSBUCKS            RURAL              TR474        19   12  MAIN         230.88   GALESBURG       GALES-QUINCY

AUGUSTA                                  ADAMS                                       SEVENTH

063778A  CROSSBUCKS            RURAL              TR 19        25   12  MAIN         230.08   GALESBURG       GALES-QUINCY

AUGUSTA                                  ADAMS                                       SEVENTH

072410W  CROSSBUCKS            RURAL              TR 271      259   12  MAIN         253.90   GALESBURG       GALES-QUINCY

FOWLER                                   ADAMS                                       SEVENTH

072411D  CROSSBUCKS            RURAL              TR 166       59   12  MAIN         252.83   GALESBURG       GALES-QUINCY

FOWLER                                   ADAMS                                       SEVENTH

072414Y  CROSSBUCKS            RURAL              TR 261       59   12  MAIN         251.40   GALESBURG       GALES-QUINCY

FOWLER                                   ADAMS                                       SEVENTH

072416M  CROSSBUCKS            RURAL              TR 244       19   12  MAIN         249.62   GALESBURG       GALES-QUINCY

PALOMA                                   ADAMS                                       SEVENTH

072417U  CROSSBUCKS            RURAL              TR 286       25   12  MAIN         246.70   GALESBURG       GALES-QUINCY

PALOMA                                   ADAMS                                       SEVENTH

072422R  CROSSBUCKS            RURAL              TR 338      125   12  MAIN         244.28   GALESBURG       GALES-QUINCY

CAMP POINT                               ADAMS                                       SEVENTH

072427A  CROSSBUCKS            RURAL              TR 362A      50   12  MAIN         243.03   GALESBURG       GALES-QUINCY

CAMP POINT                               ADAMS                                       SEVENTH

072620L  CROSSBUCKS            RURAL              TR 5         19   12  MAIN         228.99   GALESBURG       GALES-QUINCY

LA PRAIRIE                               ADAMS                                       SEVENTH

------------------------------------------------------------------------------------------------------------------------------------

* CRITERIA PER FHWA TRAFFIC CONTROL DEVICES HANDBOOK (PASSIVE WARNING DEVICES; LOCAL ROADS; UNDER 400

ADT-RURAL, 1500 ADT-URBAN; AT LEAST 10 TRAINS/DAY; SINGLE TRACK).  REFER TO THE HANDBOOK FOR DETAILS.

PUBLIC HIGHWAY-RAIL CROSSINGS                                              PAGE   19

IN KANSAS                                                         01/12/94

EQUIPPED WITH AUTOMATIC WARNING DEVICES

AND WITH TRAIN SPEED VARIATIONS GE 30 MPH

IN THE COUNTY OF ELLSWORTH

MX                    TT            SPD

XING    COUNTY                                                                              TT      MAX    MIN    SPD     SPD   SEL

NUMBER   MAP-REF         CITY               ROAD            HWY NBR RLRD WARNING DEVICE      SP      SPD    SPD    VAR     VAR   EQP

------------------------------------------------------------------------------------------------------------------------------------

814511K   10          IN ELLSWORTH          DOUGLAS AVE     K 14    UP   FLASHING LIGHTS     35       35      5     30      30    NO

814525T  545        NEAR WILSON             GOLDEN BELT HWY RS- 240 UP   FLASHING LIGHTS     60       60     20     40      40    NO

814537M  539          IN WILSON                             K-232   UP   AUTOMATIC GATES     40       40     10     30      30    NO

814539B  3            IN WILSON             AVE "E"                 UP   FLASHING LIGHTS     40       40      5     35      35    NO

814541C  2            IN WILSON             AVE. F                  UP   FLASHING LIGHTS     40       40      5     35      35    NO

818248A   13          IN KANOPOLIS          MISSOURI ST             UP   AUTOMATIC GATES     60       60     20     40      40    NO

APPENDIX C

FRA MAGNETIC TAPE FORMAT

APPENDIX C

National Highway-Rail Grade Crossing Inventory

Data Field Specifications for Magnetic Tape Updates

Columns 1 - 25

The first 25 columns of each record are to contain the identification data for the crossing.  The data in these columns is validated against the inventory record for the crossing before any of the update data is posted.

Columns 26 - 80

Columns 26 - 80 may contain one or more sets of information.  Each set of information or data-unit identifies the data element and the data.  It consists of four parts:  the data element identifier, a comma, the data, and a slash.  The slash signifies the end of the data-unit.

The first set of characters, followed by a comma, is the data element number.  The remaining set of characters, from the comma to the slash, is the data itself.  The data can again be divided into two parts, the first character(s) of data being a number and the remaining characters being the description.  This applies only to the following elements:

a.	Part I	16C	Private crossing signs or signals

b.	Part II	3	Non-main tracks

c.	Part II	4	Other RR separate track

d.	Part II	5	Other RR same track

e.	Part II	6A-05	Other signs

f.	Part II	6A-07	Other signs

g.	Part II	6B-14	Other flashing lights

A slash is entered when the description is complete.  For descriptive data the length is equal to or less than the number of spaces indicated on the form.  For numeric information the data length is the exact number, zero filled to the left.

When one data element is completed, another may be started by entering the data element number, a comma, the data, then a slash.  This can be continued up to and including column 80.  If the ending slash was not entered in column 80, then duplicate the first 25 columns and continue in column 26 on the next line.  A slash must end a data unit even if the information has to be entered with only the identification information and a slash in column 26.

Since there are no easily keyed Roman equivalents of sections I, II, III, and IV, the numerics 1, 2, 3, and 4 are to be used.

The following tables present a concise guide to entering the data accurately and in the proper format.  Table C-1 begins with the guidelines for completing the identification data into columns 1 through 25.  Tables C-2 through C-5 continue with the rules for entering the data into columns 26 - 80, as necessary, for the data elements within each of the sections of the four-part form.  Information for the same set of identification data can be continued from column 80 to column 26 of the following line, with no spaces, as long as a data unit ends with a slash.  The identification data must be repeated in columns 1 - 25 for each line in which its data is entered.

| Field Name | Length/Type | Position | Valid Values |
| --- | --- | --- | --- |
| Init(iating) Agency (A) | (1N) | 1 | "1" or "2" |
| Crossing No. (B) | (6N, + 1A) | 2-8 | First 6 = Num, 7th = Alpha, leading zeros must be entered, valid check character |
| Reason (C) | (1N) | 9 | "1" = Change "3" = Closed crossing |
| E(ffective) Date (D) | (6N) | 10-15 | "MMDDYY" - Month/Day/Year |
| State | (2A) | 16-17 | Valid 2-digit State code |
| County | (3N) | 18-20 | Valid county code |
| RR Code | (4A) | 21-24 | Valid DOT-AAR code |
| Control | (1A) | 25 | Leave blank |
| Sample Updates: (seven closed and two changes) 1631267H305059337071DTNC 1631269W305059337071DTNC 1631270R305059337071DTNC 1631271X305059337071DTNC 1631272E305059337071DTNC 1631273L305059337071DTNC 1631274T305059337071DTNC 1163548A104279317031CSX 110,51ST COURT(1-WAY)/114,0001.28/221,010/222,000/223,0 1163548A104279317031CSX 10/ 1346514N104299317189CSX 114,0434.26/221,030/222,000/223,030/232,01INDUSTRY /26 1346514N104299317189CSX 01,0/2613,4/2618,2/32,3/38,6/ This is an example of two types of updates. The first seven lines represent updates to close seven grade crossings. Note that for each closing, only the identification data is required in the 80-character record. The last four lines are changes to two grade crossings. |  |  |  |

Table C-1.  Identification Data

| Data Element Number | Field Name | Length/Type | Valid Values |
| --- | --- | --- | --- |
| 11, | RR Operating Company (1) | (4A) | Valid DOT-AAR railroad code |
| 12, | RR Division (2) | (14 A/N) | Any 1-14 alphanumeric |
| 13, | RR Subdivision (3) | (14 A/N) | Any 1-14 alphanumeric |
| 14, | State (4) | (2A) | Valid 2-digit State code |
| 15, | County (5) | (3 A/N) | Valid 3-digit county code |
| 16, | County Map Ref (6) | (10 A/N) | Any 1-10 alphanumeric |
| 17, | City (7) | (4N) | Valid 4-digit city code |
| 18, | Near (EST) City | (1 A/N) | "0" if in-city, "1" if near city |
| 19, | Hwy Type | (7 A/N) | Any 1-7 alphanumeric |
| 110, | Street/Road (10) | (17 A/N) | Any 1-17 alphanumeric |
| 111, | RRID No. (11) | (10 A/N) | Any 1-10 alphanumeric |
| 112, | TT Station (12) | (6 N) | Valid 6-digit SPLC code |
| 113, | Branch (13) | (15 A/N) | Any 1-15 alphanumeric |
| 114, | Milepost (14) | (7 A/N) | Any alphanumeric (must have 2-digits to right of decimal) |
| 115, | Ped Xing (15) | (1 A/N) | "1", "2", or "3" |
| 116A, | Priv X-use (16A) | (1 A/N) | "1" to "4" |
| 116B, | Priv X-type (16B) | (1 A/N) | "5", "6", or "7" |
| 116C, | Priv X-WD (16C) | (1 A/N; 15A) | "8", "9", or "0" (+ 1 to 15 character description if "8" or "9") |
| 117, | Pub Xing (17) | (1 A/N) | "1", "2", or "3" |
| Sample Updates: (Private to Public) 1632918W112018745061CSX 12,FL /13,HARTSVILLE /16,31 /17,01 1632918W112018745061CSX 90/18,0/19,SC0154 /112,445130/113,SJA /114,0 1632918W112018745061CSX 331.39/117,1/118,NINWX/211,00/212,00/213,00/214,00/215, 1632918W112018745061CSX 1/221,025/222,005/223,015/231,1/232,00 /24,2 1632918W112018745061CSX /25,2 /2601,2/2620,0/27,1/ 1632918W112018745061CSX 28,3/29,2/31,1/32,3/33,2/34,2/35,1/36,2/37,1/38,3/39,2/ 1632918W112018745061CSX 310,2/41,04/42,1/43,07/44,001050/45,08/ This is an example of a crossing that was either Private or Pedestrian and is being changed to Public-at-Grade. For this type of change, all of Parts II through IV information must be provided. Only one type of crossing can be given. Sample updates: (RR Change - TT to CSX) 1851573G109289239173TT 11,CSX /222,020/223,025/ This update shows a change in the Railroad Operating Company from TT to CSX. |  |  |  |

Table C-2.  Part I Data

Table C-3.  Part II Data

| Data Element Number | Field Name | Length/Type | Valid Values |
| --- | --- | --- | --- |
| 211, | Day Thru Trn (1A-1) | (2N) | 00 to 99 |
| 212, | Day Switch Trn (1A-2) | (2N) | 00 to 99 |
| 213, | Night Thru Trn (1A-3) | (2N) | 00 to 99 |
| 214, | Night Switch Trn (1A-4) | (2N) | 00 to 99 |
| 215, | Less 1/day (1A-5) | (1N) | "1" or "0" |
| 221, | Max Timetable Speed (2A-1) | (3N) | 001 to 130 |
| 222, | Typ Min Spd | (3N) | 000 to 130 |
| 223, | Typ Max Spd (2B-3) | (3N) | 001 to 130 |
| 231, | Main Track (3-1) | (1N) | 0 to 9 |
| 232, | Other Track (3-2) | (12 A/N) | First 2 = Num (00-99) last 10 = any 1 to 10 alphanumeric |
| 24, | RR Sep Trk (4-1 & 2) | (17 A/N) | "1" or "2" (+ valid DOT-AAR RR code(s) in groups of four characters each if "1") |
| 25, | RR Same Trk (5-1 & 2) | (17 A/N) | "1" or "2" (+ valid DOT-AAR RR code(s) in groups of four characters each if "1") |
| 2601, | Xbucks - Ref-01 (6A-01) | (1N) | 0 to 9 |
| 2602, | Xbucks - Non-02 (6A-02) | (1N) | 0 to 9 |
| 2603, | Stop - Stand-03 (6A-03) | (1N) | 0 to 9 |
| 2604, | Stop - Other-04 (6A-04) | (1N) | 0 to 9 |
| 2605, | Oth Sign-05 (6A-05 & 06) | (11 A/N) | First character = 0 to 9, next 10 = any 1 to 10 alphanumeric if first character is greater than 0 |
| 2607, | Oth Sign-07 (6A-07 & 08) | (11 A/N) | First character = 0 to 9, next 10 = any 1 to 10 alphanumeric if first character is greater than 0 |
| 2609, | Gates - 09 (6B-09) | (1N) | 0 to 9 |
| 2610, | Gates - 10 (6B-10) | (1N) | 0 to 9 |
| 2611, | Fl-Over-Tr-11 (6B-11) | (1N) | 0 to 9 |
| 2612, | Fl-Not-Over-12 (6B-12) | (1N) | 0 to 9 |
| 2613, | Fl-Mast-13 (6B-13) | (1N) | 0 to 9 |
| 2614, | Fl-Oth-14 (6B-14 & 15) | (10 A/N) | First character = 0 to 9, next 9 = any 1 to 9 alphanumeric if first character is greater than 0 |
| 2616, | Tr-Sig-16 (16B16) | (1N) | 0 to 9 |
| 2617, | Wigwags-17 (6B-17) | (1N) | 0 to 9 |
| 2618, | Bells-18 (6B-18) | (1N) | 0 to 9 |
| 2619, | Special W.D. (6C-19) | (20 A/N) | Any 1 to 20 alphanumeric |
| 2620, | (6D) Nosigns-Sig-20 (6D-20) | (1N) | "0" or "1" |
| 27, | Commercial Power (7) | (1N) | "1" or "2" |
| 28, | Speed Selection (8) | (1N) | "1", "2", or "3" |
| 29, | Signal Method (9) | (1N) | "1" or "2" |
| Sample Updates: 1623372N109189212121CSX 114,0717.29/221,045/222,045/223,045/ 1623373V109189212121CSX 114,0718.20/221,045/222,045/223,045/ 1623374C109189212121CSX 114,0718.89/221,045/222,045/223,045/ 1170029P112018917021CIM 15,179/110,MCLEAN /111,105A /114,0010.33 1170029P112018917021CIM /117,1/211,04/212,04/213,04/214,00/221,010/222,001/223, 1170029P112018917021CIM 010/231,1/232,00 /24,2 /25,2 1170029P112018917021CIM /2613,2/27,1/28,2/29,2/31,3/32,3/33,2/34,2 1170029P112018917021CIM /35,1/36,3/37,2/38,6/39,2/310,1/41,08/42,2/43,19/44,000 1170029P112018917021CIM 059/45,01/ The following is a list of cross-field checks which should be performed to insure consistency in the Part II data: a. If the sum of daily train movements is zero (211,00/212,00/213,00/214,00, the Less Than One Movement field must equal "1" (215,1/). Conversely, if the sum of daily train movements exceeds 1, field 215, cannot equal "1". b. Maximum speed over crossing (223,) must not exceed maximum timetable speed (221,). c. Minimum speed over crossing (222,) must not exceed maximum speed over crossing (223,). d. Main tracks (231,) and other tracks (232,) cannot both be 0. e. If the sum of all warning devices (2601, to 2619,) exceeds 0, Field 2620, cannot equal "1". Conversely, if the total of all warning devices equals 0, Field 2620, must equal "1". f. If the sum of Train Activated Devices (2601, to 2618,) equals 0, Field 28, cannot equal "1". |  |  |  |

Table C-3.  Part II Data (Continued)

| Data Element Number | Field Name | Length/Type | Valid Values |
| --- | --- | --- | --- |
| 31, | (1) Type Devel | (1N) | "1" to "5" |
| 32, | (2) Xing Angle | (1N) | "1", "2", or "3" |
| 33, | (3) No Tr Lanes | (1N) | 0 to 9 |
| 34, | (4) Truck Pullout Lns | (1N) | "1" or "2" |
| 35, | (5) Hwy Paved? | (1N) | "1" or "2" |
| 36, | (6) Pavement Markings | (1N) | "1" or "2" or "3" or "4" |
| 37, | (7) Advance Warning | (1N) | "1" or "2" |
| 38, | (8) Crossing Surface | (1N) | "0" to "9" |
| 39, | (9) Down Street | (1N) | "1" or "2" |
| 310, | (10) Intersecting Hwy? | (1N) | "1" or "2" |
| Sample Updates: 2079899V106219355063BN 38,2/ 2079900M106219355063BN 38,2/ 2079944M106219355011BN 37,2/38,2/ 2079945U106219355011BN 38,2/ 2079951X106219355011BN 38,2/ 2079957N106219355011BN 36,1/ Cross-Field Edit: In Part III, if the highway is not paved (35,2/) pavement markings can only be "None" (36,3/). |  |  |  |

Table C-4.  Part III Data

| Data Element Number | Field Name | Length/Type | Valid Values |
| --- | --- | --- | --- |
| 41, | (1) Highway System | (2N) | "01" to "04" or "08" |
| 42, | (2) State Highway? | (1N) | "1" or "2" |
| 43, | (3) Functional Class | (2N) | "01, 02, 06, 07, 08, 09, 11, 12, 14, 16, 17, and 19" |
| 44, | (4) AADT | (6N) | 000001 to 999999 |
| 45, | (5) Percent Trucks | (2N) | 00 to 99 |
| Sample Updates: 2229275V106149321161CSX 44,002450/ 2229276C106149321161CSX 44,000600/ 2229278R106149321161CSX 44,001490/ 2229279X106149321161CSX 44,001100/ 2229280S106149321161CSX 44,001000/ 2229281Y106149321161CSX 44,001440/ 2229282F106149321161CSX 44,000800/ |  |  |  |

Table C-5.  Part IV Data

APPENDIX D

GX DATA FILE STRUCTURE

APPENDIX D

GX DATA FILE STRUCTURE (.DBF FORMAT)

FIELD NAME	FIELD TYPE		WIDTH	FULL NAME OF THE FIELD

CROSSING		Character		7		Crossing Number

RRCODE		Character		4		Railroad

TYPE_XING		Character		1		Crossing Type

POS_XING		Character		1		Crossing Position

STATE_CD		Character		2		State numeric code

COUNTY_CD		Character		4		County numeric code

CITY_CD		Character		4		City numeric code

NEAREST		Character		1		Nearest city

RR_DIV		Character		14		Railroad Division Name

RR_SUBDIV		Character		14		Railroad Sub-Division Name

HIGHWAY		Character		7		Highway Name

STREET		Character		17		Street Name

RR_ID			Character		10		Railroad Identification Number

TTCODE		Character		6		TimeTable Station numeric code

BRANCH		Character		15		Branch or line name

MILEPOST		Character		6		Railroad Milepost

MAP_REF		Character		10		County Map Reference Number

A_PRV_CAT		Character		1		Category of Private Crossing

C_PRV_IND		Character		1		Private crossing signs

PRV_SIGNS		Character		15		Private Xing signs - specify

EFFEC_DATE	Character		6		Effective Date

INIT_AGY		Character		1		Initiating agency

REASON		Character		1		Reason for Update

DAY_THRU		Numeric		2		Day thru train movements

DAY_SWT		Numeric		2		Day switching train movements

NIGHT_THRU	Numeric		2		Night thru train movement

NIGHT_SWT		Numeric		2		Night switching train movement

LESS_1_MOV		Character		1		Less than 1 movement per day?

MAX_TT_SPD	Numeric		3		Maximum time Table speed

MIN_SPD		Numeric		3		Minimum speed range over Xing

MAX_SPD		Numeric		3		Maximum speed range over Xing

MAIN_TRK		Numeric		1		Number of Main tracks

OTH_TRK		Numeric		2		Number of Other tracks

OTH_DESCRP	Character		10		Description of other tracks

SEP_IND		Character		1		Other RR on sep. trk at Xing

SEP_RR		Character		16		Specify Railroads

FIELD NAME	FIELD TYPE		WIDTH	FULL NAME OF THE FIELD

SAME_IND		Character		1		Other RR on same track at Xing

SAME_RR		Character		16		Specify Railroads

XBUCK_RF		Numeric		1		Reflectorized crossbucks

XBUCK_NRF		Numeric		1		Non-reflectorized crossbucks

STAND_STOP	Numeric		1		Standard Highway Stop sign

OTH_STOP		Numeric		1		Other Stop Signs

OTH_SG1		Numeric		1		Number of other signs #1

OTH_SGDES1	Character		10		Specify other signs

OTH_SG2		Numeric		1		Number of other signs #2

OTH_SGDES2	Character		10		Specify other signs

GATE_RW		Numeric		1		Red/White reflectorized gates

GATE_OTH		Numeric		1		Number of other colored gates

FLASH_OV		Numeric		1		Lights over traffic lanes

FLASH_NOV		Numeric		1		Lights not over traffic lanes

FLASH_MAST	Numeric		1		Mast Mounted flashing lights

OTH_FLASH		Numeric		1		Other flashing lights

OTH_FLDESC	Character		9		Description of other lights

HWY_SIGNAL	Numeric		1		Highway Traffic Signals

WIGWAGS		Numeric		1		Wigwags

BELLS		Numeric		1		Bells

SPEC_PRO		Character		20		Device not train activated

NO_SIGNS		Character		1		No signs or Signals

COM_POWER	Character		1		Commercial Power available

SGNL_EQP		Character		1		Is track equipped with signals

SPD_SELECT		Character		1		Speed of selection trains

TYP_DEVEL		Character		1		Type of development

HWY_PVED		Character		1		Highway paved

DOWN_ST		Character		1		Does track run down street

PAVE_MARKS	Character		1		Pavement Markings

HWY_NEAR		Character		1		Nearby intersecting highway

ADV_WARN_S	Character		1		Advance Warning signs ?

XNG_ANGLE		Character		1		Smallest crossing angle

XNG_SURFAC	Character		1		Crossing surface

TRAF_LANE		Character		1		No. of Traf. lanes crossing RR

TRUCK_LANE	Character		1		Truck Pullout lanes present?

ST_HWY_1		Character		1		Is crossing on Highway System?

HWY_SYS		Character		2		Highway System Code Number

FUNC_CLS		Character		2		Functional Class of road /Xing

AADT			Character		6		Estimate AADT

PERC_TRUCK	Character		2		Estimate percent trucks

CHGDATE		Date     		8		Date of record update

IDENT		Character		4		RESERVED

OUTFLAG1		Character		1		RESERVED

APPENDIX E

RAILROAD CODES

as of November, 1996

APPENDIX E

RAILROAD CODES

ARE		A & R Line Inc.

AGRD		A&G Railroad, L.L.C. (formerly Abbeville-Grimes - AG)

AG		Abbeville-Grimes Rwy Co (Use Code AGRD - A&G RR, L.L.C.)

ACWR		Aberdeen, Carolina & Western Railroad

AR		Aberdeen & Rockfish Railroad Company

AKDN		Acadiana Railway Company

ADCX		Adirondack Scenic Railroad (Previously Adir. Centennial)

ADBF		Adrian & Blissfield Railroad

AB		Akron Barberton Cluster Railway Company

ABB		Akron & Barberton Belt Railroad Company

ALAB		Alabama Railroad Company, Incorporated

AF		Alabama & Florida Railway Company

AGS		Alabama Great Southern Railroad (NS)

ABL		Alameda Belt Line

AGCR		Alamo Gulf Coast Railroad Company

ARR		Alaska Railroad Corporation

APRR		Albany Port Railroad

ARC		Alexander Railroad Company

AWW		Algers, Winslow & Western Railway Company

ALQS		Aliquippa & Southern Railroad Company

AVR		Allegheny Valley Railroad Company

ALC		Allegheny Central Railroad

ALY		Allegheny And Eastern Railroad

AL		Almanor Railroad Company

ALS		Alton & Southern Railroad

AMC		Amador Central Railroad Company

AMHR		Amhearst Railroad Industries, Incorporated/Landisville RR

ATK		Amtrak (National Railroad Passenger Corporation)

ACRC		Andalusia & Conecuh Railroad Company, Incorporated

ANR		Angelina & Neches River Railroad Company

AA		Ann Arbor Railroad

APA		Apache Railway Company

AN		Apalachicola Northern Railroad Company

APNC		Appanoose County Community Railroad Company

ARA		Arcade & Attica Railroad Corporation

AZCR		Arizona Central Railroad, Incorporated

ARZC		Arizona & California Railroad Company

AZER		Arizona Eastern Railroad

AKMD		Arkansas Midland Railroad Company, Incorporated

ALM		Arkansas Louisiana & Mississippi Railroad Company

AM		Arkansas & Missouri Railroad Company

AOK		Arkansas-Oklahoma Railroad Inc.

ACRY		Arkansas Central Railway Company

AVL		Aroostook Valley Railroad Company

ASRY		Ashland Railway Incorporated

ADN		Ashley, Drew & Northern Railway Company(Use Code ALM)

ACJR		Ashtubula, Carson & Jefferson Railroad

ATSF		Atchison, Topeka & Santa Fe Railway Company

ASML		Atlanta, Stone Mountain & Lithonia

ATW		Atlantic & Western Railway, L.P.

AGLF		Atlantic & Gulf Railroad

ATCX		Austin & Texas Central Railroad

AUNW		Austin & Northwestern Railroad

ATLT		Austin, Todd & Ladd Railroad Company

ATC		Autotrain Corporation

BLA		Baltimore & Annapolis Railroad Company

BOCT		Baltimore & Ohio Chicago Terminal Railroad Company

BAR		Bangor & Aroostook Railroad

BKRR		Batten Kill Railroad

BXN		Bauxite & Northern Railway Company

BCLR		Bay Colony Railroad

BAYL		Bay Line Railroad, L.L.C.

BMH		Beaufort & Morehead Railway Incorporated

BEEM		Beech Mountain Railroad Company

BML		Belfast & Moosehead Lake Railroad Company

BHRX		Bellefonte Historical Railroad Society

BRC		Belt Railway Company Of Chicago

BMS		Berlin Mills Railway

BLE		Bessemer & Lake Erie Railroad Company

BSDA		Bi-State Development Agency, St. Louis (Operated By RSM)

BSFX		Big South Fork Scenic Railway

BS		Birmingham Southern Railroad Company

BHC		Black Hills Central Railroad

BRW		Black River & Western Railroad

BLOL		Bloomer Shippers Connecting Railroad Company

BMRG		Blue Mountain & Reading Railroad

BRTR		Blue Rock Transportation Company

BRRX		Blue Rapids Railway

BLMR		Blue Mountain Railroad, Incorporated

BRMI		Bluegrass Railroad Museum, Incorporated

BSV		Boone Scenic Valley

BOP		Border Pacific Railroad

BM		Boston & Maine Corporation

BRAN		Brandon Corporation

BSRX		Brandywine Scenic Railroad Company

BVRY		Brandywine Valley Railroad Company

BITY		Bristol Industrial Terminal Railway

BWDX		Broadway Dinner Train

BRG		Brownsville & Rio Grande International Railroad

BCRR		Buckeye Central Scenic Railroad

BB		Buckingham Branch Railroad Company

BSOR		Buffalo Southern Railroad, Incorporated

BPRR		Buffalo & Pittsburgh Railroad, Incorporated

BJRY		Burlington Junction Railway

BN		Burlington Northern Railroad Company

CSKR		C&S Railroad Corporation

CALX		Caddo, Antonine And Little Missouri

CTML		Cairo Terminal Railroad Company

CWCY		Caldwell County Railroad Co.

CWR		California Western Railroad

CFNR		California Northern Railroad Company

CSRM		California State Railroad Museum

CSP		Camas Prairie Railroad Company

CCRA		Camp Chase Industrial Railroad Corp

CN		Canadian National - North America

CARX		Canadian Atlantic Railroad (Use Code CDAC)

CDAC		Canadian American Railroad Company

CPVM		Canadian Pacific Railway (Vermont & Michigan)

CP		Canadian Pacific Railway Company

CFWR		Caney Fork & Western Railroad

CTN		Canton Railroad Company

CCRX		Cape Cod Railroad

CF		Cape Fear Railways, Incorporated

CMSX		Cape May Seashore Lines, Inc.

CHSX		Carbon Schuykill

CALA		Carolina Southern Railroad Company

CLNA		Carolina Coastal Railway, Incorporated

CRIJ		Carolina Rail Services Company

CPDR		Carolina Piedmont Division-South Carolina Central Railroad

CARR		Carrollton Railroad (CSX)

CKSI		Carthage, Knightstown & Shirley Railroad

CASS		Cass Scenic

CMRR		Catskill Mountain Railroad

CBRX		CBEC Railway, Inc.

CVAR		Cedar Valley Railroad

CEDR		Cedar River Railroad Company

CIC		Cedar Rapids & Iowa City Railway Company

CRLX		Centex Rail Link, Limited

CERA		Central Railroad Company Of Indianapolis

CEIW		Central Indiana & Western Railroad Company

CGA		Central Of Georgia Railroad Company (NS)

CCT		Central Calfornia Traction Company

CTRN		Central Of Tennessee Railway & Navigation Company

CORP		Central Oregon & Pacific Railroad, Inc.

CV		Central Vermont Railway (Out Of Operation)

CM		Central Montana Railroad

CNYK		Central New York Railroad Corporation

CIND		Central Railroad Company Of Indiana

CKRY		Central Kansas Railway, Incorporated

CMGN		Central Michigan Railway Company

CGNE		Champagne Railroad, Incorporated

CIRR		Chattahoochee Industrial Railroad

CCKY		Chattooga & Chickamauga Railroad

CHNY		Cheney Railroad Company, Incorporated

CHRR		Chesapeake Railroad Company

CHW		Chesapeake Western Railway

CA		Chesapeake & Albemarle Railroad Company

CHR		Chestnut Ridge Railway Company

CWP		Chicago, West Pullman & Southern Railroad Company

CRI		Chicago River And Indiana Railroad Company

CHTT		Chicago Heights Terminal Transfer Railroad Company

CIW		Chicago & Illinois Western Railroad

CCUO		Chicago-Chemung Railroad Corporation

CSS		Chicago Southshore & South Bend Railroad

CRL		Chicago Rail Link

CNW		Chicago And North Western Railway Company

CIM		Chicago & Illinois Midland Railway Company

CC		Chicago, Central & Pacific Railroad Company

CSL		Chicago Short Line Railway Company

CBRM		Chillicothe-Brunswick Rail Maintenance Authority

CVR		Cimarron Valley Railroad L C

CNTP		Cincinnati, New Orleans & Texas Pacific Railway (NS)

CTER		Cincinnati Terminal Railway Company

COP		City Of Prineville Railway

CCRR		Claremont Concord Railroad Corporation

CLP		Clarendon & Pittsford Railroad Company

CTR		Clinton Terminal Railroad Company

CRLE		Coe Rail Incorporated

CISD		Colonel's Island Railroad Company

CW		Colorado & Wyoming Railway Company

CS		Colorado And Southern Railway

CT		Columbia Terminal Railroad Company

CBCX		Columbia Business Center

CLC		Columbia & Cowlitz Railway Company

CNOW		Columbia & Northern Railway Company

CUOH		Columbus & Ohio River Railroad

CAGY		Columbus & Greenville Railway Company

CWRY		Commonwealth Railway, Incorporated

CBL		Conemaugh & Black Lick Railroad Company

CDT		Conn. Dept. Of Transportation

CCCL		Connecticut Central Railroad

CTV		Connecticut Valley

CGBX		Consolidated Grain & Barge Company

CR		Consolidated Rail Corporation

CONW		Conway Scenic Railroad

CACV		Cooperstown & Charlotte Valley Railway Corporation

CMRX		Coopersville & Marne Railroad

CBRY		Copper Basin Railway, Incorporated

CRCX		Cornhusker Rail Car Services, Inc.

CCTA		Corpus Christi Terminal Association

CSRX		Corydon 1883 Scenic Railroad

CBGR		Council Bluffs Railway Company

CPRS		CP Rail System/CP Tarrif, Inc.

COER		Crab Orchard & Egyptian Railroad

CYCY		Crystal City Railroad, Incorporated

CSX		CSX Transportation

CTSR		Cumbres & Toltec Scenic Railroad

CVSX		Cuyahoga Valley Scenic Railway

CUVA		Cuyahoga Valley Railway Company

DAIR		D & I Railroad Company

DME		Dakota, Minnesota & Eastern Railroad

DAKR		Dakota Railroad

DI		Dakota & Iowa Railroad

DMVW	Dakota, Missouri Valley & Western Railroad, Incorporated

DSRC		Dakota Southern Railway Company

DGNO		Dallas, Garland & Northeastern Railroad

DTRR		Danbury Terminal Railroad Company

DMM		Dansville & Mount Morris Railroad Company

DR		Dardanelle & Russellville Railroad

DRI		Davenport, Rock Island & North Western Railway Company

DQE		De Queen & Eastern Railroad Company

DCRX		Deadwood Central Railroad Company

DT		Decatur Junction Railway Company

DH		Delaware & Hudson Railway Company

DL		Delaware Lackawanna Railroad (replaces LVAL,lRWY)

DURR		Delaware & Ulster Rail Ride

DV		Delaware Valley Railroad Company, Inc.

DC		Delray Connecting Railroad Company

DVS		Delta Valley & Southern Railway Company

DSRR		Delta Southern Railroad Company

DRGW		Denver & Rio Grande Western Railroad Company

DRIR		Denver Rock Island Railroad

DUT		Denver Union Terminal Railway

DOE		Department Of Energy

DOD		Department Of Defense - Other

DLWR		Depew, Lancaster & Western Railroad Company, Incorporated

DKS		Doniphan, Kensett & Searcy Railway

DCRR		Dubois County Railroad

DMIR		Duluth, Missabe & Iron Range Railway Company

DNE		Duluth & Northeastern Railroad Company

DWP		Duluth, Winnipeg & Pacific Railway

DSNG		Durango & Silverton Narrow Guage Railroad Company

DS		Durham And Southern Railway Company

DRHY		Durham Transport Incorporated

EPRY		East Penn Railways, Inc.

EACH		East Camden & Highland Railroad Company

EJR		East Jersey Railroad & Terminal Company

ECBR		East Cooper & Berkeley Railroad

ETRY		East Tennessee Railway, L.P.

EEC		East Erie Commercial Railroad

EPTC		East Portland Traction Company

EIRC		Eastern Illinois Railroad Company

EIRR		Eastern Idaho Railroad

ESHR		Eastern Shore Railroad Incorporated

EARY		Eastern Alabama Railway Company

EMRX		Eastern Maine Railroad

EMRY		Eastern Maine Railroad Company

TXTX		Econo Rail Corporation

EDW		El Dorado & Wesson Railway Company

EJE		Elgin, Joliet & Eastern Railway Company

ELKR		Elk River Railroad, Incorporated

EERZ		Ellis & Eastern Company (Use EE)

ETMX		Entertrainment Line

ELS		Escanaba & Lake Superior Railroad Company

EKNA		Eureka Springs & North Arkansas Railroad

EV		Everett Railroad Company

FMRC		Farmrail Corporation

FRA		Federal Railroad Administration

FMWX	Fillmore And Western

FGLK		Finger Lakes Railway Corporation

FIR		Flats Industrial Railroad Company

FGC		Florida Gulf Coast Railrod Museum, Incorporated

FWCR		Florida West Coast Railway

FMID		Florida Midland Railroad Company, Incorporated

FCEN		Florida Central Railroad Company

FEC		Florida East Coast Railway Company

FNOR		Florida Northern Railroad Company, Incorporated

FAPR		Floydada & Plainview Railroad Company

FP		Fordyce & Princeton Railroad Company

FSR		Fort Smith Railroad Company

FWWR	Fort Worth & Western Railroad

FVW		Fox Valley & Western Ltd.

FEVR		Freemont & Elkhorn Valley Railroad

FWHS		Ft. Wayne Historical Society

GVSR		Galveston Railroad, L.P.

GCW		Garden City Western Railway Company

GWWR	Gateway Western Railway

GWWE	Gateway Eastern Railroad Company

GNWR		Genesee & Wyoming Railroad Company

GRR		Georgetown Railroad Company

GC		Georgia Central Railroad

GAAB		Georgia & Alabama Division

GFRR		Georgia & Florida Railroad Co., Inc.

GNRR		Georgia Northeastern Railroad Company

GGS		Georgia Great Southern Division-South Carolina

GSWR		Georgia Southwestern Railroad Division

GMA		Georgia Marble

GSF		Georgia Southern & Florida Company (NS)

GWRC		Georgia Woodlands Railroad

GPSX		Gettysburg Passenger Service

GETY		Gettysburg Railroad Company

GRC		Glasgow Railway Company

GLSR		Gloster Southern Railroad

GCR		Gold Coast Railroad

GGMX		Golden Gate Railroad Museum

GTRA		Golden Triangle Railroad Company

GCRC		Golden Cat Railroad Corporation

GU		Grafton & Upton Railroad Company

GNBC		Grainbelt Corporation

GR		Grand Rapids Eastern Railroad Incorporated

GTW		Grand Trunk Western Railroad Incorporated

GCRX		Grand Canyon Railway

GSM		Great Smokey Mountain Railroad

GRWR		Great Walton Railroad Company

GWR		Great Western Railway Company

GTR		Great River Railroad

GMRY		Great Miami & Scioto Railway Company

GRMX		Great Miami & Western Railway

GMRC		Green Mountain Railroad Corporation

GRN		Greenville & Northern Railway Company

GUIL		Guilford Transportation Industries, Inc

GRS		Guilford Rail System

GORX		Gulf And Ohio Railways

GCSR		Gulf, Colorado San Saba Railway Corporation

GSSZ		Gulf State Steel, Inc.

GWSW	Gwi Switching Services L.P. (Formerly Genessee & Wyoming Switching Svcs)

HS		H & S Railroad Company, Incorporated

HB		Hampton & Branchville Railroad Company

HBL		Harbor Belt Line Railroad

HSRR		Hardin Southern Railroad, Incorporated

HRT		Hartwell Railroad Company

HVRX		Heber Valley Railroad Utah

HPTD		High Point, Thomasville & Denton Railroad Company

HVSR		Hocking Valley Scenic Railroad

HRS		Hollidaysburg And Roaring Spring Railroad Company

HE		Hollis & Eastern Railroad Company

HCRR		Honey Creek Railroad, Incorporated

HOS		Hoosier Southern Railroad

HRRC		Housatonic Railroad Company, Incorporated

HBT		Houston Belt & Terminal Railway Company

HRR		Huckleberry Railroad

HMCR		Huntsville & Madison County Railroad Authority

HESR		Huron & Eastern Railway

HN		Hutchinson & Northern Railway Company

INPR		Idaho Northern & Pacific Railroad Company

IRYM		Illinois Railway Museum

IMRR		Illinois & Midland Railroad Inc.

IC		Illinois Central Railroad Company

IHB		Indiana Harbor Belt Railroad Company

IHRC		Indiana Hi-rail Corporation

ISRR		Indiana Southern Railroad Company, Incorporated

IN		Indiana Northeastern Railroad Company, Incoporated

INOH		Indiana & Ohio Railroad, Incorporated

ITMZ		Indiana Transportation Museum

INRD		Indiana Rail Road Company

IOCR		Indiana & Ohio Central Railroad, Incorporated

IORY		Indiana & Ohio Railway

IOSX		Indiana & Ohio Scenic Rail Corporation

IRM		Indiana Railway Museum

ISSX		Interchange Specialty Service Inc

IATR		Iowa Traction Railroad Company

IANR		Iowa Northern Railway Company

IAIS		Iowa Interstate Railroad

ISSR		Iss Rail, Inc.

JKL		J. K. Line Incorporated (TIPP)

JGDX		Jackson Gordonville & Delta Railroad

JPA		Jacksonville Port Authority

JXPT		Jaxport Terminal Railway Company

JEFW		Jefferson Warrior Railroad

JJRX		Joliet Junction Railroad

JERX		Joppa & Eastern Railroad Company

KWT		K.W.T. Railway, Incorporated

KLSC		Kalamazoo, Lake Shore & Chicago Rwy (Reinstated 2/14/94)

KRTX		Kanawha River Terminal

KBSR		Kankakee, Beaverville & Southern Railroad Company

KCT		Kansas City Terminal Railway Company

KCS		Kansas City Southern Railway Company

KGEZ		Kansas Gas & Electric

KSW		Kansas Southwestern Railway

KRM		Kentucky Railway Museum

KT		Kentucky & Tennessee Railway

KJRY		Keokuk Junction Railway

KMRY		Kettle Moraine Railway

KRR		Kiamichi Railroad Company LLC

KKRR		Knox & Kane Railroad Company

KYLE		Kyle Railroad Company

LKWR		Lackland Western Railroad Company

LT		Lake Terminal Railroad Company

LWAT		Lake Whatcom Railway

LSMR		Lake Superior & Mississippi Railroad

LSI		Lake Superior & Ishpeming Railroad Company

LSRC		Lake State Railway Company

LSMT		Lake Superior Museum Of Transportation

LSRX		Lake Shore Railway

LVRC		Lamoille Valley Railroad Company

LC		Lancaster & Chester Railway Company

LR		Landisville Railroad                ** Use Code AMHR **

LNO		Laona & Northern Railway

LRS		Laurinburg & Southern Railroad Company

LCSR		Leadville, Colorado & Southern Railroad, Company

LSR		Leelanau Scenic Railroad

LINC		Lewis & Clark Railroad Company

LBCX		Lewisburg & Buffalo Creek Railroad

LXOH		Lexington & Ohio Railroad Company Incorporated

LFIZ		Litchfield Industrial Railroad

LRWN		Little Rock & Western Railway, L.P.

LRR		Little River Railroad

LKRR		Little Kanawha River Rail

LRPA		Little Rock Port Railroad Company

LPSG		Live Oak, Perry, South Georgia Railway Company

LAL		Livonia, Avon & Lakeville Railroad Corporation

LER		Logansport & Eel River Short Line Company, Incorporated

LI		Long Island Rail Road

LHRR		Longhorn Railway Company

LS		Longview Switching

LPN		Longview, Portland & Northern Railway Company

LAJ		Los Angeles Junction Railway Company

LNW		Louisiana & North West Railroad Company

LDRR		Louisiana & Delta Railroad

LA		Louisiana & Arkansas Railway Company

LW		Louisville & Wadley Railway Company

LIRC		Louisville & Indiana Railroad Company

LNAL		Louisville, New Albany & Corydon Railroad

LBR		Lowville & Beaver River Railroad Company

LUN		Ludington & Northern Railway

LSX		Luzerene & Susquehanna Rwy Co.

MGRI		M. G. Rail, Incorporated

CMPA		Madison Railroad Division

MAA		Magma Arizona Railroad Company

MVRY		Mahoning Valley Railroad Company

MEC		Maine Central Railroad Company

MDTT		Maine Department Of Transportation

MC		Maine Coast Railroad Corporation

MCPA		Manattee County Port Authority           ** Use Code MAUP **

MPP		Manitou & Pike's Peak Railroad Company

MJ		Manufacturers' Junction Railway Company

MRS		Manufacturers' Railway Company

MACZ		Maryland Association Of Rail Commuters

MMID		Maryland Midland Railway, Incorporated

MDT		Maryland Dept. Of Transportation

MPAD		Maryland Port Authority

MPA		Maryland & Pennsylvania Railroad Company

MDDE		Maryland & Delaware Railroad Company

MCCL		Mason City & Clear Lake Electric Railway Historical Society

MTAX		Mass Transit Administration

MCER		Massachusetts Central Railroad Corporation

MSTR		Massena Terminal Railroad Company

MCR		Mccloud Railway Company

MKC		Mckeesport Connecting Railroad Company

MCLR		Mclaughlin Line Railroad

MBRR		Meridian & Bigbee Railroad Company

MNCW	Metro North Commuter Railroad Company

MS		Michigan Shore Railroad

MSTP		Michigan State Trust For Railway Preservation

MSO		Michigan Southern Railroad Company, Incorporated

MCRY		Mid-Continent Railway

MRR		Mid-Atlantic Railroad Company, Inc. (Use Code CALA)

MMRR	Mid-Michigan Railroad Company

MIDH		Middletown & Hummelstown Railroad Company

MNJ		Middletown & New Jersey Railway Company, Incorporated

MDLR		Midland Terminal Company

MDRY		Midlands Railway

MWCL	Midwest Coal Handling, Incorporated

MBRX		Milford-Bennington Railroad Company

MRSX		Military Ocean Terminal Railroad At Sunny Point

MNNR		Minnesota Commercial Railway

MNTM	Minnesota Transportation Museum

MDW		Minnesota, Dakota & Western Railway Company

MCTA		Minnesota Central Railroad Company

MZL		Minnesota Zephyr, Limited

MMR		Minnesota & Manitoba Railroad

MSV		Mississippi & Skuna Valley Railroad Company

MSCI		Mississippi Central Railroad Company

MSE		Mississippi Export Railroad Company

MSDR		Mississippi Delta Railroad

MSRW		Mississippian Railway Cooperative, Incorporated

MNA		Missouri & Northern Arkansas Railroad Company, Inc.

MP		Missouri Pacific Railroad Company

MET		Modesto & Empire Traction Company

MHWA	Mohawk, Adirondack & Northern Railroad Corporation

MWRL	Molalla Western Railway Company

MCRR		Monongahela Connecting Railroad Company

MRL		Montana Rail Link

MWRR	Montana Western Railway Company

MRMZ	Monticello Railway Museum

MB		Montpelier And Barre Railroad Company        Former Code MN, Now WACR

ME		Morristown & Erie Railway, Incorporated

MCSA		Moscow, Camden & San Augustine Railroad

MVT		Mount Vernon Terminal Railway, Incorporated

MH		Mount Hood Railway Company

MRSR		Mount Rainier Scenic Railroad

MNL		Mountain Laurel Railroad Company

MWR		Muncie & Western Railroad Company

METW	Municipality Of East Troy

NCMX		N C Transportation Museum

NJT		N. J. Dept. Of Transportation

NVRR		Napa Valley Railroad

NCYR		Nash County Railroad

NERR		Nashville & Eastern Railroad

NRI		Nebkota Railway, Incorporated

NCRC		Nebraska Central Railroad

NNRX		Nevada Northern Railway

NSWX		Neversweat & Washoe Railroad

NEGS		New England Southern Railroad Company, Incorporated

NECR		New England Central Railroad

NYLE		New York & Lake Erie

NEWG		New Georgia Railroad

NYCH		New York Cross Harbor Railroad Terminal Corporation

NHCR		New Hampshire Central Railroad, Incorporated

NHIR		New Hope & Ivyland Railroad Company

NHN		New Hampshire North Coast Railroad

NHRR		New Hope & Ivyland Rail Road

NHVT		New Hampshire & Vermont Railroad Company

NYSW		New York, Susquehanna & Western Railroad Company

NOT		New Orleans Terminal Company

NOLR		New Orleans Lower Coast Railroad, Incorporated

NOPB		New Orleans Public Belt Railroad

NJTR		New Jersey Transit Rail Operations

NSR		Newburgh & South Shore Railroad

NPSX		Newport Star Clipper Dinner Train

NICX		Niles Canyon Railway

NTRY		Nimishillen & Tuscarawas Railway Company

NBER		Nittany & Bald Eagle

NRR		Nobles Rock Railroad (Former Code NRRX)

NPB		Norfolk & Portsmouth Belt Line Railroad Company

NFD		Norfolk, Franklin And Danville Railroad Company

NW		Norfolk & Western Railway Company

NS		Norfolk Southern Corporation

NSHR		North Shore Railroad

NCDX		North County Transit District

NCRR		North Coast Railroad

NSSR		North Shore Scenic Railroad

NCVA		North Carolina & Virginia Railroad Company

NIRC		Northeast Illinois Regional Comuter Rail Corp. (METRA)

NEKM		Northeast Kansas & Missouri

NICD		Northern Indiana Commuter Transportation District

NN		Northern Nevada Railroad Corporation

NOW		Northern Ohio & Western Railway, Ltd.

NCR		Northern Central Railway Co.    (use Code KCS)

NOKL		Northwestern Oklahoma Railroad Company

OTR		Oakland Terminal Railway

OGEE		Ogeechee Railway Company

OHIC		Ohi Rail Corporation

OSRR		Ohio Southern Railroad Company

ORY		Ohio Railway Museum

OHCR		Ohio Central Railroad Company

OHPA		Ohio & Pennsylvania Railroad Company

OCTL		Oil Creek & Titusville Lines

OAR		Old Augusta Railroad Company

OCN		Olde Colony & New Port

OLB		Omaha, Lincoln & Beatrice Railway Company

ONCT		Ontario Central Railroad Corporation

OMID		Ontario Midland Railroad Corporation

OERR		Oregon Eastern Railroad

ORR		Osage Railroad

OTVR		Otter Tail Valley Railroad Company, Incorporated

OUCH		Ouachita Railroad

OHRY		Owego & Harford Railway, Incorporated

PLW		P L & W (former Code PLWX)

PSRM		Pacific SW Railway Museum (San Diego RR Museum)

PARN		Pacific & Arctic Railroad & Navigation Company

PRSX		Pacific Rail Services

PI		Paducah & Illinois Railroad Company

PAL		Paducah & Louisville Railway Company

PARX		Palouse River Railroad Company   **Use Code BLMR**

PNR		Panhandle Northern Railroad Company

PRT		Parr Terminal Railroad

PBR		Patapsco & Back Rivers Railroad Company

PRV		Pearl River Valley Railroad Company

PVS		Pecos Valley Southern Railway Company

POVA		Pend Oreille Valley Railroad, Incorporated

PT		Peninsula Terminal Company

PC		Penn Central Transportation Company

PPHW		Peoria, Peoria Heights & Western Railroad

PPU		Peoria & Pekin Union Railway Company

PBNE		Philadelphia, Bethlehem & New England Railroad

PICK		Pickens Railway Company

PBRR		Pine Belt Southern RR

PVRR		Pioneer Valley Railroad Company, Incorporated

PWJ		Pioneer Railroad Company, Incorporated

PS		Pittsburg & Shawmut Railroad Company

PAM		Pittsburgh, Allegheny & Mckees Rocks Railroad

PCY		Pittsburgh, Chartiers & Youghiogheny Railway

PCDX		Pittsburgh & Conneaut Dock Company

PTC		Plainview Terminal Company (Peoria Terminal Used Code Til 1980)

PCIX		Plant City Industrial Park

PLL		Plymouth & Lincoln

PCN		Point Comfort & Northern Railway Company

PPBD		Port Of Palm Beach Terminal

POCA		Port Of Catoosa

POTB		Port Of Tillamook Bay Railroad

PRI		Port Railroads, Incorporated

PATH		Port Authority Trans Hudson

POBR		Port Of Royal Slope Railroad

POAK		Port Of Oakland

PRYL		Port Royal Railroad

PLCX		Port Of Lake Charles

PBVR		Port Bienville Railroad

MAUP		Port Of Manatee

PUCC		Port Utilities Commission Of Charleston, S.C.

PTRA		Port Terminal Railroad Association

PTR		Port Terminal Railroad Of South Carolina (Former Code PTRS)

PTO		Portland Terminal (Oregon)

PNWR		Portland & Western Railroad, Inc.

PDCZ		Portland Development Commission

PEX		Potomac Eagle Scenic Rail Excursion

PNW		Prescott & Northwestern Railroad Company

PGCX		Proctor & Gamble Cellulose

PW		Providence & Worcester Railroad Company

PSHX		Puget Sound Railway Historical Association

QARX		Queen Anne Railroad

QRR		Quincy Railroad Company

QBT		Quincy Bay Terminal Company

RJCM		R. J. Corman Railroad Company/Memphis Line

RJCW		R. J. Corman, Western Ohio Line

RJCR		R. J. Corman Railroad

RJCP		R. J. Corman Railroad Company/Pennsylvania Lines, Inc.

RJCL		R. J. Corman Railroad Company/Cleveland Line

RSIX		Rail Switching L.L.C.

RTI		Rail Tours, Incorporated

RSM		Railroad Switching Service Of Missouri, Incorporated

RARW		Rarus Railway Company

RERX		Reader Railroad

RBMN		Reading Blue Mountain & Northern Railroad Commision

RRVW		Red River Valley & Western Railroad

RBKR		Red Bank Railroad Company

RSNR		Red Spring & Northern Railroad

RRC		Redmont Railway Company, Inc.

RBX		Ringling Brothers, Barnum & Bailey Circus

RVSC		Rio Valley Switching Company

RT		River Terminal Railway Company

RS		Roberval & Saguenay Railway Company

RRCO		Rochelle Railroad Company

RSR		Rochester Southern Railroad, Incorporated

RSS		Rockdale, Sandow & Southern Railroad Company

RSP		Roscoe, Snyder & Pacific Railway Company

SRN		Sabine River & Northern Railroad Company

SCRT		Sacramento County Regional Transit District

SGVY		Saginaw Valley Railway Company, Incorporated

SL		Salt Lake City Southern Railroad Company, Incorporated

SLGW		Salt Lake, Garfield & Western Railway Company

SMA		San Manuel Arizona Railroad Company

SLC		San Luis Central Railroad Company

SWKR		San Pedro & Southwestern Railway Company

SJVR		San Joaquin Valley Railroad Company

SDNX		San Diego Northern Railway

SDIY		San Diego & Imperial Valley

SFBR		San Francisco Belt Railroad

SDTI		San Diego Trolley Incorporated

SS		Sand Springs Railway Company

SAN		Sandersville Railroad Company

SCBG		Santa Cruz, Big Tree & Pacific Railroad

SMV		Santa Maria Valley Railroad Company

SFS		Santa Fe Southern Railway, Incorporated

SBNX		Saracuse Binghampton And New York Railroad

SSDK		Savannah State Docks Railroad Company

SCRX		Schuylkill Country Rail Authority

SWGR		Seagraves, Whiteface & Lubbock Railroad Company

SVTX		Seaview Transportation Co., Inc.

SGLR		Seminole Gulf Railroad

SE		Semo Port Railroad, Inc.

SQVR		Sequatchie Valley Railroad

SVRR		Shamokin Valley Railroad Company

SVIZ		Shelbyville Industrial Railroad

SV		Shenandoah Valley Railroad

SMSX		Shortline Marketing  Services

SLGG		Sidney & Lowe Railroad Incorporated

SRYC		Sierra Railway Company Of California Limited

SERA		Sierra Railroad Company

SMRR		Sisseton Milbank Railroad

SSL		Skaneateles Short Line RR

SHRX		Smokey Hill Railway

SLRS		SMS Rail Service, Inc.

SOM		Somerset Railroad Company

SOO		Soo Line Railroad Company

SKJA		Soo KCS Joint Agency

SLAL		South Plains Lamesa Railroad, Ltd.

SKOL		South Kansas & Oklahoma Railroad Company

SCRF		South Carolina Central Railroad Company, Incorporated

SCFE		South Central Florida Railroad

SCTR		South Central Tennessee Railroad Corporation

SCXF		South Central Florida Express, Inc.

SBK		South Brooklyn Railway Company

SO		South Orient Railroad Company, Ltd.

SB		South Buffalo Railway Company

SBVR		South Branch Valley Railroad

SEKR		Southeast Kansas Railroad

SEPA		Southeastern Pennsylvania Transportation Authority

SEI		Southeastern International Railroad

SMRS		Southern Michigan Railroad Society

SCAX		Southern California Regional Rail Authority

SUAB		Southern Alabama Railroad Company, Incorporated

SSC		Southern Switching Company

SSWN		Southern Pacific, Chicago-St. Louis Corporation

SERX		Southern Electric Railroad

SOU		Southern Railway Company (NS)

SP		Southern Pacific Transportation Company

SEGX		Southern Electric Generating Company

SIND		Southern Indiana Railway, Incorporated

SIM		Southern Illinois & Missouri Bridge Line

SRNJ		Southern Railroad Company Of New Jersey

SWP		Southwest Pennsylvania Railroad Company

SW		Southwestern Railroad Company, Incorporated

ST		Springfield Terminal Railway Company (Vermont)

SLR		St. Lawrence & Atlantic Railroad Company

SM		St. Marys Railroad Company

SJRT		St. John's River Terminal Company

SSW		St. Louis Southwestern Railway Company

SLCR		St. Louis & Chain Of Rocks Railroad

SLOI		St. Louis, Iron Mountain, & Southern

SLST		St. Louis Steam Train Association

STLH		St. Lawrence & Hudson Railway Company Limited

STMA		St. Maries River Railroad Company

SLRR		St. Lawrence & Raquette River Railroad

SF		Steamtown Foundation     (Use Code SNCX)

SNCX		Steamtown National Historic Site  (Former Code SF)

SH		Steelton & Highspire Railroad

STRT		Stewartstown Railroad Company

STE		Stockton Terminal & Eastern Railroad

SPBR		Stockton Public Belt Railroad

SMPX		Stone Mountain Park

SMRX		Stone Mountain Railroad

SBRR		Stourbridge Railroad Company

SRC		Strasburg Railroad Company

TE		Tacoma Eastern Railway Company

TMBL		Tacoma Municipal Belt Line Railway

TSRR		Tennessee Southern Railroad Company, Incorporated

TVRM		Tennessee Valley Railroad

TENN		Tennessee Railroad Company (NS)

TKEN		Tennken Railroad Company Incorporated

TASD		Terminal Railway Alabama State Docks

TRRA		Terminal Railroad Association Of St. Louis

TCT		Texas City Terminal Railway Company

TELX		Texas Limited (excursion Train, Incorporated)

TXTC		Texas Transportation Company

TXOR		Texas & Oklahoma Railroad Company

TXNW		Texas Northwestern Railway Company

TXGN		Texas, Gonzales & Northern Railway Company

TSR		Texas State Railroad

TM		Texas Mexican Railway Company

TSE		Texas South-Eastern Railroad Company

TOE		Texas, Oklahoma & Eastern Railroad Company

TN		Texas & Northern Railway Company

TNER		Texas Northeastern Division--Mid-Michigan Railroad

TNMR		Texas & New Mexico Railroad Company (Austin & Northwestern Div)

TNOX		Texas North Orient Railroad

TBRY		Thermal Belt Railway

TCRO		Three County Commuter Rail Organ. (Use Code TCCX)

TTR		Tijuana And Tecate Railway Company

TIOC		Tioga Central Railroad

TPW		Toledo, Peoria & Western Railway Corporation

TLEW		Toledo Lake Erie & Western Railroad

TR		Tomahawk Railway, L.P.

TIRL		Tonawanda Island Railroad Incorporated

TSWR		Toppenish Simcoe & Western

TMSS		Towanda Monroeton Shippers Lifeline, Incorporated

TULX		Trains Unlimited

TCSX		Trancisco Rail Services Company

TTIS		Transkentucky Transportation Railroad, Incorporated

TCCX		Tri-County Commuter Rail Authority

TRC		Trona Railway Company

TCG		Tucson, Cornelia & Gila Bend Railroad Company

TVRR		Tulare Valley Railroad Company

TSU		Tulsa-sapulpa Union Railway Company LLC

TCKR		Turtle Creek Industrial Railroad, Incorporated

TSBY		Tuscola & Saginaw Bay Railroad Company, Incorporated

TWIN		Twin State Railroad Corporation

TSRD		Twin State Railroad Company

TCWR		Twin City & Western Railroad

TYBR		Tyburn Railroad Company

UCIR		Union County Industrial Railroad Company

UT		Union Term. Railway Co. "St. Joseph, Mo."

URR		Union Railroad Company (Pittsburgh)

UP		Union Pacific Railroad Company

USA		United States Army

UMP		Upper Merion & Plymouth Railroad Company

UCRY		Utah Central Railway Company

UTAH		Utah Railway Company

VS		V&S Railway, Incorporated

VRCX		Vaaughan Railroad Company

VR		Valdosta Railway, L.P.

VALE		Valley Railroad Company

VRRC		Vandalia Railroad Company

VCY		Ventura County Railway Company

VTR		Vermont Railway, Incorporated

VSRR		Virginia Southern Railroad

VREX		Virginia Railway Express

VTRR		Virginia & Truckee Railroad Company, Incorporated

WOHO	Wabash & Ohio Railroad (Out Of Operation)

WAER		Wabash & Erie Railroad (Out Of Operation)

WCLR		Waccamaw Coast Line Railroad Company

WS		Walkersville Southern Railroad, Incorporated

WHOE		Walking Horse & Eastern Railroad Company

WSR		Warren & Saline River Railroad Company

WTRM	Warren & Trumbull Railroad Company

WCRC		Washington Central Railroad Company

WACR		Washington County Railroad Corporation

WCTR		WCTU Railway Company

WCOR		Wellsboro & Corning Railroad Company

WTSE		West Shore Railroad

WTNN		West Tennessee Railroad Corporation

WTLR		West Texas And Lubbock Railroad Company Incorporated

WVN		West Virginia Northern Railroad, Limited

WJSL		West Jersey Short Line               ** Now Code PWJ **

WJ		West Jersey Railroad Company

WRRC		Western Railroad Company

WKRL		Western Kentucky Railway, LLC

WMSR		Western Maryland Scenic Railroad

WPSX		Western Plant Services, Incorporated (Ca)

WRM		Western Railway Museum

WCTX		Weyerhaeuder Woods Railroads

WTCX		Weyerheuser Company

WE		Wheeling & Lake Erie Railway Company

WCTU		White City Terminal & Utility Company   Use Code WCTR

WVRR		Whitewater Valley Railroad

WTA		Wichita Terminal Asosciation

WTJR		Wichita, Tillman & Jackson Railway Company, Incorporated

WBC		Wilkes-Barre Connecting Railroad Company

WPRR		Willamette & Pacific Railroad, Inc.

WVR		Willamette Valley Railway Company

WVRD		Willamette Valley Railroad       Use Code WVR

WGRR		Willamette Valley Railroad       Use Code WVR

WGR		Willamina & Grand Ronde Railway Company

WWRC	Wilmington & Western Railroad Company    (Former Code WAW)

WTRY		Wilmington Terminal Railroad Incorporated

WSRY		Winamac Southern Railway Company

WW		Winchester & Western Railroad Company

WSS		Winston-Salem Southbound Railway

WGCR		Wiregrass Central Railroad Company

WICT		Wisconsin Calumet Railroad

WSOR		Wisconsin & Southern Railroad Company

WC		Wisconsin Central Ltd. (also Railway)

WYCO		Wyoming & Colorado Railroad

YVRR		Yadkin Valley Railroad Company

YVRX		Yakima Valley Rail & Steam Museum

YSLR		Yolo Shortline  Railroad Company

YKR		Yorkrail, Incorporated

YARR		Youngstown & Austintown Railroad

YW		Yreka Western Railroad Company