Appendix DD.1

Feeding My Baby Study

ID label

Home health care Agency form – Length/Weight - ENGLISH

Date:  _____/_____/20____

Measurements

Clothing:  Child should wear only diapers

- Recumbent length (use length board with fixed headpiece and adjustable footboard)

  - Use NHANES standard procedures

  - If unable to capture acceptable length, reposition child and re-measure

│__│__│.│__│cm.

- Weight (use portable calibrated scale)

  - Use NHANES standard procedures

  - If unable to capture acceptable weight, reposition child and re-measure

│__│__│.│__│kg.

Concerns/Comments about measurements:

____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Standard NHANES procedures

Recumbent length (use length board with fixed headpiece and adjustable footboard)

- Cover length board with paper

- Lay child on length board with head towards headpiece in Frankfort horizontal plane

- Have caretaker apply gentle traction to bring the top of head in contact with fixed headpiece and secure head by lightly cupping palms of hands over ears

- Align legs by placing one hand gently but with mild pressure over knees

- With other hand slide foot piece to rest firmly at child’s heels

- Toes should point directly upward with both soles of feet flexed perpendicular against footboard.

- To encourage child to flex feet, run tip of finger down inside of foot

- Read measurement on length board that corresponds with the foot piece

- If unable to capture acceptable length, reposition child and remeasure

## Length Boards for Measuring Infants

A recumbent measuring board should have a firm, flat base with an attached measuring tape, a fixed perpendicular headpiece and a smoothly sliding foot piece constructed of a durable material like wood or plexiglass that is easy to clean. The foot piece must form a 90º angle with the measurement surface. All edges must be smooth and finished. Measurements should be readable to the nearest 1/8 inch. The board should be placed on a secure table with enough room for the Assistant to be able to stand behind the back of the head of the child.

Weight (use portable calibrated scale)

  - Infants – standard scale

    - If use standard scale, weigh caretaker first

    - Tare scale

    - Hand caretaker the child

    - Capture weight of child

  - Infants – infant scale

    - Cover scale with paper

    - Place child on back or sitting, in center of tray

    - Capture weight of child

  - Toddlers

    - Have child stand on center of scale with hands by side

    - Record weight

## Scales for Weighing Infants

Infant beam balance or digital scales are appropriate to use for weighing infants. Beam balance scales should be marked in increments of 1 ounce or less. They should have a screw-type zeroing adjustment. There should be two sliding weights on a beam balance scale, one for whole pound increments and a second for ounces. Digital scales should read to one ounce or less. Both types should have features that allow for calibration.