[Federal Register Volume 87, Number 218 (Monday, November 14, 2022)]

[Rules and Regulations]

[Pages 68270-68310]

From the Federal Register Online via the Government Publishing Office [www.gpo.gov]

[FR Doc No: 2022-23666]

[[Page 68269]]

Vol. 87

Monday,

No. 218

November 14, 2022

Part III

Department of Homeland Security

-----------------------------------------------------------------------

Coast Guard

-----------------------------------------------------------------------

46 Parts 121, 160, et al.

Survival Craft Equipment--Update to Type Approval Requirements; Final

Rule

Federal Register / Vol. 87 , No. 218 / Monday, November 14, 2022 /

Rules and Regulations

[[Page 68270]]

-----------------------------------------------------------------------

DEPARTMENT OF HOMELAND SECURITY

Coast Guard

46 CFR Parts 121, 160, 169, 184, and 199

[Docket No. USCG-2020-0107]

RIN 1625-AC51

Survival Craft Equipment--Update to Type Approval Requirements

AGENCY: Coast Guard, Department of Homeland Security (DHS).

ACTION: Final rule.

-----------------------------------------------------------------------

SUMMARY: The Coast Guard is updating the type approval requirements for

certain types of equipment that survival craft are required to carry on

U.S.-flagged vessels. This rule will remove Coast Guard type approval

requirements for nine of these types of survival craft equipment and

replace them with the requirement that the manufacturer self-certify

that the equipment complies with a consensus standard.

DATES: This final rule is effective December 14, 2022.

The incorporation by reference of certain publications listed in

the rule is approved by the Director of the Federal Register on

December 14, 2022. The incorporation by reference of certain other

publications listed in the rule were approved by the Director of the

Federal Register on October 1, 1996.

ADDRESSES: To view documents mentioned in this preamble as being

available in the docket, go to https://www.regulations.gov, type USCG-

2020-0107 in the search box and click ``Search.'' Next, in the Document

Type column, select ``Supporting & Related Material.''

FOR FURTHER INFORMATION CONTACT: For information about this document,

call or email Ms. Stephanie Groleau, Lifesaving & Fire Safety Division

(CG-ENG-4), Coast Guard; telephone 202-372-1381, email

Stephanie.M.Groleau@uscg.mil.

SUPPLEMENTARY INFORMATION:

Table of Contents for Preamble

I. Abbreviations

II. Basis, Purpose, and Regulatory History

III. Background

IV. Discussion of Comments

V. Discussion of Final Rule and Changes From NPRM

VI. Incorporation by Reference

VII. Regulatory Analyses

A. Regulatory Planning and Review

B. Small Entities

C. Assistance for Small Entities

D. Collection of Information

E. Federalism

F. Unfunded Mandates

G. Taking of Private Property

H. Civil Justice Reform

I. Protection of Children

J. Indian Tribal Governments

K. Energy Effects

L. Technical Standards and Incorporation by Reference

M. Environment

I. Abbreviations

ASTM ASTM, International

BLS U.S. Bureau of Labor Statistics

CFR Code of Federal Regulations

CG-ENG-4 Office of Design and Engineering Standards, Lifesaving &

Fire Safety Division

CGMIX U.S. Coast Guard Maritime Information Exchange

COA Certificate of approval

DHS Department of Homeland Security

ECEC Employer Costs for Employee Compensation

FDA U.S. Food and Drug Administration

FR Federal Register

IBA Inflatable buoyant apparatus

IBC Code International Code for the Construction and Equipment of

Ships Carrying Dangerous Chemicals in Bulk

IGC Code Amendments to the International Code for the Construction

and Equipment of Ships Carrying Liquefied Gases in Bulk

ICR Information collection request

IMO International Maritime Organization

ISO International Organization for Standardization

LSA Code Life-Saving Appliances Code

MISLE Marine Information for Safety and Law Enforcement

NAICS North American Industry Classification System

NPRM Notice of proposed rulemaking

OES U.S. Bureau of Labor Statistics Occupational Employment

Statistics

OMB Office of Management and Budget

OPM Office of Personnel Management

OTC Over-the-counter

RA Regulatory analysis

SOLAS International Convention for the Safety of Life at Sea

Sec.  Section

U.S.C. United States Code

II. Basis, Purpose, and Regulatory History

The legal authority for this rule is found in Title 46 of the

United States Code (U.S.C.) Sections 2103, 3103, 3306, 3703, 4102,

4302, 4502, 7101, and 8101. The Secretary of the Department of Homeland

Security (DHS) has delegated these statutory authorities to the Coast

Guard pursuant to 14 U.S.C. 502 through DHS Delegation No. 00170.1,

Revision No. 01.2, paragraph (II)(92)(a), (b), (e), and (f).

Additionally, 14 U.S.C. 102(3) grants the Coast Guard broad authority

to promulgate and enforce regulations for the promotion of safety of

life and property on waters subject to the jurisdiction of the United

States.

The purpose of this rule is to update the type approval

requirements for 12 types of survival craft equipment that survival

craft are required to carry on certain, specified U.S.-flagged

vessels--bilge pumps, compasses, fire extinguishers, first-aid kits,

fishing kits, hatchets, jackknives, knives, signaling mirrors,

provisions (food rations), emergency drinking water, and sea anchors--

as well as some of the survival craft equipment required for sailing

school vessels. For nine of these types of equipment, this rule will

replace the Coast Guard type approval requirement with a requirement

that the manufacturer self-certify that the equipment complies with a

consensus standard: bilge pumps, compasses, first-aid kits, fishing

kits, hatchets, jackknives, mirrors, sea anchors, and water. Type

approval is the primary process for equipment and materials to receive

Coast Guard approval. Updating type approval requirements for survival

craft equipment will result in cost savings to equipment manufacturers,

vessel owners and operators, and the Coast Guard.

The Coast Guard issued a notice of proposed rulemaking (NPRM) on

October 5, 2020, and solicited public comment on the proposal during a

comment period of 60 days.\1\ The comment period closed on December 4,

2020. The Coast Guard received 13 comment submissions, which are

discussed later in this document.

---------------------------------------------------------------------------

\1\ 85 FR 62842.

---------------------------------------------------------------------------

III. Background

Many of the current requirements for survival craft equipment were

developed in the 1950s and 1960s and have not been significantly

updated since they were published. After thorough review of these

requirements, as well as Coast Guard enforcement procedures, current

maritime industry practice, and the availability of new consensus

standards, we believe that the additional scrutiny provided by Coast

Guard type approval does not increase the safety of the following nine

types of survival craft equipment: bilge pumps, compasses, first-aid

kits,\2\ fishing kits, hatchets, knives (including jackknives),

mirrors, sea anchors, and emergency drinking water.

---------------------------------------------------------------------------

\2\ Different first-aid kits are required for different survival

craft, and this is explained in section IV of this rule under First-

Aid Kits.

---------------------------------------------------------------------------

For these types of equipment, the current Coast Guard type approval

requirements are outdated and overly prescriptive. This places a burden

on the equipment manufacturers, which, in turn, affects the design

costs of complying with the outdated standard, the administrative

overhead costs, and the time-to-market costs of manufacturing and

selling equipment.

[[Page 68271]]

The requirements also place a financial burden on the vessel owners and

operators who are required to carry this specific approved equipment on

board their survival craft. This equipment is frequently more costly

and more difficult to obtain than similar products that are not type-

approved. Finally, the requirements place a burden on the Coast Guard

to review and approve this equipment without commensurate increases in

safety.

IV. Discussion of Comments

The Coast Guard received 13 comment submissions in response to the

NPRM. Of those 13 comments, 1 was a duplicate and 1 was unrelated to

the rulemaking. The remaining 11 comments were from maritime

organizations, private companies, and individuals. Four comments we

classified as general comments, two comments concerned technical

standards, and five comments concerned first-aid kits. Below, we

discuss each comment and our responses.

General

The Coast Guard received four comments on the NPRM that we

categorized as general comments. One comment supported the proposed

regulatory changes for approval requirements for first-aid kits. The

Coast Guard acknowledges this comment.

Two commenters expressed concerns that removing type approval

requirements could decrease the quality of survival craft equipment. We

disagree. Even without a type approval requirement, the following

checks will remain in place. For emergency drinking water in survival

craft and rescue boats, the water quality will be verified by the local

municipality or by an independent laboratory accepted by the Coast

Guard, as required by 46 CFR 199.175(b)(40). Coast Guard-approved

liferaft servicing facilities inspect survival equipment packed in

inflatable liferafts prior to packing. Coast Guard marine inspectors

also regularly check equipment not packed in inflatable liferafts, such

as that in a lifeboat or rescue boat, or the first-aid kits carried on

small passenger vessels, when conducting the required inspections on

board commercial vessels.

Additionally, one commenter, a manufacturer of the approved Coast

Guard items, expressed multiple concerns regarding this rule and the

Coast Guard's regulatory analysis on its estimate of the impacts in the

NPRM. This commenter said that removing type approval requirements will

cause the market to be flooded with substandard products, leading to

revenue losses to the company. The commenter also said that the

liferaft and lifeboat industry has consolidated and there is little

competition, and, therefore, will not pass savings on to consumers.

For the reasons explained in our response to the two commenters

above, we do not expect reduced quality in the equipment that is no

longer required to be type-approved. We therefore do not expect a flood

of products of reduced quality that drive down prices. With this final

rule, prescriptive requirements will be replaced by consensus

standards. Conforming to these international consensus standards will

maintain the same level of safety without imposing unnecessary burdens

on the public and provide alternatives for compliance. These compliance

alternatives should result in cost savings to the directly impacted

entities, which are manufacturers and vessel owners and operators. The

Coast Guard does not have adequate industry information or data to

estimate secondary impacts and indicate whether these savings will be

passed on to the final consumers or end users of services provided by

vessel owners and operators.

The commenter also suggested that some could incur additional

testing costs as a result of this rule. Based on a review of the new

and existing standards, the Coast Guard has not found that

manufacturing firms will have new testing requirements under the

International Organization for Standardization (ISO) standards.

The commenter suggested that, as an alternative to the removal of

type approval requirements, the manufacturer could cover the cost of

the certificate of approval (COA). Requiring manufacturers to cover the

cost of the COA would result in additional costs to manufacturers

without any attendant safety benefits.

Finally, the commenter asserted that our per-device savings

estimates are too high and not the going rates in the industry. In

preparing our economic analysis, we relied primarily on websites

listing the retail prices of different products that were sold under

ISO standards instead of Coast Guard standards. We believe that the

reason our prices appear to be high to the commenter is because our

analysis was based on retail prices rather than wholesale prices, or

the prices that manufacturers use to sell their products to businesses.

Using retail prices is a common approach across Coast Guard rulemaking,

because we do not have access to consistent wholesale price data across

the industry.

F1003 and F1014 Standards

The Coast Guard received two comments recommending incorporation of

ASTM F1003 (2019), ``Standard Specification for Searchlights on Motor

Lifeboats,'' and ASTM F1014 (2020), ``Standard Specification for

Flashlights on Vessels.'' These 2019 and 2020 standards are more recent

editions of the ASTM standards we proposed to adopt.

However, these standards were updated after the NPRM was developed,

and so we were unable to include them in our proposed rule. The more

recent standards contain significant differences as compared to the

prior editions (the ones we incorporate in this rule), such that more

evaluation is necessary. We will consider incorporating these standards

in a future rulemaking.

First-Aid Kits

The Coast Guard received five comments concerning the proposed

changes to first-aid kits. The comments discussed contents of the

first-aid kits, as well as technical standards that apply to first-aid

kits.

Two commenters supported the proposed use of commercially available

first-aid kits, to remove the burden of assembling very specific kit

components.

Three commenters called for specified first-aid kit components,

rather than leaving the exact number and size of items up to

manufacturers so long as the kit meets ISO 18813:2006. These commenters

said the kit contents should be standardized, and expressed concern

that manufacturers would not provide adequate kits. One commenter also

said that ISO 18813:2006 is not a widely accepted standard and may soon

be revised; that commenter suggested the Coast Guard should develop its

own standard instead. Another commenter supported the use of the ISO

standard. We believe that the contents described in ISO 18813:2006 are

sufficient to meet the needs of basic first-aid kits required by

mariners in a survival situation. The ISO standard specifies design,

performance, and use of various items of survival equipment carried in

survival craft and rescue boats complying with the International

Convention for the Safety of Life at Sea (SOLAS), 1974 (as amended),

and the International Maritime Organization (IMO) Life-Saving Appliance

Code (LSA Code). The 2006 edition is the most current version of this

standard that is available at this time.

During periodic shipboard inspections by both Coast Guard-licensed

mariners and Coast Guard

[[Page 68272]]

marine inspectors, first-aid kits not packed in inflatable liferafts

are examined to ensure that they contain all the items listed in the

provided instructions, that each unit carton is in an intact waterproof

package, and that they meet the applicable regulatory requirements.

First-aid kits packed in inflatable liferafts are inspected by Coast

Guard-approved liferaft servicing facilities, also to ensure that they

contain all the required items.

One commenter specifically called for a particular Coast Guard-

approved watertight soft plastic pouch to contain the first-aid kit,

because rigid plastic containers can become brittle and because that

pouch is proven to meet the applicable durability requirements. ISO

18813:2006 discourages the use of rigid plastic cases that can shatter.

If the case shatters, an entirely new kit must be purchased because it

is in a not-as-approved condition, and Coast Guard inspectors would

give the vessel a deficiency for not having an approved and in-working-

condition piece of equipment. This would increase costs to the vessel.

One commenter noted that the U.S. Food and Drug Administration

(FDA) does not routinely approve over-the-counter (OTC) products; it

only reviews active ingredients. Another comment inquired about the FDA

regulatory status, product form, or type of delivery for two topical

preparations in the ISO 18813 requirements.

It is up to the first-aid kit manufacturer to determine in what

form the medicinal products are to be provided to meet the intended

needs of the first-aid kit. However, medicinal products must meet the

applicable OTC drug requirements outlined in title 21 of the Code of

Federal Regulations (CFR) part 330, which contains FDA's applicable OTC

requirements. In response to these comments, in this rule we revised

the regulatory text of Sec.  199.175(b)(10)(ii) to reference 21 CFR

part 330.

One commenter asked that the Coast Guard remove the requirement for

specific items with an expiration date (such as aspirin) and allow for

equivalent alternatives. The commenter said that getting supplies

delivered to remote locations can be challenging. The expiration date

of OTC medications is typically between one and five years after

manufacture. The commenter did not specify an alternative item without

an expiration date, but the Coast Guard believes that a year or more is

a reasonable period to plan for replacing first-aid supplies. In

general, the Coast Guard believes that expiration dates are acceptable

and can help ensure that the first-aid kit is reviewed and refreshed at

intervals. The Food and Drug Administration requires OTC medications

have expiration dates (see 21 CFR 211.137 and 211.166).

The same commenter recommended that vessel operators be allowed to

exclude analgesics (pain relief medication) from first-aid kits. This

commenter said that companies often prohibit their vessel crew members

from giving out analgesic medication because of possible adverse side

effects or interactions with other medication. In support of this

recommendation, the commenter said that most passenger vessels operate

near shore with easy access to shoreside medical services.

While access to shoreside medical resources may be available in

certain areas of operation, these should not be relied on to provide

the required first-aid supplies. Shoreside medical resources will not

be readily available to someone with an injury or emergency on the

vessel. The first-aid kit for survival craft is intended to be used in

an emergency away from shore.

Licensed mariners operating vessels in commercial service are

required to have basic first-aid training. Any application of first aid

should be given at the discretion of the licensed mariner and not at a

level beyond the training or capability of the mariner administering

the first aid. Analgesics are common OTC medications that do not

require medical supervision, and the decision to take them is up to the

person who requests them. Accordingly, the Coast Guard has decided to

retain the requirement for analgesics in first-aid kits.

V. Discussion of Final Rule and Changes From NPRM

This final rule amends several approval and carriage requirements

in title 46 CFR. Specifically, this final rule updates the requirements

in part 199, subchapter W, related to the equipment on survival craft

and rescue boats on inspected vessels by replacing the requirement to

carry Coast Guard-approved equipment with self-certification to

voluntary consensus standards for certain equipment. This rule also

makes conforming changes to part 169, subchapter R, for sailing school

vessels that are not covered by subchapter W. In addition, this final

rule revises part 160, subchapter Q, to remove approval standards for

the survival craft equipment that is no longer required to be approved

by the Coast Guard, and it updates the requirements for approval of

emergency provisions to replace prescriptive Coast Guard requirements

with consensus standards. A new subpart 160.046, Emergency Provisions,

is added, to consolidate the applicable standards. Finally, this rule

removes the requirement in part 121, subchapter K, and part 184,

subchapter T, that first-aid kits carried on small passenger vessels

must be approved by the Coast Guard, and updates those requirements to

consensus standards to align with the revised approval requirements.

This final rule includes incorporation by reference of several

voluntary consensus standards consistent with the National Technology

Transfer and Advancement Act of 1995, Public Law 104-113 (codified as a

note to 15 U.S.C. 272). Three of the consensus standards this rule

incorporates are international standards: ISO 18813:2006, ``Ships and

marine technology--Survival equipment for survival craft and rescue

boats'' (referred to as ISO 18813); ISO 17339:2018, ``Ships and marine

technology--Sea anchors for survival craft and rescue boats'' (referred

to as ISO 17339); and ISO 25862:2009, ``Ships and marine technology--

Marine magnetic compasses, binnacles and azimuth reading devices''

(referred to as ISO 25862).

While the IMO does specify some standards for survival craft

equipment affected by this rule, it does not stipulate that the

affected survival craft equipment be approved by the Administration. In

some cases (such as first-aid kits and drinking water), the LSA Code

references ISO 18813 as an acceptable standard for the equipment to

meet, whereas in others (such as fishing tackle), the LSA Code merely

requires that the equipment be carried aboard the specified survival

craft.

A more detailed explanation of the amendments to the aforementioned

sections can be found in the NPRM. A number of non-substantive changes

from the NPRM are made with this final rule to correct typographical,

grammar, and format errors or issues, as well as for clarification

purposes.

Lastly, as a result of public comment, this final rule requires

that medicinal products meet the applicable OTC drug requirements as

outlined in 21 CFR part 330. This administrative change is simply

updating an improper reference.

VI. Incorporation by Reference

Material incorporated by reference is currently listed in 46 CFR

199.05 and is added to the new Sec.  160.046-3. Under 5 U.S.C. 552(a)

and 1 CFR part 51, a publication is eligible for incorporation by

reference if it meets Office of the Federal Register policies and is

reasonably available to and usable by the class of persons affected.

[[Page 68273]]

Regulations in part 51 require that agencies discuss, in the final

rule, ways that the materials the agency incorporates by reference are

reasonably available, to interested parties and how interested parties

can obtain the materials. In addition, the preamble to the final rule

must summarize the material.

In accordance with the OFR's requirements, section VII.L. of this

final rule summarizes the standards that the Coast Guard incorporates

by reference in Sec. Sec.  160.046-3 and 199.05. Interested persons

have access to this material through their normal course of business,

may purchase it from the organization, or may view a copy at Coast

Guard Headquarters.

VII. Regulatory Analyses

We developed this rule after considering numerous statutes and

Executive orders related to rulemaking. Below, we summarize our

analyses based on these statutes or Executive orders.

A. Regulatory Planning and Review

Executive Orders 12866 (Regulatory Planning and Review) and 13563

(Improving Regulation and Regulatory Review) direct agencies to assess

the costs and benefits of available regulatory alternatives and, if

regulation is necessary, to select regulatory approaches that maximize

net benefits (including potential economic, environmental, public

health and safety effects, distributive impacts, and equity). Executive

Order 13563 emphasizes the importance of quantifying both costs and

benefits, of reducing costs, of harmonizing rules, and of promoting

flexibility.

The Office of Management and Budget (OMB) has not designated this

rule a significant regulatory action under section 3(f) of Executive

Order 12866. Accordingly, OMB has not reviewed it. A regulatory

analysis (RA) follows.

The Coast Guard received several public comments on the NPRM, as

discussed in section IV. of the preamble to this final rule. In

response to a comment, in this final rule we are making an editorial

change to 46 CFR 199.175(b)(10) that has no cost impact. See table 1.

Additionally, we are replacing prescriptive requirements with

international standards that provide alternatives for compliance, which

should result in cost savings to impacted entities. We also made some

changes to the regulatory analysis, including updating the population

of affected entities, and the wage rate using 2020 estimates, and

removing the renewal instruction, because it is not applicable to this

rule.

Table 1--Changes From NPRM to Final Rule

----------------------------------------------------------------------------------------------------------------

Section                  Description of change         Explanation         Cost impact of change

----------------------------------------------------------------------------------------------------------------

Sec.   199.175(b)(10)................  Editorial change that    Update the language to   No impact because it is

corrects a reference.    correctly discuss the    updating an improper

FDA's drug approval      reference.

process.

----------------------------------------------------------------------------------------------------------------

With this final rule, the Coast Guard removes the requirement for

nine types of survival craft equipment to be approved by the Coast

Guard from 46 CFR part 160 in subchapter Q (Equipment, Construction,

and Materials: Specifications and Approval) and from Sec.  199.175

(Survival Craft and Rescue Boat Equipment). The requirement for

approvals on these nine types of equipment (bilge pumps, compasses,

first-aid kits, fishing kits, hatchets, jackknives, mirrors, sea

anchors, and water) will be replaced by a self-certification

requirement, in order to comply with the LSA Code. For those types of

equipment that still require a COA, we do not estimate any changes in

costs or cost savings.\3\ Finally, this rule updates the survival craft

requirements for sailing school vessels found in Sec. Sec.  169.525

through 169.529, eliminating the unique requirements for survival craft

equipment on these vessels.

---------------------------------------------------------------------------

\3\ Knives are not required to be Coast Guard-approved; however,

they must meet the requirements in Section 4.1.5.1.2 of the LSA

Code. This is an administrative change that will lead to no cost or

cost savings.

---------------------------------------------------------------------------

Table 2 provides a summary of the affected population, costs, cost

savings, and benefits of this rule. The affected population includes

the manufacturers of survival craft equipment and the vessels equipped

with survival craft. We estimate the cost savings to manufacturers by

reducing reporting, recordkeeping, and production requirements of this

survival craft equipment. We estimate the cost savings to vessel owners

and operators by the price reductions in survival craft equipment, and

we estimate the cost savings for the Government for reducing the review

necessary for certain equipment. We estimate an annualized cost savings

to industry of $303,805 (with a 7-percent discount rate) and an

annualized cost savings to the Government of $10,087, for a total

annualized cost savings of $313,892.\4\

---------------------------------------------------------------------------

\4\ This analysis assumes the implementation year for this rule

will be 2021.

Table 2--Summary of the Affected Population, Costs, Cost Savings, and

Benefits

------------------------------------------------------------------------

Category                             Summary

------------------------------------------------------------------------

Applicability................  Revises the approval requirements

specific to nine types of survival craft

equipment by removing the Coast Guard

type approval requirements and, instead,

adopting a voluntary consensus standard,

ISO 18813, ``Ships and marine

technology--Survival equipment for

survival craft and rescue boats.'' Also

retains requirements for Coast Guard

approval of emergency provisions, but

revises the requirements to refer to ISO

18813 instead of prescriptive Coast

Guard regulations.

Affected Population..........  Includes 16 manufacturers of 28 unique

Coast Guard-approved products for 9

types of equipment; 14,747 existing U.S.-

flagged vessels with 31,729 survival

craft; and 113 new U.S.-flagged vessels

annually with 449 survival craft.

Costs........................  There will be no costs to industry or the

Federal Government as this rule will

reduce the burden(s).

Benefits.....................  There are non-monetary benefits to owners

and operators of vessels with survival

craft in having a larger selection of

equipment to choose from, allowing for

potential operational flexibility.

Industry Cost Savings *......  Annualized: -$303,805, 10-Year: -$2.13

million.

[[Page 68274]]

Government Cost Savings......  Annualized: -$10,087, 10-Year: -$70,847.

Total Cost Savings...........  Annualized: -$313,892, 10-Year: -$2.20

million.

------------------------------------------------------------------------

* The Industry Cost Savings, Government Cost Savings, and Total Cost

Savings are all discounted at 7 percent.

Affected Population

This rule impacts three separate affected populations. First, this

rule impacts manufacturers of Coast Guard-approved equipment because it

changes the standards and approval process for nine types of survival

craft equipment. Second, this rule impacts any new and existing U.S.-

flagged vessels that carry survival craft because it will reduce the

cost of buying and replacing survival craft equipment. Third, this rule

impacts small passenger vessels inspected under subchapter K or T. They

are required to maintain a separate first-aid kit stowed on board, and

this rule reduces the cost of replacing first-aid kits. This rule also

removes Table 169.527 from part 169 and removes the requirements for

equipment outlined in Sec.  169.529(a) through (mm) to conform to the

changes made in 46 CFR part 199.

Data on manufacturers comes from the U.S. Coast Guard Maritime

Information Exchange (CGMIX),\5\ which is a public-facing version of

the Marine Information for Safety and Law Enforcement (MISLE) database,

unless otherwise specified. For each subchapter of inspected vessels

that are required to carry survival craft, we used the MISLE database

to estimate the number of vessels that will be affected by this rule.

---------------------------------------------------------------------------

\5\ https://cgmix.uscg.mil/.

---------------------------------------------------------------------------

Manufacturers of Coast Guard Approved Equipment

The Coast Guard is eliminating approval requirements for nine types

of survival craft equipment, discussed in detail in section V of this

rule. These nine types of equipment include: (1) bilge pumps, (2)

compasses, (3) first-aid kits for lifeboats and for liferafts, (4)

fishing kits, (5) hatchets, (6) jackknives, (7) signaling mirrors, (8)

sea anchors, and (9) emergency drinking water. For these 9 types of

survival equipment, there are 28 unique Coast Guard type-approved

products.\6\ This rule impacts manufacturers of products currently on

the market as well as newly approved products. Currently approved

products in use on survival craft will remain acceptable for the

purpose of carriage after this rule's implementation.

---------------------------------------------------------------------------

\6\ Type Approval is the primary process for equipment and

materials to receive Coast Guard approval. The certificate is valid

for 5 years, and the approval is listed on the CGMIX.

---------------------------------------------------------------------------

The 2019 information collection request (ICR) ``Supporting

Statement for Title 46 CFR Subchapter Q: Lifesaving, Electrical,

Engineering and Navigation Equipment, Construction and Materials &

Marine Sanitation Devices (33 CFR part 159)'' (OMB Control Number:

1625-0035) estimates that companies will seek Coast Guard approval for

3 percent of the number of survival craft equipment product types on

the market each year. The Coast Guard estimates that each new product

approval replaces a preexisting product approval, such that the total

number of approved products will not change each year, as the number of

newly approved products has historically been small.

Table 3 presents the annual average of new products each year for

the nine types of survival craft equipment. To calculate the annual

average of new products, we multiplied the values in the ``Number of

Approved Products'' column (a), which contains the number of existing

approved products for each type of survival craft equipment, by 3

percent, from the ``Percentage of New Approvals Each Year'' column,

(b).

Table 3--Number of Products Currently Approved by the Coast Guard

----------------------------------------------------------------------------------------------------------------

Annual

Number of    Percentage  of      average

Equipment                        Approval        approved     new  approvals   number of new

series        products *      each year **   products each

year

(a)             (b)     (c) = (a) x

(b)

----------------------------------------------------------------------------------------------------------------

Bilge pump......................................         160.044               3               3            0.09

Compass.........................................         160.014               3               3            0.09

First-aid kit for Lifeboats.....................         160.041               5               3            0.15

First-aid kit for Liferafts.....................         160.054               5               3            0.15

Fishing kit.....................................         160.061               1               3            0.03

Hatchet.........................................         160.013               1               3            0.03

Jackknife.......................................         160.043               1               3            0.03

Mirror, Signalling..............................         160.020               2               3            0.06

Sea anchor......................................         160.019               1               3            0.03

Water...........................................         160.026               6               3            0.18

---------------------------------------------------------------

Total.......................................  ..............              28  ..............               1

----------------------------------------------------------------------------------------------------------------

Sources:

* CGMIX data pull, March 2021.

** ``Supporting Statement for Title 46 CFR Subchapter Q: Lifesaving, Electrical, Engineering and Navigation

Equipment, Construction and Materials & Marine Sanitation Devices (33 CFR 159)'' (OMB Control Number: 1625-

0035).

Note: Values may not sum due to rounding.

[[Page 68275]]

U.S.-Flagged Vessels That Carry Coast Guard-Approved Equipment

This rule impacts a total of 14,747 existing vessels. These

vessels, which are categorized by subchapter, are required to carry

survival craft in accordance with the applicable regulations. Of these

vessels, we estimate the total amount of survival craft maintained by

the affected population to be 31,729. Table 4 shows the breakdown of

the survival craft across the existing vessel population as follows:

2,612 inflatable buoyant apparatuses (IBAs), 23,748 liferafts, 2,835

lifeboats, and 2,534 rescue boats.

Table 4--Vessel and Survival Craft Population

--------------------------------------------------------------------------------------------------------------------------------------------------------

IBAs         Inflatable       Lifeboats     Rescue boats    All survival

Total number  ----------------    liferafts   --------------------------------      craft

Subchapter                 Type of vessel     of vessels (a)                 ----------------                                ---------------

Total (b)       Total (c)       Total (d)       Total (e)       Total (f)

--------------------------------------------------------------------------------------------------------------------------------------------------------

C.................................  Commercial Fishing             6,022             248           6,267             141              52           6,708

Vessels.

C.................................  Uninspected                      173              10             258               2               7             277

Passenger Vessels.

D.................................  Tank................             323               3             706             543              49           1,301

H.................................  Passenger...........             191             640             444              91             286           1,461

I.................................  Cargo...............           1,037               3           3,247           1,200             618           5,068

I-A...............................  Mobile Offshore                   57               0             263             623              37             923

Drilling Units.

K.................................  Small Passenger.....             311             512             950               2             164           1,628

L.................................  Offshore Supply                  338               0           1,393              55             322           1,770

Vessels.

M.................................  Towing Vessels......           1,434              91           1,485               2              51           1,629

R.................................  Nautical Schools....              29               2             140              79              22             243

R.................................  Sailing Schools.....              10               0              24               1               7              32

T.................................  Small Passenger.....           4,231           1,025           7,506               5             830           9,366

U.................................  Oceanographic                     74               3             260              53              36             352

Research.

Other Vessels.....................  ....................             517              75             805              38              53             971

-----------------------------------------------------------------------------------------------

Total.........................  14,747..............           2,612          23,748           2,835           2,534          31,729

--------------------------------------------------------------------------------------------------------------------------------------------------------

Table 5 presents vessels by the subchapter to which they are

inspected in 46 CFR. ``Other vessels'' includes public and recreational

vessels not subject to inspection. The owners and operators of the

14,747 identified vessels will experience cost savings from the lower

estimated cost of replacing equipment. We used this existing vessel

population data from MISLE and multiplied it by the average number of

IBAs, liferafts, lifeboats, and rescue boats per vessel, which we also

retrieved from MISLE, to obtain our estimated survival craft

population. The estimated survival craft population is the number of

survival craft that will need to replace non-durable Coast Guard-

approved equipment over the next 10 years. The replacement equipment

will be less expensive, because the replacement equipment will not need

Coast Guard approval. Those vessels with previously approved survival

craft equipment will not be required to replace their survival craft

equipment until the equipment expires or becomes unserviceable.

After establishing the existing number of current survival craft,

we then estimated the growth in the number of survival craft each year

in order to project our affected population for the next 10 years. To

calculate the number of new survival craft each year, we multiplied the

``Number of New Vessels per Year'' by each ``Average per Vessel''

column to obtain our annual totals for each new survival craft type.\7\

We estimate that 25 new IBAs, 222 new liferafts, 33 new lifeboats, and

31 new rescue boats will be outfitted with equipment subject to this

rule each year.

---------------------------------------------------------------------------

\7\ We calculate the ``Number of New Vessels per Year'' column

by taking the total number of new vessels by subchapter by year from

the MISLE database, and the ``Average per Vessel'' column by

dividing column (b) by column (a) in table 4.

---------------------------------------------------------------------------

We then sum the totals for each survival craft type across each

affected subchapter to obtain our estimated population of new survival

craft each year for this final rule. This annual growth in the survival

craft population provides an estimate of the number of new survival

craft that will enter the market each year. The vessel owners and

operators of these craft will experience cost savings from buying some

equipment, as discussed in this final rule, which will no longer need

Coast Guard approval. Table 5 presents the estimated total number of

new survival craft each year.

Table 5--Average Survival Craft per Vessel

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

IBAs              Inflatable liferafts            Lifeboats               Rescue boats

New vessels -------------------------------------------------------------------------------------------------------

Subchapter                          Type of vessel            per year   Average per               Average per               Average per               Average per

vessel       Total        vessel       Total        vessel       Total        vessel       Total

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

C..........................................  Commercial Fishing Vessels....           19         0.04            1         1.04           20         0.02            0         0.01            0

C..........................................  Uninspected Passenger Vessels.            1         0.06            0         1.49            1         0.01            0         0.04            0

D..........................................  Tank..........................            5         0.01            0         2.19           11         1.68            8         0.15            1

[[Page 68276]]

H..........................................  Passenger.....................            2         3.35            7         2.32            5         0.48            1         1.50            3

I..........................................  Cargo.........................            9            0            0         3.13           28         1.16           10         0.60            5

I-A........................................  Mobile Offshore Drilling Units            1            0            0         4.61            5        10.93           11         0.65            1

K..........................................  Small Passenger...............            5         1.65            8         3.05           15         0.01            0         0.53            3

L..........................................  Offshore Supply Vessels.......           11            0            0         4.12           45         0.16            2         0.95           10

M..........................................  Towing Vessels................           22         0.06            1         1.04           23            0            0         0.04            1

R..........................................  Nautical Schools..............            0         0.07            0         4.83            0         2.72            0         0.76            0

R..........................................  Sailing Schools...............            0            0            0         2.40            0         0.10            0         0.70            0

T..........................................  Small Passenger...............           35         0.24            8         1.77           62            0            0         0.20            7

U..........................................  Oceanographic Research........            1         0.04            0         3.51            4         0.72            1         0.49            0

Other Vessels..............................  Other Vessels.................            2         0.15            0         1.56            3         0.07            0         0.10            0

--------------------------------------------------------------------------------------------------------------------

Total..................................  ..............................          113            6           25           37          222           18           33            7           31

------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Note: Totals may not sum due to rounding.

Subchapters K and T Vessels

This rule also affects all U.S.-flagged vessel operators regulated

under subchapters K and T, as these vessel operators are required to

maintain a Coast Guard-approved first-aid kit onboard their vessels, in

addition to any first-aid kits carried in the survival craft. The

owners and operators of these small passenger vessels will no longer be

required to maintain Coast Guard-approved first-aid kits aboard the

vessels themselves. Using MISLE data, we estimate there to be 5,982

existing small passenger vessels, with 40 new vessels being built on an

annual basis. This number includes all small passenger vessels defined

in subchapters K and T, found in Sec. Sec.  121.710 and 184.710,

respectively, regardless of what type of survival craft they have on

board.

Equipment Type for Each Survival Craft

The type of equipment each survival craft is required to carry

varies depending on the intended use of the survival craft. Generally,

survival craft intended for longer (international) voyages require more

equipment than those intended to be used closer to shore. Lifeboats on

inspected vessels generally must carry an equipment pack for an

international voyage.\8\ Table 6 contains the equipment required by

pack and type of survival craft.

---------------------------------------------------------------------------

\8\ With the exception of lifeboats on sailing school vessels,

which must carry the equipment required in Sec. Sec.  169.527 and

169.529.

---------------------------------------------------------------------------

[[Page 68277]]

[GRAPHIC] [TIFF OMITTED] TR14NO22.000

[[Page 68278]]

Equipment Pack Types for Commercial Fishing Vessels

Commercial fishing vessels must be equipped with either a Coastal

Service pack, a SOLAS A pack, or a SOLAS B pack, depending on vessel

size, distance traveled, whether the ocean route is designated as a

cold-water route or warm-water route, and the number of persons on

board. Table 7 provides a brief description of the packs that can be

carried by lifeboats and liferafts.\9\

---------------------------------------------------------------------------

\9\ Readers can find more information on inflatable liferafts

for domestic service at https://ecfr.io/Title-46/sp46.6.160.160_1051.

Table 7--Description of Packs carried by Lifeboats and Liferafts

------------------------------------------------------------------------

Type of pack                           Contents

------------------------------------------------------------------------

Coastal Service pack.........  A Coastal pack will contain a Sea Anchor

(Automatically Deployed), Floating/Heavy

Line (Length 100 feet), Rain Water

Collector, Floatable Knife, Waterproof

Equipment Bag, Raft Use Instructions,

Individual Thermal Protective Aids (2

nos.), Floatable Paddles (1 pair),

Manual Inflation/Bilge Pump, Repair

Clamps (6 nos.), Adhesive and Patch

Repair Kit.

SOLAS B pack.................  In addition to the items listed in the

Coastal pack, a SOLAS B pack will

contain: Waterproof Flashlight, a Spare

Flashlight Bulb, Spare Flashlight ``D''

Cell Batteries (3 nos.), Sponges (2

nos.), Bailer, SOLAS Handheld Flares (3

nos.), SOLAS Rocket Parachute Flares (2

nos.) Buoyant Smoke Signal (1 no.),

Seasick Bags (1 per person), Water

Storage Bag, Thermal Protective Aid,

Heliograph Mirror (for signaling), First-

Aid Kit, Signaling Whistle, Anti-

Seasickness Pills (6 Per Person), Spare

Sea Anchor.

SOLAS A pack.................  In addition to the items listed in the

Coastal pack and the items listed in

SOLAS B, a SOLAS A pack will include: a

Graduated Drinking Cup, Drinking Water

(6 to 20 Person Capacity), Food Ration

(10kj per Person), Can Opener, Fishing

Kit, SOLAS Handheld Flares (Total 6

nos.) and a SOLAS Rocket Parachute Flare

(Total 4 nos.).

------------------------------------------------------------------------

Equipment Pack Types for Survival Craft

We used vessel route types from MISLE to estimate the percentage of

vessels with a SOLAS A pack compared to a SOLAS B pack. We presume that

all vessels with ``Ocean'' listed as a route type carry survival craft

with SOLAS A packs. We estimate the remaining route types, not listed

as ``Ocean,'' will have SOLAS B packs. Using commercial fishing vessel

data from MISLE and knowledge from subject matter experts from the

Coast Guard's Lifesaving & Fire Safety Division (CG-ENG-4), who

specialize in survival craft data, we estimate that 50 percent of non-

oceangoing fishing vessels will have Coastal Service packs and 50

percent of non-oceangoing fishing vessels will have SOLAS B packs.

We created a distribution of SOLAS A, SOLAS B, and Coastal Service

packs by pulling all U.S.-flagged vessels by the inspection subchapter

and then pulling these vessels by route type from the MISLE database.

We excluded any vessels that did not have survival craft or had an

unknown field for survival craft in the MISLE database. The route-type

designation included ``Ocean'' for oceangoing vessels in MISLE, which

we designated as SOLAS A vessels.\10\ We designated the remainder as

SOLAS B vessels, except for commercial fishing vessels.\11\ We then

calculated the number of SOLAS A packs by dividing the population of

our vessels (by subchapter) by the sum of vessels that had ``Ocean''

routes and dividing that sum by the sum of vessels in that given

subchapter. To calculate the percentage of SOLAS B packs, we simply

subtracted the number of SOLAS A packs from 100 percent. This data pull

provided the total number of inflatable liferafts and lifeboats,

respectively, and the percentage of each survival craft pack type by

subchapter, which is presented in table 8.

---------------------------------------------------------------------------

\10\ The ``Ocean'' designation in MISLE specifically refers to

vessels with SOLAS certificates that designate them as SOLAS A

vessels. The MISLE data being pulled is from 2008-2020.

\11\ We broke out the Coastal routes and short international

routes by vessel, because Commercial Fishing Vessels are the only

type of vessels in our affected population that will carry Coastal

Service packs instead of only having SOLAS B packs for short

international shipping routes.

Table 8--Percentage of Equipment Pack Types for Lifeboats and Liferafts by Subchapter

----------------------------------------------------------------------------------------------------------------

Short

Total number      Number of        Coastal     international/  International/

Type of vessel           of vessels (a)    oceangoing     service pack     SOLAS B (d)     SOLAS A (e)

vessels (b)    (c) (percent)     (percent)       (percent)

----------------------------------------------------------------------------------------------------------------

Commercial Fishing (Subchapter             6,022            3387              22              22              56

C).............................

Uninspected Passenger                        173             105  ..............              39              61

(Subchapter C).................

Tank (Subchapter D).............             323             313  ..............               3              97

Passenger (Subchapter H)........             191              67  ..............              65              35

Cargo and Miscellaneous                     1037             974  ..............               6              94

(Subchapter I).................

Mobile Offshore Drilling Units                57              55  ..............               4              96

(Subchapter I-A)...............

Small Passenger (Subchapter K)..             311               6  ..............              98               2

Offshore Supply (Subchapter L)..             338             335  ..............               1              99

Towing (Subchapter M)...........            1434            1123  ..............              22              78

Nautical Schools (Subchapter R).              29              28  ..............               3              97

Sailing Schools (Subchapter R)..              10               2  ..............              80              20

Small Passenger (Subchapter T)..            4231             872  ..............              79              21

Oceanographic Research                        74              42  ..............              43              57

(Subchapter U).................

Other...........................             517             300  ..............              42              58

----------------------------------------------------------------------------------------------------------------

Note: Totals may not sum due to rounding.

[[Page 68279]]

We then estimated the number of liferafts and lifeboats by

equipment pack type for existing and new vessels by looking at the

total number of packs carried by lifeboats and liferafts. Table 9

presents the number of SOLAS A, SOLAS B, and Coastal Service packs by

liferaft and lifeboat for each subchapter of vessels.

We calculated the total number of inflatable liferafts with Coastal

Service Packs (column (a) in table 9) by multiplying the percentage of

Coastal Service Packs in liferafts and lifeboats (column (c) in table

8) by the total number of inflatable liferafts by subchapter (column

(c) in table 4). We calculated column (b) in table 9, ``Short

International/SOLAS B packs for inflatable liferafts,'' by multiplying

column (d) in table 8, which is the percentage of Short International/

SOLAS B packs by vessel subchapter, by column (c) in table 4, which is

the total number of inflatable liferafts by subchapter. We calculated

column (c) in table 9, ``International/SOLAS A packs for liferafts,''

by multiplying column (e) in table 8, which is the percentage of

International/SOLAS A packs by vessel subchapter, by column (c) in

table 4, which is the total number of inflatable liferafts by

subchapter. We calculated column (e) in table 9, ``Short International/

SOLAS B packs for lifeboats,'' by taking the sum of multiplying columns

(c) and (d), the percentages of Coastal packs and Short International/

SOLAS B packs in table 8 by column (d) in table 4, which is the total

number of lifeboats by subchapter. Finally, we calculated column (f) in

table 9, ``International/SOLAS A packs for lifeboats'' by multiplying

column (e) from table 8, which is the percentage of International

Packs/SOLAS A, by column (d) in table 4, which is the total number of

lifeboats by subchapter.

BILLING CODE 9110-04-P

[[Page 68280]]

[GRAPHIC] [TIFF OMITTED] TR14NO22.001

Table 10 presents the total number of new packs needed each year

for new survival craft. We calculated this table by taking the number

of new lifeboats and liferafts presented in table 5 and multiplying

that figure by the

[[Page 68281]]

distribution in table 8 to obtain the number of new packs needed for

the new liferafts and lifeboats on vessels each year.

BILLING CODE 9110-04-C

Table 10--Lifeboats and Liferafts by Equipment Pack Type Needed on an Annual Basis Broken Out by Subchapter

--------------------------------------------------------------------------------------------------------------------------------------------------------

Inflatable liferafts                                          Lifeboats

---------------------------------------------------------------------------------------------------------------

Type of vessel                                    Short                                           Short

Coastal     international/  International/       Total      international/  International/       Total

service pack       SOLAS B         SOLAS A                         SOLAS B         SOLAS A

--------------------------------------------------------------------------------------------------------------------------------------------------------

Commercial Fishing (Subchapter C).......               4               5              11              20               0               0               0

Uninspected Passenger (Subchapter C)....  ..............               0               1               1               0               0               0

Tank (Subchapter D).....................  ..............               0              11              11               0               8               8

Passenger (Subchapter H)................  ..............               3               2               5               1               0               1

Cargo and Miscellaneous (Subchapter I)..  ..............               2              26              28               1               9              10

Mobile Offshore Drilling Units            ..............               0               5               5               0              11              11

(Subchapter I-A).......................

Small Passenger (Subchapter K)..........  ..............              15               0              15               0               0               0

Offshore Supply (Subchapter L)..........  ..............               0              45              45               0               2               2

Towing (Subchapter M)...................  ..............               5              18              23               0               0               0

Nautical Schools (Subchapter R).........  ..............               0               0               0               0               0               0

Sailing Schools (Subchapter R)..........  ..............               0               0               0               0               0               0

Small Passenger (Subchapter T)..........  ..............              49              13              62               0               0               0

Oceanographic Research (Subchapter U)...  ..............               2               2               4               0               1               1

Other Vessels...........................  ..............               1               2               3               0               0               0

---------------------------------------------------------------------------------------------------------------

Total...............................               4              82             136             222               2              31              33

--------------------------------------------------------------------------------------------------------------------------------------------------------

Note: Values may not sum due to rounding.

Benefits

In addition to the nonquantified benefits discussed in table 2,

this rule will generate a cost savings as follow:

Cost Savings

This rule will generate a cost savings to: (1) vessel owners and

operators from having the option to purchase less expensive survival

craft equipment; (2) equipment manufacturers from reducing reporting,

recordkeeping, and production requirements of survival craft equipment;

and (3) the Federal Government from reducing recordkeeping

requirements. The details and calculations of the cost savings are

discussed later in this final rule.

Wages

This rule will reduce the burden of review that is required by both

industry and the Federal Government. This review includes preparing COA

applications, renewals, and product instructions by certain

manufacturers. We presume clerical employees will be responsible for

all the manufacturer's recordkeeping activities, and production

employees will be responsible for marking equipment and packing

instructions. Federal Government employees who possess the technical

knowledge to review submissions to ensure safety standards will be

senior engineers at the GS-14 grade. These employees will be

responsible for the review of all the submitted information.

We calculate the costs for each activity by estimating the labor

hours required in each labor category and then multiplying those

burdens by the wage rate for each labor category. For this analysis, we

calculated private sector wages using 2020 wage data from the U.S.

Bureau of Labor Statistics (BLS) Occupational Employment Statistics

(OES) for the miscellaneous manufacturing sector (North American

Industry Classification System (NAICS) 339000).\12\ We added a load

factor to the industry wages using December 2020 wage and total

compensation data from the BLS Employer Costs for Employee Compensation

(ECEC) survey, which accounts for employee benefits. This load factor

represents the total benefits as a percentage of total salary.\13\

Table 11 summarizes the loaded wage rates for industry used in this RA.

---------------------------------------------------------------------------

\12\ https://www.bls.gov/oes/2020/may/naics3_339000.htm.

\13\ A loaded labor rate is what a company pays per hour to

employ a person beyond the hourly wage. Instead, the loaded labor

rate includes the cost of benefits (health insurance, vacation,

etc.). We calculate the load factor for wages by dividing total

compensation by wages and salaries. For this analysis, we used BLS'

Employer Cost for Employee Compensation/Manufacturing Occupations,

Private Industry report (Series IDs, CMU2013000000000D and

CMU2023000000000D for all workers using the multi-screen data

search). Using 2020 Quarter 4 Manufacturing data, we divided the

total compensation amount of $40.02 by the wage and salary amount of

$26.56 to get the load factor of 1.51 ($40.02 divided by $26.56).

This data is found in table 4 of the Employer Costs for Employee

Compensation December 2020 News Release available at Employer Costs

for Employee Compensation Archived News Releases: U.S. Bureau of

Labor Statistics (bls.gov).

Table 11--Derivation of 2020 Loaded Industry Wage Rates

[Rounded to the nearest dollar]

--------------------------------------------------------------------------------------------------------------------------------------------------------

2020 hourly wage     Load factor      Loaded hourly

------------------------------------       wage

Personnel  category                                  Data source(s)                                                      -----------------

(a)               (b)         (c) = (a) x (b)

--------------------------------------------------------------------------------------------------------------------------------------------------------

Technical......................................  Wage Rate: Mean hourly wage for Industrial                   $44.10              1.51               $67

Engineers, including Health and Safety:

Occupation code (17-2110) under the

miscellaneous manufacturing sector (NAICS

339000) from the BLS OES. Link: https://www.bls.gov/oes/2020/may/naics3_339000.htm#17-0000 0000.

[[Page 68282]]

Loading Factor: Calculated from December 2020 BLS

ECEC non-seasonally adjusted data for wage and

salaries (CMU2013000000000D) and total

compensation (CMU2023000000000D) for private

industry workers in the miscellaneous

manufacturing sector.

Clerical.......................................  Wage Rate: Mean hourly wage for Information and              $19.87              1.51               $30

Record Clerks: Occupation code (43-4000) under

the miscellaneous manufacturing sector (NAICS

339000) from the BLS OES. Link: https://www.bls.gov/oes/2020/may/naics3_339000.htm#43-4000 4000.

Loading Factor: Calculated from December 2020 BLS

ECEC non-seasonally adjusted data for wage and

salaries (CMU2013000000000D) and total

compensation (CMU2023000000000D) for private

industry workers in the manufacturing sector.

Production.....................................  Wage Rate: Mean hourly wage for Assemblers:                  $17.22              1.51               $26

Occupation code (51-2000) in the miscellaneous

manufacturing sector (NAICS 339000) from the BLS

OES. Link: https://www.bls.gov/oes/2020/may/naics3_339000.htm#51-2000.

Loading Factor: Calculated from December 2020 BLS

ECEC non-seasonally adjusted data for wage and

salaries (CMU2013000000000D) and total

compensation (CMU2023000000000D) for private

industry workers in the manufacturing sector.

--------------------------------------------------------------------------------------------------------------------------------------------------------

Note: Values may not sum due to rounding.

For Federal Government employees, The Office of Personnel

Management (OPM) lists the hourly pay for Federal employees according

to the Washington, DC area General Schedule (GS) pay tables.\14\ OPM

records the hourly pay of GS-14, step 5 (the midpoint of the pay band)

as $65.88. We calculate the share of total compensation of Federal

employees to account for a government employee's non-wage benefits. The

Congressional Budget Office (2017) reports total compensation to

Federal employees as $64.80 per hour and wages as $38.30.\15\ We

determine the load factor to be approximately 1.69.\16\ We multiplied

$65.88 by 1.69 to obtain a loaded hourly wage rate of approximately

$111.34 for a GS-14 senior engineer.

---------------------------------------------------------------------------

\14\ https://www.opm.gov/policy-data-oversight/pay-leave/salaries-wages/salary-tables/20Tables/html/DCB_h.aspx.

\15\ Congressional Budget Office (2017), ``Comparing the

Compensation of Federal and Private-Sector Employees, 2011 to

2015,'' https://www.cbo.gov/system/files/115th-congress-2017-2018/reports/52637-federalprivatepay.pdf.

\16\ $64.80 divided by 38.30.

---------------------------------------------------------------------------

Cost Savings to Equipment Manufacturers

We estimate that manufacturers of Coast Guard-approved equipment

will have a cost savings associated with no longer having to complete

applications to obtain and maintain Coast Guard approval. In addition,

this rule will remove recordkeeping and reporting requirements, and

reduce testing requirements for some pieces of survival equipment.

Number of Survival Craft Products

This rule modifies the approval requirements for nine categories of

survival craft equipment. In total, there are 28 approvals for these 9

categories of survival craft equipment. These are the specific items

that vessel owners and operators purchase to comply with the vessel

carriage regulations found in 46 CFR chapter I, subchapters C, T, K,

and W.\17\ These items are required to be stowed on board survival

craft.

---------------------------------------------------------------------------

\17\ Refer to the appendix titled ``Appendix C: Carriage

Requirements for all the Survival Craft Equipment'' in the docket

folder for more information on carriage requirements for all vessels

affected by this final rule.

---------------------------------------------------------------------------

To comply with the lifesaving equipment regulations in 46 CFR

chapter I, subchapter Q, manufacturers submit an application to the

Coast Guard for review and approval. Once approved, the manufacturer of

each piece of equipment must mark it (or stamp it) with its approval

number (see table 12).

There are two types of survival craft equipment: (1) items that are

durable and need not be replaced or serviced frequently, such as bilge

pumps, compasses, fishing kits,\18\ jackknives, signaling mirrors,

hatchets, and sea anchors; and (2) items that are not durable, expire,

and must be replaced, such as first-aid kits and emergency drinking

water. We used the annual total number of pieces of survival craft

equipment needed to stock new survival craft in order to forecast the

number of new pieces of equipment manufactured and stamped on an annual

basis. We estimate that, in the long term, the supply of new survival

equipment will equal the demand of new survival craft equipment.

---------------------------------------------------------------------------

\18\ There is currently one Coast Guard-approved fishing kit on

CGMIX. The only non-durable aspect of the fishing kit is the bait,

which is made of a synthetic resin known as plastisol. If stored

properly, plastisol has an indefinite shelf life.

---------------------------------------------------------------------------

The Coast Guard does not have substantive data on how long these

durable goods last, and we estimate that these goods will last as long

as the survival craft themselves.

We discuss the renewal rate of non-durable goods, first-aid kits,

and water later in this analysis.\19\ Table 12 lists the estimated

number of pieces of survival craft equipment manufactured on an annual

basis.

---------------------------------------------------------------------------

\19\ Refer to the sections titled First-Aid Kits, First-Aid Kits

for Liferafts and IBA, and Emergency Water further in the regulatory

analysis.

[[Page 68283]]

Table 12--Estimated Number of Pieces of Equipment Manufactured Annually

------------------------------------------------------------------------

Annual  number

Equipment                    Approval      of  pieces of

series         equipment

------------------------------------------------------------------------

Compass.................................         160.014              87

First-aid kit for Lifeboats.............         160.041             188

First-aid kit for Liferafts.............         160.054             285

Fishing kit.............................         160.061              38

Hatchet.................................         160.013              92

Jackknife...............................         160.043              46

Mirror, Signaling.......................         160.020             338

-------------------------------

Total...............................  ..............           1,074

------------------------------------------------------------------------

Equipment Approval and Markings

In the current regulations, manufacturers seeking Coast Guard

approval must submit a COA application with information such as

technical plans, drawings, specifications, instructional materials, and

test reports. In addition to the initial application, manufacturers of

Coast Guard-approved equipment must also submit application renewals

every 5 years to maintain their approval status. Table 3 presents the

estimated number of new COA applications for each equipment type, as

the annual average number of new products each year.

Table 13 presents the estimated number of application renewals for

each equipment type. Since the Coast Guard estimates that 1 of every 5

applications will be renewed on an annual basis, the number of renewal

applications is equal to 20 percent of the total number of products.

Once a product has been approved, the manufacturer must stamp each

individual piece of survival craft equipment with the Coast Guard

approval number and other information.

Table 13--Total Number of New Renewals

----------------------------------------------------------------------------------------------------------------

Annual

Approval          Total      percentage  of    Total renewal

Equipment                        series         products        COAs for       applications

renewals         annually

(a)             (b)   (c) = (a) x (b)

----------------------------------------------------------------------------------------------------------------

Bilge pump....................................         160.044               3              20               0.6

Compass.......................................         160.014               3              20               0.6

First-aid kit for Lifeboats *.................         160.041               5              20                 1

First-aid kit for Liferafts...................         160.054               5              20                 1

Fishing kit...................................         160.061               1              20               0.2

Hatchet.......................................         160.013               1              20               0.2

Jackknife.....................................         160.043               1              20               0.2

Mirror, Signaling.............................         160.020               2              20               0.4

Sea anchor....................................         160.019               1              20               0.2

Water **......................................         160.026               6              20               1.2

-----------------------------------------------------------------

Total.....................................  ..............              28              20                 6

----------------------------------------------------------------------------------------------------------------

Note: Values may not sum due to rounding.

* This includes the first-aid kits described in the subchapters K and T section of this preamble, which are

covered under the same approval subpart in the CFR.

** For emergency drinking water, this only includes implementation in the first 5 years of the analysis period.

We present the number of affected products in Years 6 through 10 of the analysis period later in this RA.

We estimate that it will take the technical staff 2 hours to

prepare a new application, and the clerical staff will spend 0.17 hours

(10 minutes) \20\ per application on recordkeeping, for a total cost of

$139 per new application [(2 technical hours x $67) + (0.17 clerical

hours x $30) = $139]. For renewal applications, we estimate a burden of

0.5 technical hours and 0.17 clerical hours, for a total cost of $39

[(0.5 technical hours x $67) + (0.17 clerical hours x $30) = $39].

Under this rule, the Coast Guard no longer requires approval

applications for any new survival craft equipment. As shown in table

14, we estimate this will result in a cost saving to industry of

approximately $117 per year for new applications, and approximately

$219 per year for renewal applications. This results in a total annual

cost savings of about $336.

---------------------------------------------------------------------------

\20\ Based on information from the subchapter Q ICR.

[[Page 68284]]

Table 14--Annual Cost Savings of Industry for No Longer Having to Submit New and Renewal Certificate of Approval Applications

--------------------------------------------------------------------------------------------------------------------------------------------------------

New applications                Renewal applications

Approval    --------------------------------------------------------------------    Total cost

Equipment                          series      Total  number of    Total cost    Total  number of    Total cost         savings

applications *      savings      applications **      savings

(a)  (b) = (a) x [-               (c)  (d) = (c) x [-   (e) = (b) + (d)

$139]                              $39]

--------------------------------------------------------------------------------------------------------------------------------------------------------

Bilge pump........................................         160.044              0.09            -$13              0.60            -$23              -$36

Compass...........................................         160.014              0.09             -13              0.60             -23               -36

First-aid kit for Lifeboats.......................         160.041              0.15             -21              1.00             -39               -60

First-aid kit for Liferafts.......................         160.054              0.15             -21                 1             -39               -60

Fishing kit.......................................         160.061              0.03              -4              0.20              -8               -12

Hatchet...........................................         160.013              0.03              -4              0.20              -8               -12

Jackknife.........................................         160.043              0.03              -4              0.20              -8               -12

Mirror, Signaling.................................         160.020              0.06              -8               0.4             -16               -24

Sea anchor........................................         160.019              0.03              -4              0.20              -8               -12

Water.............................................         160.026              0.18             -25              1.20             -47               -72

-----------------------------------------------------------------------------------------------------

Total.........................................  ..............  ................            -117  ................            -219              -336

--------------------------------------------------------------------------------------------------------------------------------------------------------

Note: Values may not sum due to rounding.

* Refer to column (c) in table 3.

** Refer to column (c) in table 13.

The Coast Guard is removing requirements that equipment must be

marked with a Coast Guard approval number. With the exception of

compasses and hatchets, equipment needs to be marked only to indicate

that it meets standards set in ISO 18813. Compasses will no longer need

to be marked with their Coast Guard approval number, but will still

need to be marked to indicate they meet ISO 25862, as is currently

required by the Coast Guard approval guidelines for magnetic compasses

in lifeboats and rescue boats. Hatchets will not need to be marked at

all, as they do not have to meet any consensus standard and because

this rule removes the marking required by Sec.  160.013-5.

The Coast Guard assumes the burden to mark the equipment is the

same whether it is marked with a Coast Guard approval number or whether

it is marked indicating that it meets the ISO standard; therefore, this

change will only result in a cost savings to the manufacturers of

hatchets. The Coast Guard estimates that it takes industry 0.06 hours

of production labor time \21\ to mark each individual piece of

equipment at a cost of $1.56 (0.06 hours x $26 = $1.56) per piece of

equipment. We estimate that 92 hatchets will no longer need to be

marked each year (see table 12), for a total cost savings of

approximately $144 ($1.56 x 92).\22\

---------------------------------------------------------------------------

\21\ This is based on information from the subchapter Q ICR.

\22\ This value is incorporated in column (a) of table 19.

---------------------------------------------------------------------------

Instructions

The Coast Guard currently requires that equipment manufacturers

provide instruction material with certain types of equipment to ensure

that crew members have access to information on the proper use of the

equipment. We currently require instructions for five of the nine types

of equipment subject to this rulemaking: compasses, first-aid kits,

mirrors, fishing kits, and jackknives. ISO 18813 requires instructions

for three types of equipment: first-aid kits, mirrors, and fishing

kits. ISO 18813 does not state that instructions need to be provided

for compasses and jackknives; therefore, the manufacturers of compasses

and jackknives will no longer have to develop and maintain instructions

for their products under this rule.

Based on information in the current subchapter Q ICR (OMB Control

Number 1625-0035), we estimate that it takes about 8 hours of time to

prepare a set of instructional materials for new equipment, for a cost

of about $536 (8 hours x $67/hour).

Table 15 presents the total annual industry cost savings, $64, for

no longer having to develop new instructions for some types of new

survival craft equipment. The total cost in columns (b) and (d), $536,

is the loaded wage of a safety engineer and inspector, $67, multiplied

by the estimated burden of work, 8 hours, for preparing a set of new

instructions. This table presents the baseline scenario burden, the

proposed post-regulatory scenario burden, and the difference between

the two as cost savings.

Table 15--Annual Cost Savings of Modifying New Instruction Requirements for Applicable Equipment

--------------------------------------------------------------------------------------------------------------------------------------------------------

Baseline scenario             Post-regulatory scenario

Approval    --------------------------------------------------------------------    Total cost

Equipment                          series         Total new                         Total new                           savings

instructions      Total cost      instructions      Total cost

(a)  (b) = (a) x $536             (c)  (d) = (c) x $536     (e) = (d)-(b)

--------------------------------------------------------------------------------------------------------------------------------------------------------

Compass...........................................         160.014            0.09               $48               0                $0              -$48

First-aid kit for Lifeboats.......................         160.041            0.15                80            0.15                80                 0

First-aid kit for Liferafts.......................         160.054            0.15                80            0.15                80                 0

Fishing kit.......................................         160.061            0.03                16            0.03                16                 0

Jackknife.........................................         160.043            0.03                16               0                 0               -16

Mirror, Signaling.................................         160.020            0.06                32            0.06                32                 0

-----------------------------------------------------------------------------------------------------

Total.........................................  ..............            0.51               272            0.39               208               -64

--------------------------------------------------------------------------------------------------------------------------------------------------------

Note: Totals may not sum due to rounding.

[[Page 68285]]

Laboratory Testing and Recordkeeping

As current regulations stand, the Coast Guard requires product

testing and recordkeeping for some lifesaving equipment to ensure the

equipment meets minimum performance requirements. Table 16 presents a

comparison of the current Coast Guard testing requirements and the

testing requirements stated in ISO 18813 and ISO 25862 (for compasses).

This table also contains a qualitative description of the change in

costs associated with modifying the current testing requirements. We

were unable to obtain any cost data from the Coast Guard-approved labs

that conduct the testing of this equipment, and we received no comments

to the NPRM on this.\23\

---------------------------------------------------------------------------

\23\ We asked four Coast Guard-approved laboratories for cost

estimates for the testing requirements, but the labs were unable to

provide any cost information.

Table 16--Previous and New Product Testing Requirements

----------------------------------------------------------------------------------------------------------------

Previous testing           New testing

Product                     requirements            requirements               Cost impact

----------------------------------------------------------------------------------------------------------------

Compasses..........................   All testing     All testing    No cost change, as the

requirements from       requirements for        requirements of ISO 613

section 4 of ISO 613    class B Compasses as    and ISO 25862 are not

\a\.                    stated in ISO 25862.    substantively different.

Dry Heat \a\.   Dry Heat.....

Low             Damp Heat....

Temperature \a\.        Low

Vibration \a\   Temperature.

Solar           Vibration....

Radiation \a\.          Solar

Corrosion \a\   Radiation.

Corrosion....

Bilge Pump.........................   Capacity        Capacity       None. Testing requirements

Testing \b\.            Testing.                are the same.

Head Pressure   Head Pressure

Testing \b\.            Testing.

Operating       Operating

Lever Testing \b\.      Lever Testing.

Jackknife..........................   Hardness Test   Cutting Tests  Unquantified cost savings.

\c\.                                            The Coast Guard is unable

Bending and                             to assess the change in

Drop Tests \c\.                                 burden; there is no

Cutting Tests                           substantive data.

\c\.

First-Aid for Lifeboats............   Accelerated     None.........  Unquantified cost savings.

weathering \d\.                                 There is no change in

Salt spray                              testing requirements;

\d\.                                            therefore, there is no

Temperature                             change in burden.

change \d\.

Container

watertightness \d\.

Carton

watertightness \d\.

First-Aid for Liferafts............   Accelerated     None.........  Unquantified cost savings.

weathering \e\.                                 There is no change in

Salt Spray                              testing requirements;

\e\.                                            therefore, there is no

change in burden.

Mirrors............................   Reflection      Reflection     Unknown change in cost. The

Test \f\.               Test.                   Coast Guard is unable to

Flatness        Flatness Test   assess the change in

Tests \f\.              Dropping Test   burden as there is no

Dropping Test   Oil-            substantive data.

\f\.                    Resistance Test.

Salt Spray      Lanyard

\f\.                    Strength Test.

Watertightness.

Emergency Water....................   Chemical and    Water quality  None. Testing requirements

biological analysis.    must be verified by     are the same, as under the

Temperature     the local               ISO standard the water

Storage.                municipality or         must satisfy international

Leakage......   independent lab.        chemical and

Water           Low and High    microbiological

Immersion Testing.      Temperature Storage.    requirements. Concerning

Durability...   Leakage......   the water quality testing,

Corrosion....   Water           the Coast Guard was unable

Drop.........   Immersion Testing.      to obtain any cost data

Durability...   from the laboratories.

Corrosion....

Drop.........

----------------------------------------------------------------------------------------------------------------

Sources:

\a\ ``United States Coast Guard Approval Guideline for Magnetic Compasses in Lifeboats/Rescue Boats,'' USCG

Approval Series 160.014, December 2005.

\b\ Sec.   160.044-4

\c\ Sec.   160.043-5

\d\ Sec.   160.041-5

\e\ Sec.   160.054-5

\f\ Documentation provided by subject matter experts in CG-ENG-4.

Based on the information from the current subchapter Q ICR, we

estimate that recordkeeping takes 2 hours of clerical time per year and

costs $60 (2 hours x $30 clerical staff loaded hourly wage rate). The

Coast Guard is removing the requirements for testing records for seven

types of equipment listed in this final rule, as these manufacturers no

longer need these records to document that their products meet the

requirements of the ISO 18813. Table 17 presents the total cost savings

of about $1,500 to industry from removing requirements to keep records

of laboratory testing. The $60 figure used in calculating total cost in

columns (b) and (d) represents the loaded hourly

[[Page 68286]]

wage of a record clerk ($30) multiplied by the estimated burden of work

for fulfilling recordkeeping requirements (2 hours). This table

presents the baseline scenario burden and the post-regulatory scenario

burden and then presents the difference of the two burdens as cost

savings.

Table 17--Annual Cost Savings to Manufacturers for Testing Recordkeeping Requirements

--------------------------------------------------------------------------------------------------------------------------------------------------------

Baseline scenario             Post-regulatory scenario

Approval    --------------------------------------------------------------------    Total cost

Equipment                          subpart          Total                             Total                             savings

products        Total cost        products        Total cost

(a)   (b) = (a) x $60             (c)   (d) = (c) x $60   (e) = (d) - (b)

--------------------------------------------------------------------------------------------------------------------------------------------------------

Bilge pump........................................         160.044               3              $180               0                $0             -$180

Compass...........................................         160.014               3               180               0                 0              -180

First-aid kit for Lifeboats.......................         160.041               5               300               0                 0              -300

First-aid kit for Liferafts.......................         160.054               5               300               0                 0              -300

Jackknife.........................................         160.043               1                60               0                 0               -60

Mirror, Signaling.................................         160.020               2               120               0                 0              -120

Water.............................................         160.026               6               360               0                 0              -360

-----------------------------------------------------------------------------------------------------

Total.........................................  ..............              25             1,500               0                 0            -1,500

--------------------------------------------------------------------------------------------------------------------------------------------------------

Note: Totals may not sum due to rounding.

Laboratory Inspections

The Coast Guard currently requires inspectors to examine the

manufacturing process in order to ensure that quality control is

maintained. This rule removes these requirements; however, the Coast

Guard is unable to determine if this removal will generate any cost

savings to industry. Hence, the Coast Guard is not quantifying it as a

cost savings. Manufacturers are likely to still have their production

line inspected to ensure quality as part of best industry practices.

Moreover, manufacturers may continue third-party testing to maintain

certifications, such as the ISO 9001 standard, or to meet other

regulatory obligations. At the time of this final rule, the Coast Guard

does not have enough information to quantify any potential changes in

cost resulting from the changes in inspection requirements.

Additionally, the Coast Guard requires inspecting entities to issue

annual reports to enable a comparison between the production line and

the prototype tested by the Coast Guard.\24\ We were able to estimate a

cost savings that resulted from the removal of this reporting

requirement using information from the subchapter Q ICR, which

estimated that this recordkeeping takes 24 hours of clerical time per

year on average and costs $720 (24 hours x $30 clerical wage rate). The

Coast Guard is removing this reporting requirement for all types of

survival craft equipment. As shown in table 18, we estimate a total

annual cost savings of approximately $17,280. This table presents the

baseline scenario burden, the post-regulatory scenario burden, and the

difference between the two as cost savings.

---------------------------------------------------------------------------

\24\ While the Coast Guard currently requires testing for

jackknives, it does not require laboratory inspections. Therefore,

there are no cost savings to jackknife manufacturers from this

change.

Table 18--Annual Cost Savings for Laboratory Inspection Records

--------------------------------------------------------------------------------------------------------------------------------------------------------

Baseline scenario             Post-regulatory scenario

Approval    --------------------------------------------------------------------  Total change in

Equipment                          series           Total                             Total                              cost

products        Total cost        products        Total cost

(a)  (b) = (a) x $720             (c)  (d) = (c) x $720   (e) = (d) - (b)

--------------------------------------------------------------------------------------------------------------------------------------------------------

Bilge pump........................................         160.044               3            $2,160               0                $0           -$2,160

Compass...........................................         160.014               3             2,160               0                 0            -2,160

First-aid kit for Lifeboats.......................         160.041               5             3,600               0                 0            -3,600

First-aid kit for Liferafts.......................         160.054               5             3,600               0                 0            -3,600

Mirror, Signaling.................................         160.020               2             1,440               0                 0            -1,440

Water.............................................         160.026               6             4,320               0                 0            -4,320

-----------------------------------------------------------------------------------------------------

Total.........................................  ..............              24            17,280               0                 0           -17,280

--------------------------------------------------------------------------------------------------------------------------------------------------------

Note: Totals may not sum due to rounding.

Total Cost Savings to Manufacturers

Table 19 presents the annual total cost savings to equipment

manufacturers. We estimate that manufacturers of Coast Guard-approved

bilge pumps, lifeboats, compasses, first-aid kits, fishing kits,

hatchets, jackknives, signaling mirrors, sea anchors, and emergency

water will save approximately $19,324 per year.

Table 19--Total Annual Cost Savings to Equipment Manufacturers

--------------------------------------------------------------------------------------------------------------------------------------------------------

Application

Equipment                           Approval       and marking     Instruction       Product       Laboratory       Total cost

series       requirements    requirements       testing       inspections        savings

(a)             (b)             (c)             (d)   (e) = (a) + (b)

+ (c) + (d)

--------------------------------------------------------------------------------------------------------------------------------------------------------

Bilge pump............................................         160.044            -$36              $0           -$180         -$2,160           -$2,376

Compass...............................................         160.014             -36             -48            -180          -2,160            -2,424

[[Page 68287]]

First-aid kit for Lifeboats...........................         160.041             -60              -0            -300          -3,600            -3,960

First-aid kit for Liferafts...........................         160.054             -60              -0            -300          -3,600            -3,960

Fishing kit...........................................         160.061             -12              -0               0               0               -12

Hatchet...............................................         160.013            -156               0               0               0              -156

Jackknife.............................................         160.043             -12             -16             -60               0               -88

Mirror, Signaling.....................................         160.020             -24              -0            -120          -1,440            -1,584

Sea anchor............................................         160.019             -12               0               0               0               -12

Water.................................................         160.026             -72               0            -360          -4,320            -4,752

-------------------------------------------------------------------------------------------------

Total.............................................  ..............            -480             -64          -1,500         -17,280           -19,324

--------------------------------------------------------------------------------------------------------------------------------------------------------

Note: Totals may not sum due to rounding.

Cost Savings to Vessel Owners or Operators

After gathering price data from a variety of sources, we estimate

that removing approval requirements will allow owners and operators of

vessels to purchase less expensive equipment.\25\ While there are

several companies selling Coast Guard-approved equipment, online

information generally does not specify whether the equipment meets ISO

18813 or similar standards. As a result, we had difficulty finding

price data for survival craft equipment products clearly stating that

they met ISO 18813 standards. However, we were able to identify prices

for two products--emergency provisions and emergency water--that the

manufacturer or advertiser explicitly stated met the requirements of

the ISO 18813 standard.

---------------------------------------------------------------------------

\25\ We looked at online retailers of survival craft equipment

to assess price data. A search of online retailers determined that

equipment that was not type-approved was less expensive than similar

equipment that was type-approved.

---------------------------------------------------------------------------

We then applied percentage price difference between emergency water

products and emergency provisions that had both Coast Guard approval

and met the requirements of ISO 18813, and those emergency provisions

and water products that met only the requirements of ISO 18813.\26\ We

estimate that products without Coast Guard approval affected by this

rule were approximately 28 percent less expensive than products with

Coast Guard approval.\27\

---------------------------------------------------------------------------

\26\ Although emergency provisions are not subject to changes in

this final rule, we still examined them for the purposes of price

comparison, as doing so provided a depth of data allowing us to

determine a more robust ratio.

\27\ We calculated this figure by finding the price differential

for those products that were Coast Guard type-approved and those

products that were not Coast Guard-approved but met ISO standards.

We were not able to derive this figure for all of the products due

to lack of industry data. However, given the similarity of the

equipment type, we assume the price differences would be similar for

all products.

---------------------------------------------------------------------------

We applied this 28-percent price decrease to all the products

affected by this rule, with the exception of first-aid kits, because

the kit content requirements differ between the ISO standard and

current Coast Guard standards, and we estimate the change in price for

first-aid kits by the difference in replacement costs for first-aid

kits. These differences are explained in further detail in the section,

First-Aid Kits, in this RA. For this analysis, we quantified the cost

savings to new vessels from being able to purchase less expensive

equipment, and the cost savings to existing vessels of replacing

expired items with less costly items. For durable items, without data

to estimate how frequently these items are replaced, we are not able to

estimate the cost savings to the owners and operators of existing

vessels for purchasing replacement equipment that we estimate will be

28 percent cheaper. However, since emergency water and first-aid kits

expire, we estimate the cost savings for purchasing replacement

equipment for the owners and operators of both new and existing vessels

based on how frequently this non-durable equipment must be replaced.

This information is presented later in this RA.

Durable Equipment: Bilge Pumps, Compasses, Fishing Kits, Hatchets,

Jackknives, Mirrors, and Sea Anchors

We estimate that only new vessels will purchase bilge pumps,

compasses, fishing kits, hatchets, jackknives, mirrors, and sea anchors

for their survival craft. Based on population estimates (presented in

table 5), 25 new IBAs, 222 new liferafts, 33 new lifeboats, and 31 new

rescue boats will be subject to this rule each year. Table 6 lists the

survival equipment that lifeboats, liferafts, rescues boats, and IBAs

are required to carry. We multiply the populations in table 5 by the

carriage requirements in table 6 to yield the total number of items

purchased for new survival craft in table 20. The Coast Guard requires

new lifeboats to be equipped with bilge pumps, and there were 33 new

lifeboats recorded in table 5, meaning there will be 33 purchases of

new bilge pumps per year.\28\ Only the new lifeboats with equipment

packs for international voyages will require fishing kits (see table

6), and all new lifeboats and rescue boats will be equipped with

compasses, for a total of 64 purchases of compasses each year. All 280

new IBAs, liferafts, and lifeboats are required to be equipped with

mirrors. Finally, 218 liferafts with a SOLAS A or SOLAS B pack will be

equipped with 2 sea anchors each. This rule will require that 93 IBAs,

lifeboats, rescue boats, and liferafts with coastal service packs each

have 1 sea anchor.

---------------------------------------------------------------------------

\28\ The Coast Guard requires all non-self-bailing lifeboats and

rescue boats to have bilge pumps. Based on discussions with subject

matter experts in CG-ENG-4, the Coast Guard estimates that all new

lifeboats will be non-self-bailing and will therefore require bilge

pumps, and all new rescue boats that are not also lifeboats will be

self-bailing and therefore will not require bilge pumps.

---------------------------------------------------------------------------

Table 20 presents the annual cost savings from new vessels removing

Coast Guard approval for bilge pumps, compasses, fishing kits,

hatchets, jackknives, mirrors, and sea anchors. In total, we estimate

an annual cost savings of approximately $78,324 for U.S.-flagged

vessels by removing the type approvals for these 7 types of survival

craft equipment.

[[Page 68288]]

Table 20--Annual Cost Savings to New Vessels From Removing Coast Guard Approval for Bilge Pumps, Compasses, Fishing Kits, Hatchets, Jackknives, Mirrors,

and Sea Anchors

--------------------------------------------------------------------------------------------------------------------------------------------------------

Average  price      Estimated

of  coast     equipment price                                        Average

Equipment                           guard-        without coast      Difference       Number  of       number of      Total cost

approved      guard approval                     survival craft     items per        savings

equipment      requirements                                      survival craft

(a)  (b) = (a) x 0.72     (c) = (b)-(a)             (d)             (e)     (f) = (c) x

(d) x (e)

--------------------------------------------------------------------------------------------------------------------------------------------------------

Bilge pump..........................................            $276              $199              -$77              33               1         -$2,541

Compass.............................................           1,250               900              -350              64               1         -22,400

Fishing kit.........................................              41                30               -11              31               1            -341

Hatchet.............................................              28                20                -8              33               2            -528

Jackknife...........................................              34                24               -10              33               1            -330

Mirror, Signaling...................................              19                14                -5             280               1          -1,400

Sea anchor (Liferafts with SOLAS A and SOLAS B                   343               247               -96             218               2         -41,856

packs).............................................

Sea anchor (Other Survival Craft)...................             343               247               -96              93               1          -8,928

---------------------------------------------------------------------------------------------------

Total...........................................  ..............  ................  ................  ..............  ..............         -78,324

--------------------------------------------------------------------------------------------------------------------------------------------------------

Note: Totals may not sum due to rounding. All product prices are rounded to the nearest whole dollar.

Jackknives as a Replacement for Can Openers

As specified in Sec.  199.175(b)(5), the Coast Guard allows

jackknives to meet the requirements of a can opener, thereby permitting

jackknives to fulfill two requirements. Table 1 in Sec.  199.175 states

that only lifeboats and rigid liferafts with SOLAS A packs require can

openers, and only lifeboats may carry jackknives. This means that rigid

liferafts with SOLAS A packs are currently carrying both knives and can

openers. This rule will allow these vessels to replace their knives

with jackknives, resulting in a cost savings to vessel owners from

being able to purchase only a jackknife instead of both a knife and a

can opener. We estimate that there are a total of 136 new liferafts

each year that carry SOLAS A packs and, further, assume that these

vessel owners and operators will choose to replace a knife with a

jackknife, thus forgoing the need to purchase a can opener.\29\ We

estimate the price of a can opener meeting the requirements of ISO

18813 to be $6.\30\ Therefore, we estimate that vessel owners and

operators will save $816 (136 SOLAS A liferafts x $6 per can opener)

for no longer needing can openers, because of meeting the jackknife

requirements.

---------------------------------------------------------------------------

\29\ We estimate the cost savings for only one can opener

because the use of a jackknife will only fulfill the replacement

requirement for one can opener.

\30\ We calculated this by taking the average of 10 can opener

products on the market that meet ISO 18813 requirements. The Coast

Guard will now require that can openers meet the standards of ISO

18813.

---------------------------------------------------------------------------

Emergency Water

The Coast Guard requires survival craft with SOLAS A packs be

stocked with 3 liters of water per person, and that lifeboats with

SOLAS B packs be stocked with 1.5 liters of water per person. We

estimate the average cost of Coast Guard-approved water to be $4 per

liter,\31\ while the cost of 1 liter of emergency water that meets the

ISO 18813 standard to be $3.\32\ The price difference between the Coast

Guard-approved water and water approved under ISO 18813 is $1 per

liter.\33\ This is the estimated additional cost of Coast Guard

approval, which is counted as cost savings. Emergency water expires and

will need to be replaced every 5 years; therefore, the Coast Guard

estimates that 20 percent of existing survival craft and 100 percent of

new survival craft will need to purchase emergency water annually.

---------------------------------------------------------------------------

\31\ We calculated this by taking the average of 14 Coast Guard-

approved emergency drinking water products on the market.

\32\ We calculated this by taking the average of 14 available

emergency drinking water products on the market that were compliant

with ISO 18813 only.

\33\ To calculate this, we took the average of emergency

drinking water prices that were Coast Guard-approved and subtracted

them from emergency drinking water prices that need only meet the

ISO standard.

---------------------------------------------------------------------------

We estimate that industry will save a total of $183,255 on an

annual basis (3,215 survival craft x 19 people per survival craft x 3

liters of water x $1 cost savings) for survival craft with SOLAS A

packs during Years 1 through 5 of implementation.\34\ To calculate this

cost savings, we took the 12,690 existing liferafts with SOLAS A packs

and 2,552 lifeboats with international voyage packs (see table 9) for a

total of 15,242 existing survival craft that are required to stock

emergency water. We then estimated that 20 percent (100 percent of

these survival craft / 5 years) or 3,048 survival craft [(12,690

liferafts x 20 percent) + (2,552 lifeboats x 20 percent)] will replace

their emergency water annually. Additionally, all 31 new lifeboats with

international packs and 136 new liferafts with SOLAS A packs (see table

10) are required to buy emergency water. We summed these totals to get

3,215 survival craft that will need to purchase emergency water on an

annual basis (3,048 existing survival craft + 31 new lifeboats + 136

new liferafts). Table 21 presents these cost savings.

---------------------------------------------------------------------------

\34\ We calculated this by taking the average of the survival

craft capacity for all survival craft. We retrieved this data from

the MISLE database in November 2020.

---------------------------------------------------------------------------

In Years 6 through 10, there will be more cost savings, because

vessels will have entirely replaced their survival craft equipment by

Year 6, as described earlier in this rule. Therefore, we estimate an

annual cost savings of about $192,774 [3,382 survival craft (3,215 +

167 new craft) x 19 people per survival craft x 3 liters of water x -$1

cost savings] for survival craft with SOLAS A packs. Table 22 presents

these cost savings.

[[Page 68289]]

Table 21--Total Cost Savings for Coast Guard Approval for Reduced Prices in Emergency Water for SOLAS A Packs in Years 1 Through 5

--------------------------------------------------------------------------------------------------------------------------------------------------------

Total        New                 Person per                 Total

liferafts   lifeboats     Total       life      Liters of     water      Cost of   Total cost

Years 1 through 5                         and         and      survival     saving       water     needed in     water      savings

lifeboats   liferafts     craft       craft     required     liters

(a)         (b)   (c) = (a)         (d)         (e)   (f) = (c)         (g)   (h) = (g)

+ (b)                             x (d) x                   x (f)

(e)

--------------------------------------------------------------------------------------------------------------------------------------------------------

Baseline................................................       3,048         167       3,215          19           3     183,255          $4    $733,020

Post-Regulatory.........................................       3,048         167       3,215          19           3     183,255           3     549,765

Change..................................................  ..........  ..........  ..........           0           0           0          -1    -183,255

--------------------------------------------------------------------------------------------------------------------------------------------------------

Note: Totals may not sum due to rounding.

Table 22--Total Cost Savings for Coast Guard Approval for Reduced Prices in Emergency Water for SOLAS A Packs in Years 6 Through 10

--------------------------------------------------------------------------------------------------------------------------------------------------------

Total        New                 Person per                 Total

liferafts   lifeboats     Total       life      Liters of     water      Cost of   Total cost

Years 6 through 10                         and         and      survival     saving       water     needed in     water      savings

lifeboats   liferafts     craft       craft     required     liters

(a)         (b)   (c) = (a)         (d)         (e)   (f) = (c)         (g)   (h) = (g)

+ (b)                             x (d) x                   x (f)

(e)

--------------------------------------------------------------------------------------------------------------------------------------------------------

Baseline................................................       3,215         167       3,382          19           3     192,774          $4    $771,096

Post-Regulatory.........................................       3,215         167       3,382          19           3     192,774           3     578,322

Change..................................................  ..........  ..........  ..........           0           0           0          -1    -192,774

--------------------------------------------------------------------------------------------------------------------------------------------------------

Note: Totals may not sum due to rounding.

We used the same methodology when calculating the number of SOLAS A

packs in Years 1 through 10 of implementation to estimate the total

costs savings for survival craft with SOLAS B packs. There are a total

of 283 existing lifeboats with SOLAS B packs (see table 9). We estimate

that 20 percent of these survival craft or 57 survival craft (283

lifeboats x 20 percent) will replace their emergency water annually.

Additionally, all 2 new lifeboats with SOLAS B packs are required to

buy emergency water, for a total of 59 survival craft (57 lifeboats + 2

new lifeboats) purchasing emergency water in Years 1 through 5. In

Years 6 through 10, the number of existing lifeboats will increase by 2

to account for the new vessels that will be built in Years 1 through 5

(59) for a total of 61 survival craft (59 existing survival craft + 2

new lifeboats).

The cost savings for survival craft with SOLAS B packs purchasing

emergency water will be approximately $1,682 (59 survival craft x 19

people per survival craft x 1.5 liters of water x -$1 cost savings) in

Years 1 through 5 and approximately $1,739 (61 survival craft x 19

people per survival craft x 1.5 liters of water x -$1 cost savings) in

Years 6 through 10. Table 23 presents these cost savings in Years 1

through 5 of implementation, and table 24 presents these cost savings

in Years 6 through 10 of implementation.

Table 23--Total Cost Savings for Coast Guard Approval for Reduced Prices in Emergency Water for SOLAS B Packs in Years 1 Through 5

--------------------------------------------------------------------------------------------------------------------------------------------------------

Person per

New         New      Total new     life      Liters of     Total                Total cost

Water Years 1-5                       liferafts   lifeboats   survival     saving       water       water       Cost       savings

craft       craft     required

(a)         (b)   (c) = (a)         (d)         (e)  (f) = [(c)         (g)   (h) = (f)

+ (b)                             x (d) x                   x (g)

(e)]

--------------------------------------------------------------------------------------------------------------------------------------------------------

Baseline................................................          57           2          59          19         1.5       1,682          $4      $6,728

Post-Regulatory.........................................          57           2          59          19         1.5       1,682           3       5,046

Change..................................................           0           0           0           0           0           0          -1      -1,682

--------------------------------------------------------------------------------------------------------------------------------------------------------

Note: Totals may not sum due to rounding.

Table 24--Total Cost Savings for Coast Guard Approval for Reduced Prices in Emergency Water for SOLAS B Packs in Years 6 Through 10

--------------------------------------------------------------------------------------------------------------------------------------------------------

Person per

New         New      Total new     life      Liters of     Total                Total cost

Water years 6-10                       liferafts   lifeboats   survival     saving       water       water       Cost       savings

craft       craft     required

(a)         (b)   (c) = (a)         (d)         (e)  (f) = [(c)         (g)   (h) = (f)

+ (b)                              x(d) x                   x (g)

(e)]

--------------------------------------------------------------------------------------------------------------------------------------------------------

Baseline................................................          59           2          61          19         1.5       1,739          $4      $6,956

Post-Regulatory.........................................          59           2          61          19         1.5       1,739           3       5,217

Change..................................................           0           0           0           0           0           0          -1      -1,739

--------------------------------------------------------------------------------------------------------------------------------------------------------

Note: Totals may not sum due to rounding.

Table 25 presents the total annualized cost savings to vessel

owners and operators from removing Coast Guard approval requirements

for emergency water. The Coast Guard estimates an annualized cost

savings of about

[[Page 68290]]

$188,923 with a 7-percent discount rate ($189,372 with 3-percent

discount rate).

Table 25--Total Cost Savings to Vessels From Removing Coast Guard Approval for Reduced Prices in Emergency Water

----------------------------------------------------------------------------------------------------------------

Cost savings    Cost savings                       Annualized cost savings

for vessels     for vessels     Total cost   -------------------------------

Year                 with SOLAS A    with SOLAS B       savings

packs           packs                            3%              7%

----------------------------------------------------------------------------------------------------------------

(a)                                          (b)             (c)     (d) = (b) +     (e) = (d) /     (f) = (d) /

(c)      1.03 \(a)\      1.07 \(a)\

----------------------------------------------------------------------------------------------------------------

1...............................        $183,255          $1,682        $184,937        $179,550        $172,838

2...............................        -183,255          -1,682        -184,937        -174,321        -161,531

3...............................        -183,255          -1,682        -184,937        -169,244        -150,964

4...............................        -183,255          -1,682        -184,937        -164,314        -141,088

5...............................        -183,255          -1,682        -184,937        -159,528        -131,858

6...............................        -192,774          -1,739        -194,513        -162,902        -129,612

7...............................        -192,774          -1,739        -194,513        -158,157        -121,133

8...............................        -192,774          -1,739        -194,513        -153,550        -113,208

9...............................        -192,774          -1,739        -194,513        -149,078        -105,802

10..............................        -192,774          -1,739        -194,513        -144,736         -98,881

----------------------------------------------------------------------------------------------------------------

Total.......................      -1,880,145         -17,105      -1,897,250       1,615,380      -1,326,915

----------------------------------------------------------------------------------------------------------------

Annualized..............  ..............  ..............  ..............        -189,372        -188,923

----------------------------------------------------------------------------------------------------------------

Note: Totals may not sum due to rounding.

First-Aid Kits

The Coast Guard is modifying the requirements for first-aid kits so

that all first-aid kits in survival craft must meet the standards

outlined in ISO 18813. In addition to removing the testing requirements

for the kits, this change modifies the required contents of first-aid

kits by removing the requirements for some items, adding additional

items, or changing the number of mandatory items. Since items within

the kits expire and need to be replaced, the change impacts both new

and existing vessels, including small passenger vessels described in

the Subchapters K and T section in this preamble. Table 26 highlights

these differences in the first-aid kit requirement. Due to the

differences in the first-aid kits, we estimate the cost of purchasing

each of the individual items in the kit.

Table 26--Crosswalk of First-Aid Kit Content Requirements

----------------------------------------------------------------------------------------------------------------

Number of items required

--------------------------------------------------------------------------

Item                    Lifeboats and rescue       Liferaft and IBA

boat  requirements    requirements under Sec.   ISO 18813 requirements

under Sec.   160.041-4          160.054-4

----------------------------------------------------------------------------------------------------------------

Adhesive Plasters....................  32 1-inch waterproof     16 1-inch waterproof     20 bandages in assorted

bandages.                bandages.                sizes.

Ammonia Inhalants....................  10.....................  10.....................  0.

Analgesic Medication.................  50 doses...............  20 doses...............  48 doses.

Antiseptic Preparations..............  10 iodine swabs........  10 iodine swabs........  10 applications.

Burn Preparations....................  0......................  0......................  12 applications.

Compression Bandage (for wounds).....  5 4-inch bandages 8 2-   1 4-inch bandage 4 2-    10 sterile bandages in

inch bandages.           inch bandages.           assorted sizes.

Compression Bandage (for securing      2 2-inch-by-6-yard       2 2-inch-by-6-yard       4 meters (4.4 yards) of

splints, dressings, etc.).             bandages.                bandages.                adhesive elastic

bandage.

Eye Dressing Packet..................  3......................  3......................  0.

Instructions.........................  1......................  1......................  1.

Sterile Gauze Compress...............  12 3-by-18-inch          4 3-by-18-inch           2.

compresses.              compresses.

Tourniquet, with forceps, scissors     1, 1, 1, and 12,         1, 1, 1, and 12,         0.

and pins.                              respectively.            respectively.

Triangle Bandage.....................  3 40-inch bandages.....  0......................  2.

Waterproof Container.................  1......................  1......................  1.

Wire Splint..........................  1......................  1......................  0.

----------------------------------------------------------------------------------------------------------------

First-Aid Kits for Lifeboats and Rescue Boats

We estimate that new vessels with lifeboats or rescue boats will

have a cost savings as a result of the changes to first-aid kits,

because we estimate that first-aid kits that meet the standard are $41

less expensive than Coast Guard-approved kits under approval series

160.041. We estimate that a total of 64 new lifeboats and rescue boats

will purchase a first-aid kit each year for a total costs savings of

approximately $2,624 (64 survival craft x $41 cost savings).

The Coast Guard is not requiring existing vessels to replace their

current kits; however, existing vessels must replace medication and

ointments within the kits by their expiration date.

[[Page 68291]]

Currently, vessels must replace their iodine swabs, pain relief

medication, and eye ointment, which we estimate costs about $19 per

kit.\35\ We calculated the cost per kit by taking the average price for

10 different iodine swab products, 12 different pain relief medication,

and 8 different eye ointments. Under this rule, these vessels will no

longer have to replace eye ointment, and will need to replace fewer

doses of pain relief medication. Additionally, vessel operators will be

able to replace iodine swabs with less expensive antiseptic

preparation. However, under this rule, vessels will incur an additional

cost from replacing the burn cream in the kits, as required by ISO

18813 shown in table 26. We estimate the cost of replacing these items

to be $19, meaning the change is cost-neutral to existing vessels with

lifeboat first-aid kits.\36\

---------------------------------------------------------------------------

\35\ ISO 18813 uses the specific language of Analgesic and

Ophthalmic when describing the medication in the first-aid kits.

Refer to the appendix titled ``Appendix B: Product Prices'' in the

docket folder for more information on product prices for these items

that comprise the first-aid kit.

\36\ The Coast Guard used the same price estimation for the

average cost of these items as the cost it would take to replace

them.

---------------------------------------------------------------------------

First-Aid Kits for Liferafts and IBAs

We estimate that first-aid kits that meet the requirements of ISO

18813 will be, on average, $1 less expensive than the Coast Guard-

approved kits for liferafts and IBAs.\37\ All 218 new liferafts and all

25 new IBAs will need to be equipped with the kits each year for an

annual cost savings of $243 (243 survival craft x -$1 cost saving).\38\

Liferaft first-aid kits are sealed in plastic bags, and most drugs

expire within a 2- to 3-year timeframe. Vessel owners and operators

have to replace the entire first-aid kit with a brand new kit after

using even one item. Once the packaging for the kit is opened, the

majority of items in it will have the same expiration date, not just

the individual item.\39\ Therefore, the Coast Guard estimates that

vessels will replace the items in their first-aid kits once they have

expired, every 2.5 years (average of 2 and 3 years), and this process

occurs during the annual servicing at an approved servicing facility.

---------------------------------------------------------------------------

\37\ The Coast Guard took the average price of six Coast Guard-

approved first-aid kits and subtracted it from an average of six

first-aid kits that met ISO standards.

\38\ There are 222 liferafts affected by this rule, but those

requiring SOLAS A and B packs (218 liferafts) will be required to

have first-aid kits.

\39\ We contacted a liferaft servicing firm to determine how the

expired items in liferaft and lifeboat first-aid kits are replaced.

---------------------------------------------------------------------------

We calculate that 40 percent (1 replacement every 2.5 years) of

vessels will replace these items annually. Forty percent of all

existing 2,612 IBAs and 22,377 liferafts [table 9 (sum of the totals

for SOLAS A and SOLAS B for inflatable liferafts columns)] is 9,996

survival craft [(2,612 IBAs x 40 percent) + (22,377 liferafts x 40

percent)]. Beginning in Year 3, the new survival craft from Year 1 will

need to replace their kits for a total of 10,239 survival craft (9,996

existing survival craft + 243 survival craft built in Year 1). In Year

4, the new survival craft from Year 2 will need to replace their kits,

but those from Year 1 will not need to do this, since they will have

replaced their kits in the prior year. Therefore, the total needing to

replace first-aid kits will still be 10,239 survival craft (9,996

existing survival craft + 243 survival craft built in Year 2). In Year

5, the survival craft built in Year 1 and Year 3 will replace their

kits for a total of 10,482 survival craft (9,996 existing survival

craft + 243 survival craft built in Year 1 + 243 survival craft built

in Year 3). This pattern continues over the 10-year analysis period. In

conclusion, we estimate the total annualized cost savings from removing

Coast Guard approval for liferaft first-aid kits will be $10,660 with a

7-percent discount rate as shown in table 27.

Table 27--Total Cost Savings to Vessels From Removing Coast Guard Approval Requirements for First-Aid Kits in Liferafts and IBAs

--------------------------------------------------------------------------------------------------------------------------------------------------------

Cost savings for replacement kits                               Annualized cost savings

-------------------------------------------------                 ---------------------------------

Year                   Cost savings   Total survival   Cost savings      Total cost       Total cost

to new vessels       craft            for         savings for        savings             3%               7%

replacing kits    replacement     replacements

(a)                                              (b)             (c)             (d)  (e) = (c) x (d)  (f) = (b) + (e)      (g) = (f) /      (h) = (f) /

1.03 \(a)\       1.07 \(a)\

--------------------------------------------------------------------------------------------------------------------------------------------------------

1...................................           -$243           9,996             -$1          -$9,996         -$10,239          -$9,941          -$9,569

2...................................            -243           9,996              -1           -9,996          -10,239           -9,651           -8,943

3...................................            -243          10,239              -1          -10,239          -10,482           -9,593           -8,556

4...................................            -243          10,239              -1          -10,239          -10,482           -9,313           -7,997

5...................................            -243          10,482              -1          -10,482          -10,725           -9,251           -7,647

6...................................            -243          10,482              -1          -10,482          -10,725           -8,982           -7,147

7...................................            -243          10,725              -1          -10,725          -10,968           -8,918           -6,830

8...................................            -243          10,725              -1          -10,725          -10,968           -8,658           -6,383

9...................................            -243          11,968              -1          -11,968          -11,211           -8,592           -6,098

10..................................            -243          11,968              -1          -11,968          -11,211           -8,342           -5,699

--------------------------------------------------------------------------------------------------------------------------------------------------------

Total...........................  ..............  ..............  ..............  ...............  ...............          -91,242          -74,870

--------------------------------------------------------------------------------------------------------------------------------------------------------

Annualized..................  ..............  ..............  ..............  ...............  ...............          -10,696          -10,660

--------------------------------------------------------------------------------------------------------------------------------------------------------

Note: Totals may not sum due to rounding.

First-Aid Kits for Small Passenger Vessels (Subchapter K and Subchapter

T)

This final rule will also remove Coast Guard approval requirements

for first-aid kits aboard small passenger vessels, which the Coast

Guard regulates under subchapters K and T. Small passenger vessels are

currently required to have first-aid kits approved under approval

series 160.041; therefore, we used the same cost savings estimates for

replacing first-aid kits in the section titled First-Aid Kits for

Lifeboats and Rescue Boats. This comes to $41 per first-aid kit. The

Coast Guard applied these estimates to small passenger vessels, which

will no longer need Coast Guard approval for the first-aid kits aboard

the vessels themselves. We estimate that there will be 40 new small

passenger vessels every year (see table 5). All of the 40 new passenger

vessels will need to be equipped with first-aid

[[Page 68292]]

kits each year, for an annual cost savings of $1,640.

Total Cost Savings to Vessel Owners and Operators

Table 28 presents the annual undiscounted total cost savings to

vessel owners and operators by equipment type, and table 29 presents

the total annualized cost savings. We estimate the total undiscounted

costs savings to vessel owners and operators at $2.85 million over a

10-year period of analysis, with an annualized total cost savings of

about $284,481 discounted at 7 percent ($284,966 with a 3-percent

discount rate).

BILLING CODE 9110-04-P

[[Page 68293]]

[GRAPHIC] [TIFF OMITTED] TR14NO22.002

BILLING CODE 9110-04-C

[[Page 68294]]

Table 29--Annualized Cost Savings to Vessel Owners and Operators

----------------------------------------------------------------------------------------------------------------

Annualized cost savings

Year                               Total cost    -------------------------------------

savings               3%                 7%

(a)                                                                    (b)   (c) = (b) / 1.03   (d) = (b) / 1.07

\(a)\              \(a)\

----------------------------------------------------------------------------------------------------------------

1......................................................          -$280,074          -$271,917          -$261,751

2......................................................          -$280,074          -$263,997          -$244,627

3......................................................          -$280,317          -$256,530          -$228,822

4......................................................          -$280,317          -$249,058          -$213,852

5......................................................          -$280,560          -$242,014          -$200,035

6......................................................          -$290,136          -$242,984          -$193,330

7......................................................          -$290,379          -$236,105          -$180,833

8......................................................          -$290,379          -$229,228          -$169,003

9......................................................          -$290,622          -$222,738          -$158,079

10.....................................................          -$290,622          -$216,250          -$147,737

Total..............................................        -$2,853,480        -$2,430,819        -$1,998,072

----------------------------------------------------------------------------------------------------------------

Annualized.....................................  .................          -$284,966          -$284,481

----------------------------------------------------------------------------------------------------------------

Note: Totals may not sum due to rounding.

Total Cost Savings to Industry

Table 30 presents the total annualized costs savings to industry

over the 10-year period of analysis. At a 7-percent discount rate, the

cost savings is approximately $303,805.

Table 30--Total Annualized Cost Savings to Industry

--------------------------------------------------------------------------------------------------------------------------------------------------------

Total cost         Total cost                               Annualized cost savings

Year                                savings to         savings to         Total cost    -------------------------------------

manufacturers *       vessels **          savings               3%                 7%

(a)                                                                      (b)                (c)    (d) = (b) + (c)   (e) = (d) / 1.03   (f) = (d) / 1.07

\(a)\              \(a)\

--------------------------------------------------------------------------------------------------------------------------------------------------------

1........................................................           -$19,324          -$280,074          -$299,398          -$290,678          -$279,811

2........................................................            -19,324           -280,074           -299,398           -282,211           -261,506

3........................................................            -19,324           -280,317           -299,641           -274,214           -244,596

4........................................................            -19,324           -280,317           -299,641           -266,227           -228,595

5........................................................            -19,324           -280,560           -299,884           -258,683           -213,813

6........................................................            -19,324           -290,136           -309,460           -259,168           -206,206

7........................................................            -19,324           -290,379           -309,703           -251,817           -192,867

8........................................................            -19,324           -290,379           -309,703           -244,482           -180,250

9........................................................            -19,324           -290,622           -309,946           -237,548           -168,590

10.......................................................            -19,324           -290,622           -309,946           -230,629           -157,561

Total................................................           -193,240         -2,853,480         -3,046,720         -2,595,657         -2,133,796

--------------------------------------------------------------------------------------------------------------------------------------------------------

Annualized.......................................  .................  .................  .................           -304,290           -303,805

--------------------------------------------------------------------------------------------------------------------------------------------------------

Note: Totals may not sum due to rounding.

* Table 19.

** Table 28.

Federal Government Cost Savings

We estimate that this rule will reduce costs to the Federal

Government, since the Coast Guard will no longer review COA

applications, application renewals, or inspection reports for the

equipment that is subject to this rule. The Coast Guard does not

anticipate that this rule will generate any cost savings from vessels

inspections, as this rule does not modify any inspection requirements.

Equipment Approval

In addition to generating a cost savings to industry by removing

COA application requirements, this rule will also create a cost savings

to the Federal Government, as Coast Guard staff will no longer review

new COA applications and renewals. We estimate that it takes 24 hours

of a GS-14's time to review each new application and 4 hours to review

each renewal.\40\ We estimate the cost of reviewing a new application

at $2,672 (rounded) per applicant (24 hours x $111.34), and the cost

for reviewing a renewal application at $445(rounded) per renewal (4

hours x $111.34). In table 31, the cost of reviewing a new application

is captured in column (b) and the cost of a renewal application is

captured in column (d). In total, we estimate the Federal Government

will save $4,735 each year, due to this rule removing the requirements

of having to review COA applications.

---------------------------------------------------------------------------

\40\ This is based on information from the subchapter Q ICR. For

the wage rate, $111.34, please see the Wages section of this RA.

[[Page 68295]]

Table 31--Annual Cost Savings to Federal Government for No Longer Having To Review New and Renewal Certificate of Approval Applications

--------------------------------------------------------------------------------------------------------------------------------------------------------

New applications              Renewal applications

----------------------------------------------------------------  Total change

Equipment                            Approval      Total number                    Total number                      in cost =

series            of          Total cost          of          Total cost      total cost

applications                    applications                       savings

(a)  (b) = (a) x [-             (c)  (d) = (c) x [-     (e) = (b) +

$2,672]                           $445]             (d)

--------------------------------------------------------------------------------------------------------------------------------------------------------

Bilge pump..............................................         160.044            0.09           -$240            0.60           -$267           -$507

Compass.................................................         160.014            0.09            -240            0.60            -267            -507

First-aid kit for Lifeboats.............................         160.041            0.15            -401               1            -445            -846

First-aid kit for Liferafts.............................         160.054            0.15            -401               1            -445            -846

Fishing kit.............................................         160.061            0.03             -80            0.20             -89            -169

Hatchet.................................................         160.013            0.03             -80            0.20             -89            -169

Jackknife...............................................         160.043            0.03             -80            0.20             -89            -169

Mirror, Signaling.......................................         160.020            0.06            -160             0.4            -178            -338

Sea anchor..............................................         160.019            0.03             -80            0.20             -89            -169

Water...................................................         160.026            0.18            -481            1.20            -534           -1015

-----------------------------------------------------------------------------------------------

Total...............................................  ..............  ..............          -2,243  ..............          -2,492          -4,735

--------------------------------------------------------------------------------------------------------------------------------------------------------

Note: Totals may not sum due to rounding.

Laboratory Inspections

The Coast Guard currently requires manufacturers of some equipment

to submit an annual report with the results of laboratory inspections,

allowing the Coast Guard to ensure the production stock of the

equipment will be identical to those originally tested and approved by

the Coast Guard. This rule removes this reporting requirement for

equipment that is now self-certified by the manufacturer. We were

unable to obtain data about the costs related to laboratory

inspections.

We estimate that it takes approximately 2 hours of a GS-14 senior

engineer's time to review each report, costing $223 (2 hours x

$111.34). Table 32 presents the total annual cost saving to the Federal

Government for no longer having to review laboratory inspection

reports. We estimate these cost savings will be $5,352 per year.

Table 32--Annual Federal Government Cost Savings for No Longer Having to Review Laboratory Inspection Records

--------------------------------------------------------------------------------------------------------------------------------------------------------

Baseline scenario           Post-regulatory scenario      Total change

Approval    ----------------------------------------------------------------    in cost =

Equipment                             series                                                                        total cost

Total products    Total cost    Total products    Total cost        savings

(a)     (b) = (a) x             (c)     (d) = (c) x     (e) = (d) -

$223                            $223             (b)

--------------------------------------------------------------------------------------------------------------------------------------------------------

Bilge pump..............................................         160.044               3            $669               0              $0           -$669

Compass.................................................         160.014               3             669               0               0            -669

First-aid kit for Lifeboats.............................         160.041               5           1,115               0               0          -1,115

First-aid kit for Liferafts.............................         160.054               5           1,115               0               0          -1,115

Mirror, Signaling.......................................         160.020               2             446               0               0            -446

Water...................................................         160.026               6           1,338               0               0          -1,338

-----------------------------------------------------------------------------------------------

Total...............................................  ..............              24           5,352               0               0           5,352

--------------------------------------------------------------------------------------------------------------------------------------------------------

Note: Totals may not sum due to rounding.

Total Federal Government Savings

Table 33 presents the total annual cost savings to the Federal

Government. In total, the Coast Guard estimates this rule to generate a

cost savings of approximately $10,087 per year.

Table 33--Total Annual Cost Savings to the Federal Government

----------------------------------------------------------------------------------------------------------------

New           Renewed         Avoided

Equipment                Approval      applications    applications     inspection      Total cost

series          avoided         avoided         reports         savings

(a)             (b)             (c)     (d) = (a) +

(b) + (c)

----------------------------------------------------------------------------------------------------------------

Bilge pump......................         160.044           -$240           -$267           -$669         -$1,176

Compass.........................         160.014            -240            -267            -669          -1,176

First-aid kit for Lifeboats.....         160.041            -401            -445          -1,115          -1,961

First-aid kit for Liferafts.....         160.054            -401            -445          -1,115          -1,961

[[Page 68296]]

Fishing kit.....................         160.061             -80             -89               0            -169

Hatchet.........................         160.013             -80             -89               0            -169

Jackknife.......................         160.043             -80             -89               0            -169

Mirror, Signaling...............         160.020            -160            -178            -446            -784

Sea anchor......................         160.019             -80             -89               0            -169

Water...........................         160.026            -481            -534          -1,338          -2,353

-------------------------------------------------------------------------------

Total.......................  ..............          -2,243          -2,492          -5,352         -10,087

----------------------------------------------------------------------------------------------------------------

Note: Totals may not sum due to rounding.

Change in Safety

Many of the current Coast Guard type approval requirements for

survival craft equipment were developed in the 1950s and 1960s and have

not been significantly updated since they were initially published.

Upon a thorough review of these requirements, Coast Guard enforcement

procedures, current maritime industry practice, and the availability of

new international standards, we have determined that the additional

scrutiny of the Coast Guard type approval does not increase or decrease

the safety for the equipment subject to this rule. For these nine types

of survival craft equipment, the current Coast Guard type approval

requirements are outdated and overly prescriptive. Therefore, the Coast

Guard anticipates that by having equipment meet consensus standards, as

opposed to Coast Guard standards, there will be no decrease in the

level of safety in the maritime environment.

No Cost Changes

This rule will also implement several changes with no cost impacts.

The vast majority of these changes are the result of modifying the

current lifeboat equipment requirements for sailing school vessels as

stated in Sec.  169.527 to align them with the requirements stated in

Sec.  199.175. Table 34 summarizes these changes.

Table 34--Summary of Regulatory Changes With No Cost Impacts

----------------------------------------------------------------------------------------------------------------

CFR subpart/        Affected

Equipment                section(s)        population            Changes           Basis for no cost

----------------------------------------------------------------------------------------------------------------

Bailer.......................  Sec.   169.529(a)  New U.S.-flagged  Removes requirements   This is an

Sailing School    that bailers in        administrative

Vessels with      lifeboats on sailing   change that allows

Lifeboats.        school vessels meet    the Coast Guard to

the requirements of    consolidate its

Sec.   169.529(a)      survival craft

and instead, they      equipment standards,

must meet the          and the requirements

requirements of Sec.   of Sec.  Sec.

199.175(b)(1).       169.529(a) and

199.175(b)(1) are

identical.

Boathooks....................  Sec.   169.529(c)  New U.S.-flagged  Removes requirements   Sections 169.529(c)

Sailing School    that boathooks in      and 199.175(b)(3)

Vessels with      lifeboats on sailing   set different

Lifeboats.        school vessels meet    standards for

the prescribed         boathooks; however,

design requirements    only new U.S.-

of Sec.   169.529(c)   flagged sailing

and instead, they      school vessels will

must meet the          be impacted by the

requirements of Sec.   change, and the

199.175(b)(3) and    Coast Guard

be designed to         estimates that no

minimize the           new U.S.-flagged

possibility of         sailing school

damage.                vessels will be

built during the

analysis period.

Can Openers..................               Sec.  All U.S.-flagged  Can openers must meet  ISO 18813 requires

199.175(b)(5)   Vessels with      the standards of ISO   that can openers in

Lifeboats or      18813.                 liferafts be of the

Liferafts with                           safety type. The

SOLAS A packs.                           Coast Guard

estimates that all

liferafts are

currently equipped

with either a safety

can opener or a can

opener within the

jackknife;

therefore, this

change poses no

additional cost to

industry.

Cover, Protecting............               Sec.  New U.S.-flagged  Fully enclosed         Only new U.S.-flagged

169.529(ll)   Sailing School    lifeboats on sailing   sailing school

Vessels with      school vessels do      vessels will be

Lifeboats.        not need to be         impacted by the

equipped with a        change, and the

cover.                 Coast Guard

estimates that no

new U.S.-flagged

sailing school

vessels will be

built during the

analysis period. In

addition, fully

enclosed lifeboats

do not require a

cover; therefore, it

is likely they are

not equipped with

one under the

baseline.

[[Page 68297]]

Ditty Bag....................  Sec.   169.529(f)  New U.S.-flagged  Motor-propelled        Only new U.S.-flagged

Sailing School    lifeboats on sailing   sailing school

Vessels with      school vessels no      vessels will be

Lifeboats.        longer need to carry   impacted by the

a ditty bag.           change, and the

Coast Guard

estimates that no

new U.S.-flagged

sailing school

vessels will be

built during the

analysis period.

Drinking Cups................  Sec.   169.529(g)  New U.S.-flagged  Removes requirements   This is an

Sailing School    that drinking cups     administrative

Vessels with      in lifeboats on        change that allows

Lifeboats.        sailing school         the Coast Guard to

vessels meet the       consolidate its

requirements of Sec.   survival craft

169.529(g) and       equipment standards,

instead, they must     and the requirements

meet the               of Sec.  Sec.

requirements of Sec.   169.529(g) and

199.175(b)(8).       199.175(b)(8) are

identical.

Fire Extinguisher............               Sec.  All New U.S.-     Updates fire           This change does not

169.529(h),   flagged Vessels   extinguisher rating    require fire

Sec.   with IBAs,        names from B-C, size   extinguishers meet

199.175(b)(9)   Liferafts,        II to 40-B to match    any different

Lifeboats, or     other regulatory       requirements as laid

Rescue Boats.     text in title 46 of    out in the final

the CFR.               rule,

``Harmonization of

Standards for Fire

Protection,

Detection and

Extinguishing

Equipment'' (81 FR

482200 July 22,

2016), only that

they have a label.

A review of portable

marine fire

extinguishers found

that both the Coast

Guard and UL ratings

are currently

provided for each

product.

First-Aid Kits...............     Sec.   121.710  All U.S.-flagged  All medicinal          The Coast Guard

Sec.   160.010-   Vessels with      products within the    estimates that,

3(e)(7)(ii)   IBAs, Liferafts   first-aid kits must    under the baseline,

Subpart 160.041   with a SOLAS A    use active             all medicinal

Subpart 160.054   or B pack,        ingredients that       products meet U.S.

Sec.   160.151-   Lifeboats, or     conform to OTC drug    OTC drug standards.

21(h)   Rescue Boats.     regulations set out    The Coast Guard did

Sec.   169.529(i)   All small         in 21 CFR part 330.    an extensive inquiry

Sec.   184.710   passenger                                to ensure that the

Sec.   199.050(c)   vessels in                               medicinal products

Sec.   Subchapters K                            were FDA compliant.

199.175(b)(10)   and T.

Flashlights..................  Sec.   169.529(j)  New U.S.-flagged  Removes requirement    This is an

Sailing School    that flashlights in    administrative

Vessels with      lifeboats on sailing   change that allows

Lifeboats.        school vessels meet    the Coast Guard to

the prescribed         consolidate its

design requirements    survival craft

of Sec.   169.529(j)   equipment standards.

and instead, they

must meet the

requirements of Sec.

199.175(b)(12) and

be constructed and

marked according to

the American Society

for Testing and

Materials' ASTM

F1014 standard

already incorporated

by reference in that

section.

Heaving Lines................  Sec.   169.529(l)  New U.S.-flagged  Removes requirement    This is an

Sailing School    that heaving lines     administrative

Vessels with      on lifeboats on        change that allows

Lifeboats.        sailing school         the Coast Guard to

vessels meet the       consolidate its

requirements of Sec.   survival craft

169.529(l), and      equipment standards,

instead, they must     and the requirements

meet the               of Sec.  Sec.

requirements of Sec.   169.529(l) and

199.175(b)(14).      199.175(b)(14) are

identical.

Ladder.......................  Sec.   169.529(n)  New U.S.-flagged  Removes requirement    This is an

Sailing School    that ladders on        administrative

Vessels with      lifeboats on sailing   change that allows

Lifeboats.        school vessels meet    the Coast Guard to

the requirements of    consolidate its

Sec.   169.529(n),     survival craft

and instead, they      equipment standards,

must meet the          and the requirements

requirements of Sec.   of Sec.  Sec.

199.175(b)(18).      169.529(n) and

199.175(b)(18) are

identical.

Lanterns.....................  Sec.   169.529(o)  New U.S.-flagged  Removes requirement    Only new U.S.-flagged

Sailing School    that lifeboats on      sailing school

Vessels with      sailing school         vessels are impacted

Lifeboats.        vessels carry          by the change, and

lanterns.              the Coast Guard

estimates that no

new U.S.-flagged

sailing school

vessels will be

built during the

analysis period.

Lifelines....................  Sec.   169.529(p)  New U.S.-flagged  Removes lifeline       This is an

Sailing School    standards from Sec.    administrative

Vessels with       169.529(p).           change, as lifelines

Lifeboats.                               are not survival

craft equipment and

are, instead,

regulated as part of

the lifeboat design

requirements under

Sec.   160.135-7.

[[Page 68298]]

Life Preservers..............  Sec.   169.529(q)  New U.S.-flagged  Removes requirement    Only new U.S.-flagged

Sailing School    that lifeboats on      sailing school

Vessels with      sailing school         vessels will be

Lifeboats.        vessels carry two      impacted by the

additional life        change, and the

preservers in their    Coast Guard

lifeboat.              estimates that no

new U.S.-flagged

sailing school

vessels will be

built during the

analysis period.

Lockers......................  Sec.   169.529(r)  New U.S.-flagged  Removes requirement    Only new U.S.-flagged

Sailing School    that lifeboats on      sailing school

Vessels with      sailing school         vessels will be

Lifeboats.        vessels have lockers   impacted by the

for the storage of     change, and the

small items.           Coast Guard

estimates that no

new U.S.-flagged

sailing school

vessels will be

built during the

analysis period.

Mast and Sail................  Sec.   169.529(s)  New U.S.-flagged  Clarifies that motor-  Only new U.S.-flagged

Sailing School    propelled lifeboats    sailing school

Vessels with      on sailing school      vessels will be

Lifeboats.        vessels do not need    impacted by the

to carry a mast or     change, and the

sails.                 Coast Guard

estimates that no

new U.S.-flagged

sailing school

vessels will be

built during the

analysis period.

In addition,

motorized boats do

not require a mast

or sails; therefore,

they are not

equipped with them

under the baseline.

Matches......................  Sec.   169.529(t)  New U.S.-flagged  Removes requirement    Only new U.S.-flagged

Sailing School    that lifeboats on      sailing school

Vessels with      sailing school         vessels will be

Lifeboats.        vessels carry          impacted by the

matches.               change, and the

Coast Guard

estimates that no

new U.S.-flagged

sailing school

vessels will be

built during the

analysis period.

Oars.........................  Sec.   169.529(v)  New U.S.-flagged  Removes requirement    This is an

Sailing School    that oars on           administrative

Vessels with      lifeboats on sailing   change that allows

Lifeboats.        school vessels meet    the Coast Guard to

the requirements of    consolidate its

Sec.   169.529(v),     survival craft

and instead, they      equipment standards,

must meet the          and the requirements

requirements of Sec.   of Sec.  Sec.

199.175(b)(20). In   169.529(v) and

addition, the Coast    199.175(b)(20) are

Guard is modifying     identical. There are

the number of          no cost savings

required oars from     because there are no

four rowing and one    sailing school

steering, to the       vessels with

number required by     lifeboats.

the manufacturer.     In addition, only new

U.S.-flagged sailing

school vessels will

be impacted by the

change, and the

Coast Guard

estimates that no

new U.S.-flagged

sailing school

vessels will be

built during the

analysis period.

Oil, Illuminating............  Sec.   169.529(w)  New U.S.-flagged  Removes requirement    Only new U.S.-flagged

Sailing School    that lifeboats on      sailing school

Vessels with      sailing school         vessels will be

Lifeboats.        vessels carry          impacted by the

illuminating oil for   change, and the

lanterns.              Coast Guard

estimates that no

new U.S.-flagged

sailing school

vessels will be

built during the

analysis period.

Oil, Storm...................  Sec.   169.529(x)  New U.S.-flagged  Removes requirement    Only new U.S.-flagged

Sailing School    that lifeboats on      sailing school

Vessels with      sailing school         vessels will be

Lifeboats.        vessels carry storm    impacted by the

oil to calm the seas.  change, and the

Coast Guard

estimates that no

new U.S.-flagged

sailing school

vessels will be

built during the

analysis period.

Painters.....................  Sec.   169.529(y)  New U.S.-flagged  Removes requirement    This is an

Sailing School    that painters on       administrative

Vessels with      lifeboats on sailing   change that allows

Lifeboats.        school vessels meet    the Coast Guard to

the requirements of    consolidate its

Sec.   169.529(y),     survival craft

and instead, they      equipment standards,

must meet the          and the requirements

requirements of Sec.   of Sec.  Sec.

199.175(b)(21).      169.529(n) and

199.175(b)(18) are

identical.

Plug.........................  Sec.   169.529(z)  New U.S.-flagged  Removes plug           This is an

Sailing School    standards from Sec.    administrative

Vessels with       169.529(z).           change, as plugs are

Lifeboats.                               not survival craft

equipment and are,

instead, regulated

as part of the

lifeboat design

requirements under

Sec.   160.135-7

[[Page 68299]]

Provisions...................    Subpart 160.046  All               Adds to the scope:     This is an

manufacturers     emergency provisions   administrative

of Coast Guard-   approved to be         change, as this rule

approved          carried in lifeboats   will update Sec.

provisions.       and liferafts. These   199.175(b)(22) and

provisions meet the    add regulatory text

IMO recommendations    to subpart 160.046

for emergency food     stating that the

rations.               provisions or food

rations must comply

with ISO 18813

paragraph 4.31,

which is the same as

the current

standard.

Rowlocks.....................               Sec.  New U.S.-flagged  Removes requirement    This is an

169.529(bb)   Sailing School    that rowlocks on       administrative

Vessels with      lifeboats on sailing   change that allows

Lifeboats.        school vessels meet    the Coast Guard to

the requirements of    consolidate its

Sec.   169.529(bb)     survival craft

and instead, they      equipment standards,

must meet the          and the requirements

requirements of Sec.   of Sec.  Sec.

199.175(b)(20).      169.529(bb) and

199.175(b)(20) are

identical.

Rudder and Tiller............               Sec.  New U.S.-flagged  Removes rudder and     This is an

169.529(cc)   Sailing School    tiller standards       administrative

Vessels with      from Sec.              change, as Sec.

Lifeboats.        169.529(cc), which     169.035-3(f) was

state the rudder and   removed previously

tiller must be         from the CFR, and

constructed            the section no

according to Sec.      longer exists.

169.035-3(f).

Signals, Distress Floating                  Sec.  New and Existing  Removes requirement    The change will apply

Orange Smoke.                       169.529(ee)   U.S.-flagged      that distress          to both new U.S.-

Sailing School    floating orange        flagged sailing

Vessels with      smoke signals on       school vessels with

Lifeboats.        lifeboats on sailing   lifeboats, and

school vessels meet    existing sailing

the requirements of    school vessels with

Sec.   169.529(ee),    lifeboats, as these

and instead, they      vessels will have to

must meet the          replace their smoke

requirements of Sec.   signals after they

199.175(b)(30).      expire.

The Coast Guard

estimates that no

new U.S.-flagged

sailing school

vessels will be

built during the

analysis period. In

addition, there are

no existing sailing

school vessels with

lifeboats;

therefore, no

existing vessels

will be impacted by

the change.

Signals, Distress Red Hand                  Sec.  All U.S.-flagged  Removes requirement    The change will apply

Flare.                              169.529(ff)   Sailing School    that distress red      to both new U.S.-

Vessels with      hand flare signals     flagged sailing

Lifeboats.        on lifeboats on        school vessels with

sailing school         lifeboats, and

vessels meet the       existing sailing

requirements of Sec.   school vessels with

169.529(ff), and     lifeboats, as these

instead, they must     vessels will have to

meet the               replace their smoke

requirements of Sec.   signals after they

199.175(b)(31).      expire.

The Coast Guard

estimates that no

new U.S.-flagged

sailing school

vessels will be

built during the

analysis period. In

addition, there are

no existing sailing

school vessels with

lifeboats;

therefore, no

existing vessels

will be impacted by

the change.

Signals, Distress Red                       Sec.  All U.S.-flagged  Removes requirement    The change will apply

Parachute Flare.                    169.529(gg)   Sailing School    that distress red      to both new U.S.-

Vessels with      parachute flares on    flagged sailing

Lifeboats.        lifeboats on sailing   school vessels with

school vessels meet    lifeboats and

the requirements of    existing sailing

Sec.   169.529(gg),    school vessels with

and instead, they      lifeboats, as these

must meet the          vessels will have to

requirements of Sec.   replace their smoke

199.175(b)(32).      signals after they

expire.

The Coast Guard

estimates that no

new U.S.-flagged

sailing school

vessels will be

built during the

analysis period. In

addition, there are

no existing sailing

school vessels with

lifeboats;

therefore, no

existing vessels

will be impacted by

the change.

Table of Lifesaving Signals..               Sec.  New U.S.-flagged  Removes requirement    This is an

169.529(mm)   Sailing School    that table of          administrative

Vessels with      lifesaving signals     change that allows

IBAs,             on lifeboats on        the Coast Guard to

Liferafts,        sailing school         consolidate its

Lifeboats, or     vessels meet the       survival craft

Rescue Boats.     requirements of Sec.   equipment standards,

169.529(mm), and     and the requirements

instead, they must     of Sec.  Sec.

meet the               169.529(mm) and

requirements of Sec.   199.175(b)(36) are

199.175(b)(36).      identical.

[[Page 68300]]

Tool Kit.....................               Sec.  New U.S.-flagged  Removes requirements   This is an

169.529(hh)   Sailing School    that toolkits on       administrative

Vessels with      lifeboats on sailing   change that allows

IBAs,             school vessels meet    the Coast Guard to

Liferafts,        the requirements of    consolidate its

Lifeboats, or     Sec.   169.529(hh),    survival craft

Rescue Boats.     and instead, they      equipment standards,

must meet the          and the requirements

requirements of Sec.   of Sec.  Sec.

199.175(b)(38).      169.529(hh) and

199.175(b)(38) are

identical.

Whistle......................               Sec.  New U.S.-Flagged  Removes requirement    This is an

169.529(jj)   Sailing School    that whistles on       administrative

Vessels with      lifeboats on sailing   change that allows

IBAs,             school vessels meet    the Coast Guard to

Liferafts,        the requirements of.   consolidate its

Lifeboats, or    Sec.   169.529(jj),     survival craft

Rescue Boats.     and instead, they      equipment standards,

must meet the          and the requirements

requirements of Sec.   of Sec.  Sec.

199.175(b)(41).      169.529(jj) and

199.175(b)(41) are

identical.

----------------------------------------------------------------------------------------------------------------

Total Cost Savings

Table 35 presents the total annualized cost savings of this final

rule to both industry and the Federal Government for the 10-year period

of analysis. The Coast Guard estimates an annualized cost savings of

approximately $314,377 with a 3-percent discount rate, and $313,892

with a 7-percent discount rate.

Table 35--Total Annualized Cost Savings to Industry and Federal Government

----------------------------------------------------------------------------------------------------------------

Total cost                        Annualized cost savings

Total cost      savings to      Total cost   -------------------------------

Year                  savings to        federal         savings

industry *     government **                        3%              7%

(a)                                          (b)             (c)     (d) = (b) +     (e) = (d) /     (f) = (d) /

(c)      1.03 \(a)\      1.07 \(a)\

----------------------------------------------------------------------------------------------------------------

1...............................       -$299,398        -$10,087       -$309,485       -$300,471       -$289,238

2...............................        -299,398         -10,087        -309,485        -291,719        -270,316

3...............................        -299,641         -10,087        -309,728        -283,445        -252,830

4...............................        -299,641         -10,087        -309,728        -275,189        -236,290

5...............................        -299,884         -10,087        -309,971        -267,384        -221,005

6...............................        -309,460         -10,087        -319,547        -267,616        -212,928

7...............................         309,703          10,087         319,790         260,019         199,149

8...............................        -309,703         -10,087        -319,790        -252,445        -186,121

9...............................        -309,946         -10,087        -320,033        -245,279        -174,077

10..............................        -309,946         -10,087        -320,033        -238,135        -162,689

-------------------------------------------------------------------------------

Total.......................      -3,046,720        -100,870      -3,147,590      -2,681,701      -2,204,643

Annualized..............  ..............  ..............  ..............         314,377         313,892

----------------------------------------------------------------------------------------------------------------

Note: Totals may not sum due to rounding.

* Table 30.

** Table 33.

Discussion of Alternatives

When creating this rule, the Coast Guard considered four

alternatives, one of which was suggested by public comment. In this

section, we examine how the cost of the rulemaking changes with each

alternative.

Alternative 1: No Action

Using this alternative, the Coast Guard will accept the status quo

and not replace the current approval requirements with an international

consensus standard. This alternative will not harmonize Coast Guard

standards with industry consensus standards, nor reduce the burden to

industry. This will not incur approximately $314,000 in annual cost

savings with no estimated benefits.

Alternative 2: Preferred Alternative--Remove the Need for Coast Guard

Approval

Using this alternative, the Coast Guard will implement the changes

regarding the removal of Coast Guard approval standards. This will lead

to an estimated $314,000 in annual cost savings without any estimated

reduction in benefits, as this analysis shows.

Alternative 3: Remove the Need for Coast Guard Approval and Marking

Requirements

Under this alternative, the Coast Guard will implement the changes

in the preferred alternative, but will, in addition, remove the

requirement that equipment be marked to indicate it meets ISO 25862,

ISO 17339, or ISO 18813. This will lead to an additional annual cost

savings of approximately $397,433. We estimate this by multiplying

254,765 pieces of equipment by $1.56 (allowing 0.06 hours x $26

production rate per hour for the time and cost to mark each piece of

equipment). This will lead to a total cost savings of $711,433, which

we calculated by adding the additional savings from no markings

($397,433) to the total estimated cost savings of this rule, as shown

in alternative 2 ($314,000).

We rejected this alternative for the preferred alternative, since

eliminating the markings will make it impossible for the Coast Guard to

verify if equipment complies with regulations. This alternative could

potentially lead to a decrease in safety, if vessel owners and

operators purchased non- ISO-

[[Page 68301]]

compliant products that were not sufficiently safe or reliable for

usage on board a survival craft. The potential for the additional

burden on the Coast Guard to research and ascertain the compliance

status of a piece of survival craft equipment could lead to much more

significant costs than the current additional cost of $397,433 from

marking equipment.

Alternative 4: Require Manufacturers To Cover the Cost of a COA

The Coast Guard received a public comment suggesting that the

manufacturers should cover the cost of COAs. We interpreted this

comment as suggesting that manufacturers should reimburse the Coast

Guard for the estimated $2,672 in cost per new COA and the $445 in cost

per renewal COA. This alternative will introduce a transfer to cover

the Coast Guard's cost of the approvals. Because this alternative will

introduce a transfer, there will be no net cost saving from this

action. Instead, manufacturing firms will experience an extra $2,672 in

costs each time they apply for a new COA and an extra $445 in costs

each time they try to renew a COA. By raising the costs of approval,

the Coast Guard will be increasing entry barriers to manufacturing PFD

devices.

Additionally, because our preferred alternative removes the

requirements for a COA on nine types of equipment, this alternative

will decrease cost savings by both the government cost savings of

$4,735 and the industry cost savings of $336. Because this alternative

will not decrease costs, and increases the entry barrier faced by

manufacturing firms, we rejected this alternative.

B. Small Entities

Under the Regulatory Flexibility Act, 5 U.S.C. 601-612, we have

considered whether this rule will have a significant economic impact on

a substantial number of small entities. The term ``small entities''

comprises small businesses, not-for-profit organizations that are

independently owned and operated and are not dominant in their fields,

and governmental jurisdictions with populations of less than 50,000.

The Coast Guard expects that this rule will not have a significant

economic impact on small entities. We expect this rule to result in net

cost savings to regulated entities.

We added two years of data to our data analysis in the NPRM;

however, the random sample of our dataset is still valid. Using the

same number of companies we used in the proposed rule for the final

rule, we estimate there to be 11,139 unique vessel operators and 16

equipment manufacturers affected by this rule. For this analysis, we

presumed any company for which we were not able to find Small Business

Administration (SBA) size data to be a small entity. An estimated 94

percent of the regulated entities (including the companies without SBA

size data) are considered to be small by SBA industry size standards.

Using MISLE data, the Coast Guard estimates there to be 11,155 unique

companies affected in this rule, of which 10,487 (0.94 x 11,155) are

small. We estimate that the average costs to equipment manufacturers

will be reduced by $1,418 per year, and the average costs to vessel

owners and operators will be reduced by $60 per year as a result of

removing Coast Guard approval for the equipment subject to this

rulemaking. We found that all small vessel operators and small

equipment manufacturers impacted by this rule will have a cost savings

less than 1 percent of their annual revenue. No small governmental

jurisdictions will be impacted by this rule.

Therefore, the Coast Guard certifies under 5 U.S.C. 605(b) that

this rule will not have a significant economic impact on a substantial

number of small entities.

C. Assistance for Small Entities

Under section 213(a) of the Small Business Regulatory Enforcement

Fairness Act of 1996, Public Law 104-121, we offer to assist small

entities in understanding this rule so that they can better evaluate

its effects on them and participate in the rulemaking. The Coast Guard

will not retaliate against small entities that question or complain

about this rule or any policy or action of the Coast Guard.

Small businesses may send comments on the actions of Federal

employees who enforce, or otherwise determine compliance with, Federal

regulations to the Small Business and Agriculture Regulatory

Enforcement Ombudsman and the Regional Small Business Regulatory

Fairness Boards. The Ombudsman evaluates these actions annually and

rates each agency's responsiveness to small business. If you wish to

comment on actions by employees of the Coast Guard, call 1-888-REG-FAIR

(1-888-734-3247).

D. Collection of Information

This rule calls for a revision to an approved collection of

information under the Paperwork Reduction Act of 1995, 44 U.S.C. 3501-

3520. As defined in 5 CFR 1320.3(c), ``collection of information''

comprises reporting, recordkeeping, monitoring, posting, labeling, and

other similar actions. The title and description of the information

collections, a description of those who must collect the information,

and an estimate of the total annual burden follow. The estimate covers

the time for reviewing instructions, searching existing sources of

data, gathering and maintaining the data needed, and completing and

reviewing the collection.

Title: Title 46 CFR Subchapter Q: Lifesaving, Electrical,

Engineering and Navigation Equipment, Construction and Materials &

Marine Sanitation Devices (33 CFR 159).

OMB Control Number: 1625-0035.

Summary of the Collection of Information: The Coast Guard currently

collects information from lifesaving equipment manufacturers under 46

CFR chapter I, subchapter Q. The current ICR, 201811-1625-005 (OMB

Control Number 1625-0035), accounts for the following collections of

information: New Approval Applications, Renewal Approval Applications,

Manufacturer Recordkeeping, Servicing Facility Recordkeeping, Servicing

Facility Problem Reports, Instruction Materials, Markings, Production

Tests and Laboratory Inspections, and Independent Laboratory

Applications and Recognized Laboratory Applications.

Need for Information: The Coast Guard needs this information to

ensure that the manufactured safety equipment meets minimum levels of

performance safety and helps prevent death, injuries, and property

damage associated with commercial maritime operations.

Proposed Use of Information: The Coast Guard uses the technical

plans, drawings, specifications, instruction materials, and markings to

determine compliance with the technical regulatory requirements for

each piece of equipment. Independent laboratory reports ensure that

product and material testing complies with the applicable Coast Guard

regulations. Production testing reports ensure that the production

stock of the equipment is identical to the stock that was originally

tested and approved by the Coast Guard. Independent and recognized

laboratory applications ensure that the laboratories have the technical

capabilities to conduct the required testing and are independent for

the organizations whose products they will test.

Description of the Respondents: The respondents are manufacturers

of the safety equipment subject to Coast Guard approval, accepted and

recognized independent laboratories that conduct testing of the

equipment, and liferaft servicing facilities.

Number of Respondents: The Coast Guard estimates there will be 856

[[Page 68302]]

respondents, comprised of 480 equipment manufacturers, 233 liferaft

servicing facilities, 139 accepted independent laboratories, and 4

recognized independent laboratories. This rule will impact 16 of these

respondents. We do not expect this rule to reduce the total number of

respondents, because equipment manufacturers may still manufacture

other Coast Guard-approved lifesaving equipment that is not subject to

this rule.

Frequency of Response: The number of responses per year will vary

by requirement. New application materials, instructions, and markings

are required with the initial COA application, and renewal application

materials and markings are required 5 years after the initial

application. Production test records and laboratory inspection records

are required to be kept annually. The Coast Guard estimates this rule

will reduce the number of responses for the following collections of

information, presented in table 37, along with the current estimated

time to complete each collection.

Table 37--Time Burden Estimate by Application Type

------------------------------------------------------------------------

Hours

------------------------------------------------------------------------

New Application.........................................               2

Renewal Applications....................................             0.5

Manufacturer Records....................................            0.17

Packing Instruction Materials...........................             0.1

Markings for New Products...............................             0.1

Marking for Revisions...................................             0.1

Testing Records.........................................               2

Laboratory Inspection Records...........................              24

------------------------------------------------------------------------

In table 38, we estimate the reduction in the number of annual

responses based on application type.

Table 38--Number of Responses Reduced Annually by Application Type

----------------------------------------------------------------------------------------------------------------

Previous

Response type                            iteration of      Change in      Updated ICR

ICR Appendix B      burden        Appendix B

----------------------------------------------------------------------------------------------------------------

New Application.................................................              82               1              81

Renewal Applications............................................             544               6             538

Manufacturer Records............................................           2,715              27           2,688

Packing Instruction Materials...................................         272,200             800         271,400

Markings for New Products.......................................          13,575               5          13,570

Marking for Revisions...........................................         108,600              40         108,560

Testing Records.................................................           1,828               6           1,820

Laboratory Inspection Records...................................           1,828               6           1,820

----------------------------------------------------------------------------------------------------------------

Burden of Response: This rule will not modify the burden of

response for any other existing collections of information.

Estimate of Total Annual Burden: The current ICR estimates the

total annual burden to be 114,586 hours. As a result of this rule, we

estimate the annual burden will be 86,430 hours, for an annual

reduction of 28,156 hours. Together, these changes account for a total

annual reduction in burden of 27,903 hours. These changes are

summarized in table 39.

Table 39--Summary of the Change in Burden

------------------------------------------------------------------------

------------------------------------------------------------------------

Baseline total burden...................................         114,586

Program Changes.........................................         -27,903

Adjustment Changes......................................            -253

Total Changes...........................................         -28,156

Proposed Total Burden...................................          86,430

------------------------------------------------------------------------

This rule is making an adjustment to the current OMB ICR. As

required by 44 U.S.C. 3507(d), we will submit a copy of this rule to

OMB for its review of the collection of information. You are not

required to respond to a collection of information unless it displays a

currently valid OMB control number.

E. Federalism

A rule has implications for federalism under Executive Order 13132

(Federalism) if it has a substantial direct effect on States, on the

relationship between the National Government and the States, or on the

distribution of power and responsibilities among the various levels of

government. We have analyzed this rule under Executive Order 13132 and

have determined that it is consistent with the fundamental federalism

principles and preemption requirements described in Executive Order

13132. Our analysis follows.

It is well settled that States may not regulate in categories

reserved for regulation by the Coast Guard. It is also well settled

that all of the categories regulated under 46 U.S.C. 2103, 3103, 3306,

3703, 4102, 4502, 7101, and 8101 (design, construction, alteration,

repair, maintenance, operation, equipping, personnel qualification, and

manning of vessels), as well as any other category in which Congress

intended the Coast Guard to be the sole source of a vessel's

obligations, are within the field foreclosed from regulation by the

States. See, e.g., United States v. Locke, 529 U.S. 89 (2000) (finding

that the States are foreclosed from regulating tanker vessels), see

also Ray v. Atlantic Richfield Co., 435 U.S. 151, 157 (1978) (State

regulation is preempted where ``the scheme of federal regulation may be

so pervasive as to make reasonable the inference that Congress left no

room for the States to supplement it [or where] the Act of Congress may

touch a field in which the federal interest is so dominant that the

federal system will be assumed to preclude enforcement of state laws on

the same subject.'' (Citations omitted)) Because this rule involves the

design, maintenance, and equipping of vessels; specifically regarding

certain survival craft equipment required to be carried in survival

craft and rescue boats on certain, specified U.S.-flagged vessels, it

relates to vessel standards that are subject to a pervasive scheme of

Federal regulation and is therefore foreclosed from regulation by the

States. Therefore, because the States may not regulate within these

categories, this rule is consistent with the fundamental federalism

principles and preemption requirements described in Executive Order

13132.

F. Unfunded Mandates

The Unfunded Mandates Reform Act of 1995, 2 U.S.C. 1531-1538,

requires Federal agencies to assess the effects of their discretionary

regulatory actions. In particular, the Act addresses actions that may

result in the expenditure by a

[[Page 68303]]

State, local, or tribal government, in the aggregate, or by the private

sector of $100,000,000 (adjusted for inflation) or more in any one

year. Although this rule will not result in such an expenditure, we do

discuss the effects of this rule elsewhere in this preamble.

G. Taking of Private Property

This rule will not cause a taking of private property or otherwise

have taking implications under Executive Order 12630 (Governmental

Actions and Interference with Constitutionally Protected Property

Rights).

H. Civil Justice Reform

This rule meets applicable standards in sections 3(a) and 3(b)(2)

of Executive Order 12988 (Civil Justice Reform) to minimize litigation,

eliminate ambiguity, and reduce burden.

I. Protection of Children

We have analyzed this rule under Executive Order 13045 (Protection

of Children from Environmental Health Risks and Safety Risks). This

rule is not an economically significant rule and will not create an

environmental risk to health or risk to safety that might

disproportionately affect children.

J. Indian Tribal Governments

This rule does not have tribal implications under Executive Order

13175 (Consultation and Coordination with Indian Tribal Governments),

because it will not have a substantial direct effect on one or more

Indian tribes, on the relationship between the Federal Government and

Indian tribes, or on the distribution of power and responsibilities

between the Federal Government and Indian tribes.

K. Energy Effects

We have analyzed this rule under Executive Order 13211 (Actions

Concerning Regulations That Significantly Affect Energy Supply,

Distribution, or Use). We have determined that it is not a

``significant energy action'' under that order because it is not a

``significant regulatory action'' under Executive Order 12866 and is

not likely to have a significant adverse effect on the supply,

distribution, or use of energy.

L. Technical Standards and Incorporation by Reference

The National Technology Transfer and Advancement Act, codified as a

note to 15 U.S.C. 272, directs agencies to use voluntary consensus

standards in their regulatory activities unless the agency provides

Congress, through OMB, with an explanation of why using these standards

will be inconsistent with applicable law or otherwise impractical.

Voluntary consensus standards are technical standards (e.g.,

specifications of materials, performance, design, or operation; test

methods; sampling procedures; and related management systems practices)

that are developed or adopted by voluntary consensus standards bodies.

This rule uses the following voluntary consensus standards: ASTM

F1003-02, ASTM F1014-02, ISO 18813:2006, ISO 25862:2009, and ISO

17339:2018. The sections that reference these standards and the

locations where these standards are available are listed in 46 CFR

160.046-3 and 199.05.

This rule uses technical standards developed by voluntary consensus

standards bodies to meet the stringent equipment requirements for

survival craft and rescue boats on board U.S.-flagged vessels. These

standards provide internationally accepted and recognized parameters

that equipment must meet in order to ensure its safety, proper usage,

and preservation on the seas. The standards being incorporated were

developed by either the ASTM or the ISO, which are voluntary consensus

standard-setting organizations. The sections that reference these

standards and the locations where these standards are available are

listed in 46 CFR parts 160 and 199.

Two ASTM standards will be updated and incorporated by reference in

this rulemaking: (1) ASTM F1003-02 (Reapproved 2007), ``Standard

Specification for Searchlights on Motor Lifeboats'' (2007); and (2)

ASTM F1014-02 (Reapproved 2007), ``Standard Specification for

Flashlights on Vessels'' (2002).

These ASTM standards specify requirements for construction of

searchlights and flashlights (respectively), including materials,

dimensions, performance, and capability. The newer versions of these

standards are not materially different from the previous versions. We

are not updating the third ASTM standard already incorporated in Sec.

199.05, ASTM 93-97, ``Standard Test Methods for Flash Point by Pensky-

Martens Closed Cup Tester.''

The following three ISO standards are incorporated by reference in

this rulemaking:

1. ISO 18813:2006, Ships and marine technology--Survival equipment

for survival craft and rescue boats.

This standard specifies design, performance, and use of various

items of survival equipment carried in survival craft and rescue boats

complying with SOLAS and the LSA Code. It also includes guidelines for

maintenance and periodic inspections by Administrations or ships' crews

for many items.

2. ISO 25862:2009, Ships and marine technology--Marine magnetic

compasses, binnacles and azimuth reading devices.

This standard gives requirements regarding construction and

performance of marine magnetic compasses for navigation and steering

purposes, binnacles, and azimuth reading devices.

3. ISO 17339:2018, Ships and marine technology--Life saving and

fire protection--Sea anchors for survival craft and rescue boats.

This standard specifies requirements for the design, performance,

and prototype testing of sea anchors fitted to survival craft

(liferafts and lifeboats) and rescue boats in accordance with the LSA

Code.

With this rulemaking, we also updated our incorporation by

reference of International Code for the Construction and Equipment of

Ships Carrying Dangerous Chemicals in Bulk (IBC Code), 2016 edition,

and the Amendments to the International Code for the Construction and

Equipment of Ships Carrying Liquefied Gases in Bulk, (IGC Code),

adopted May 22, 2014, to reflect the updated editions. No changes to

the specific referenced material have been made between the older

editions and the more recent editions. The IBC Code provides an

international standard for the safe transport by sea of dangerous and

noxious liquid chemicals in bulk. The purpose of the IGC Code is to

provide an international standard for the safe transport by sea in bulk

of liquefied gases and certain other substances.

The Director of the Federal Register has approved the material in

Sec. Sec.  160.046-3 and 199.05 for incorporation by reference under 5

U.S.C. 552 and 1 CFR part 51. Copies of the material are available from

the sources listed in Sec. Sec.  160.046-3 and 199.05.

Consistent with 1 CFR part 51 incorporation by reference

provisions, this material is reasonably available. Interested persons

have access to it through their normal course of business, may purchase

it from the organization identified in 46 CFR 160.046-3 or 199.05, or

may view a copy by means we have identified in those sections.

M. Environment

We have analyzed this rule under Department of Homeland Security

Management Directive 023-01, Rev. 1, associated implementing

instructions, and Environmental Planning

[[Page 68304]]

COMDTINST 5090.1 (series), which guide the Coast Guard in complying

with the National Environmental Policy Act of 1969 (42 U.S.C. 4321-

4370f), and have made a determination that this action is one of a

category of actions that do not individually or cumulatively have a

significant effect on the human environment. A Record of Environmental

Consideration supporting this determination is available in the docket.

For instructions on locating the docket, see the ADDRESSES section of

this preamble. This rule is categorically excluded under paragraphs

L52, L57, and L58 of Appendix A, Table 1 of DHS Instruction Manual 023-

01-001-01, Rev. 1. Paragraph L52 pertains to regulations concerning

vessel and operation safety standards. Paragraph L57 pertains to

regulations concerning manning, documentation, admeasurements,

inspection, and equipping of vessels. Paragraph L58 pertains to

regulations concerning equipment approval and carriage requirements.

This rule removes the Coast Guard type approval requirement for

some survival craft equipment, and replaces it with the requirement

that the manufacturer self-certify that their equipment complies with a

consensus standard.

List of Subjects

46 CFR Part 121

Communications equipment, Marine safety, Navigation (water),

Passenger vessels.

46 CFR Part 160

Incorporation by reference, Marine safety, Reporting and

recordkeeping requirements.

46 CFR Part 169

Fire prevention, Incorporation by reference, Marine safety,

Reporting and recordkeeping requirements, Schools, Vessels.

46 CFR Part 184

Communications equipment, Marine safety, Navigation (water),

Passenger vessels, Reporting and recordkeeping requirements.

46 CFR Part 199

Cargo vessels, Incorporation by reference, Marine safety, Oil and

gas exploration, Passenger vessels, Reporting and recordkeeping

requirements.

For the reasons discussed in the preamble, the Coast Guard amends

46 CFR parts 121, 160, 169, 184, and 199 as follows:

PART 121--VESSEL CONTROL AND MISCELLANEOUS SYSTEMS AND EQUIPMENT

0

1. The authority citation for part 121 is revised to read as follows:

Authority: 46 U.S.C. 2103, 3306; E.O. 12234, 45 FR 58801, 3 CFR,

1980 Comp., p. 277; DHS Delegation 00170.1, Revision No. 01.2.

0

2. Revise Sec.  121.710 to read as follows:

Sec.  121.710  First-aid kits.

A vessel must carry either a first-aid kit that meets the

requirements in 46 CFR 199.175(b)(10) or a kit with equivalent contents

and instructions. For equivalent kits, the contents must be stowed in a

suitable, watertight container that is marked ``First-Aid Kit''. A

first-aid kit must be easily visible and readily available to the crew.

PART 160--LIFESAVING EQUIPMENT

0

3. The authority citation for part 160 is revised to read as follows:

Authority: 46 U.S.C. 2103, 3103, 3306, 3703, 4102, 4302, and

4502; and DHS Delegation 00170.1, Revision No. 01.2, paragraph

(II)(92)(b).

0

4. Amend Sec.  160.010-3 by revising paragraphs (a)(12)(ii) and

(e)(7)(ii) to read as follows:

Sec.  160.010-3  Inflatable buoyant apparatus.

(a) * * *

(12) * * *

(ii) Knives. One knife, of a type designed to minimize the chance

of damage to the inflatable buoyant apparatus and secured with a

lanyard ready for use near the painter attachment. Any knife may be

replaced with a jackknife meeting the requirements in 46 CFR

199.175(b)(16). In addition, an inflatable buoyant apparatus that is

permitted to accommodate 13 persons or more must be provided with a

second knife that is of the non-folding type;

* * * * *

(e) * * *

(7) * * *

(ii) First-aid kit. A first-aid kit as described in 46 CFR

199.175(b)(10);

* * * * *

Subpart 160.013 [Removed and Reserved]

0

5. Remove and reserve subpart 160.013, consisting of Sec. Sec.

160.013-1 through 160.013-5.

Subpart 160.026 [Removed and Reserved]

0

6. Remove and reserve subpart 160.026, consisting of Sec. Sec.

160.026-1 through 160.026-7.

Subpart 160.041 [Removed and Reserved]

0

7. Remove and reserve subpart 160.041, consisting of Sec. Sec.

160.041-1 through 160.041-6.

Subpart 160.043 [Removed and Reserved]

0

8. Remove and reserve subpart 160.043, consisting of Sec. Sec.

160.043-1 through 106.043-6.

Subpart 160.044 [Removed and Reserved]

0

9. Remove and reserve subpart 160.044, consisting of Sec. Sec.

160.044-1 through 160.044-5.

0

10. Add subpart 160.046, consisting of Sec. Sec.  160.046-1 through

160.046-11, to read as follows:

Subpart 160.046--Emergency Provisions

Sec.

160.046-1 Scope.

106.046-3 Incorporation by reference.

160.046-5 General requirements for emergency provisions.

160.046-7 Independent laboratory.

160.046-9 Manufacturer certification and labeling.

160.046-11 Manufacturer notification.

Sec.  160.046-1  Scope.

This subpart applies to emergency provisions approved to be carried

in lifeboats and liferafts, in accordance with 46 CFR 199.175(b)(22).

Sec.  160.046-3  Incorporation by reference.

(a) Certain material is incorporated by reference into this subpart

with the approval of the Director of the Federal Register under 5

U.S.C. 552(a) and 1 CFR part 51. All approved incorporation by

reference (IBR) material is available for inspection at the Coast Guard

Headquarters. Contact the Coast Guard at: Commandant (CG-ENG-4), U.S.

Coast Guard Stop 7509, 2703 Martin Luther King Jr. Avenue SE,

Washington, DC 20593-7509; email: typeapproval@uscg.mil; website:

www.dco.uscg.mil/CG-ENG-4/. It is also available for inspection at the

National Archives and Records Administration (NARA). For information on

the availability of this material at NARA, email

fr.inspection@nara.gov; website: www.archives.gov/federal-register/cfr/ibr-locations.html. All approved material is available from the

source(s) listed in this section.

[[Page 68305]]

(b) International Organization for Standardization (ISO), Chemin de

Blandonnet 8, CP 401, 1214 Vernier, Geneva, Switzerland; phone: +41 22

749 01 11; email: central@iso.org; web: www.iso.org.

(1) ISO 18813:2006(E), Ships and marine technology--Survival

equipment for survival craft and rescue boats, First edition, April 1,

2006; IBR approved for Sec. Sec.  160.046-5; 160.046-7; 160.046-11.

(2) [Reserved]

Sec.  160.046-5  General requirements for emergency provisions.

Emergency provisions must meet the requirements found in ISO

18813:2006(E) paragraph 4.31 (incorporated by reference, see Sec.

160.046-3).

Sec.  160.046-7  Independent laboratory.

Unless the Commandant directs otherwise, an independent laboratory

accepted by the Coast Guard under 46 CFR part 159, subpart 159.010,

must perform or witness, as appropriate, inspections, tests, and

oversight required by ISO 18813:2006(E) paragraph 4.31 (incorporated by

reference, see Sec.  160.046-3). Approval and production tests of

emergency provisions must be carried out in accordance with the

procedures for independent laboratory inspections in 46 CFR part 159,

subpart 159.007, and in this section unless the Commandant authorizes

alternative tests and inspections. The Commandant may prescribe

additional production tests and inspections necessary to maintain

quality control and to monitor compliance with the requirements of this

subpart.

Sec.  160.046-9  Manufacturer certification and labeling.

(a) Each provision must be certified by the manufacturer as

complying with the requirements of this subpart.

(b) The container should be clearly and permanently marked with:

(1) The name and address of the approval holder;

(2) The U.S. Coast Guard Approval number;

(3) The total food energy value of provisions in the container in

calories and kiloJoules;

(4) The lot number;

(5) The month and year the provision was packed; and

(6) The month and year of expiration (5 years after the date of

packing).

(c) The emergency provision must include waterproof instructions

for use, assuming consumption of 3350 kiloJoules per person per day.

Sec.  160.046-11  Manufacturer notification.

(a) Each manufacturer of emergency provisions approved in

accordance with the specifications of this subpart must send a test

report required by ISO 18813:2006(E) paragraph 4.31.2 (incorporated by

reference, see Sec.  160.046-3) to the Commandant (CG-ENG-4), U.S.

Coast Guard Stop 7509, 2703 Martin Luther King Jr. Avenue SE,

Washington, DC 20593-7509 or email typeapproval@uscg.mil:

(1) With the application for approval;

(2) Every year as long as the manufacturer continues to produce

provisions; and

(3) Each time the contents of the emergency provisions change.

(b) [Reserved]

0

11. Amend Sec.  160.051-11 by revising paragraph (b) to read as

follows:

Sec.  160.051-11  Equipment required for Coastal Service inflatable

liferafts.

* * * * *

(b) Knife. One knife, of a type designed to minimize the chance of

damage to the inflatable liferaft and secured with a lanyard. In

addition, an inflatable liferaft that is permitted to accommodate 13

persons or more must be provided with a second knife that is of the

non-folding type. Any knife may be replaced with a jackknife meeting

the requirements in 46 CFR 199.175(b)(16).

Subpart 160.054 [Removed and Reserved]

0

12. Remove and reserve subpart 160.054, consisting of Sec. Sec.

160.054-1 through 106.054-7.

Subpart 160.061 [Removed and Reserved]

0

13. Remove and reserve subpart 160.061, consisting of Sec. Sec.

160.061-1 through 106.061-5.

0

14. Amend Sec.  160.135-7 by revising paragraph (b)(23) to read as

follows:

Sec.  160.135-7  Design, construction, and performance of lifeboats.

* * * * *

(b) * * *

(23) Bilge pump. Each lifeboat that is not automatically self-

bailing must be fitted with a manual bilge pump that meets the

requirements in 46 CFR 199.175(b)(2). Each such lifeboat with a

capacity of 100 persons or more must carry an additional manual bilge

pump or an engine-powered bilge pump.

* * * * *

0

15. Amend Sec.  160.151-21 by revising paragraphs (b), (h), (o), and

(q) through (s) as follows:

Sec.  160.151-21   Equipment required for SOLAS A and SOLAS B

inflatable liferafts.

* * * * *

(b) Jackknife (IMO LSA Code, as amended by Resolution MSC.293(87),

Chapter IV/4.1.5.1.2). Each folding knife must be a jackknife meeting

the requirements in 46 CFR 199.175(b)(16).

* * * * *

(h) First-aid kit (IMO LSA Code, as amended by Resolution

MSC.293(87), Chapter IV/4.1.5.1.8). Each first-aid kit must meet the

requirements in 46 CFR 199.175(b)(10).

* * * * *

(o) Signalling mirror (IMO LSA Code, as amended by Resolution

MSC.293(87), Chapter IV/4.1.5.1.15). Each signalling mirror must meet

the requirements in 46 CFR 199.175(b)(19).

* * * * *

(q) Fishing tackle (IMO LSA Code, as amended by Resolution

MSC.293(87), Chapter IV/4.1.5.1.17). The fishing tackle must meet the

requirements in 46 CFR 199.175(b)(11).

(r) Food rations (IMO LSA Code, as amended by Resolution

MSC.293(87), Chapter IV/4.1.5.1.18). The food rations must meet the

requirements in 46 CFR 199.175(b)(22).

(s) Drinking water (IMO LSA Code, as amended by Resolution

MSC.293(87), Chapter IV/4.1.5.1.19). Emergency drinking water must meet

the requirements in 46 CFR 199.175(b)(40). The desalting apparatus or

reverse osmosis desalinator must be approved by the Commandant under

approval series 160.058.

* * * * *

0

16. Amend Sec.  160.156-7 by revising paragraph (b)(22) to read as

follows:

Sec.  160.156-7  Design, construction and performance of rescue boats

and fast rescue boats.

* * * * *

(b) * * *

(22) Manual bilge pump. Each rescue boat that is not automatically

self-bailing must be fitted with a manual bilge pump that meets the

requirements in 46 CFR 199.175(b)(2), or an engine-powered bilge pump.

* * * * *

PART 169--SAILING SCHOOL VESSELS

0

17. The authority citation for part 169 is revised to read as follows:

Authority: 33 U.S.C. 1321(j); 46 U.S.C. 3306, 6101; E.O. 11735,

38 FR 21243, 3 CFR, 1971-1975 Comp., p. 793; DHS Delegation 00170.1,

Revision No. 01.2; Sec.  169.117 also issued under the authority of

44 U.S.C. 3507.

0

18. Amend Sec.  169.115 by revising paragraphs (a) and (e) to read as

follows:

[[Page 68306]]

Sec.  169.115   Incorporation by reference.

(a) Certain material is incorporated by reference into this part

with the approval of the Director of the Federal Register in accordance

with 5 U.S.C. 552(a) and 1 CFR part 51. All approved incorporation by

reference (IBR) material is available for inspection at the Coast Guard

Headquarters and at the National Archives and Records Administration

(NARA). Contact the Coast Guard at: Commandant (CG-ENG-4), U.S. Coast

Guard Stop 7509, 2703 Martin Luther King Jr. Avenue SE, Washington, DC

20593-7509; email: typeapproval@uscg.mil; website: www.dco.uscg.mil/CG-ENG-4/. For information on the availability of this material at NARA,

email: fr.inspection@nara.gov; website: www.archives.gov/federal-register/cfr/ibr-locations.html. The material may be obtained from the

source(s) in the following paragraph(s) of this section.

* * * * *

(e) The Textile Color Card Association of the United States, Inc.

200 Madison Avenue, New York. (For availability of this material,

contact the Coast Guard--see paragraph (a) of this section.)

(1) Cable No. 70072, Standard Color Card of America, Ninth edition,

1941 for Sec.  169.529(b).

(2) [Reserved]

* * * * *

0

19. Revise Sec.  169.527 to read as follows:

Sec.  169.527   Required equipment for lifeboats.

(a) All lifeboats must be equipped in accordance with table 1 to 46

CFR 199.175 except as provided in paragraphs (b) and (c) of this

section.

(b) The following equipment must be carried in addition to the

equipment required under 46 CFR 199.175:

(1) Cover;

(2) Ditty bag; and

(3) Mast and sail.

(c) If operating on protected waters, lifeboat equipment need only

to consist of the following:

(1) Boathook--(1);

(2) Bucket--(1);

(3) Fire extinguisher--(2) U.S. Coast Guard-approved Type B:C

(motor propelled lifeboats only);

(4) Hatch--(1);

(5) Lifeline--(1);

(6) Oar unit--(1);

(7) Painter--(1);

(8) Plug--(1);

(9) Oarlock unit--(1); and

(10) Toolkit (motor propelled lifeboats only).

0

20. Revise Sec.  169.529 to read as follows:

Sec.  169.529   Description of lifeboat equipment.

(a) All lifeboat equipment must meet the requirements under 46 CFR

199.175, except as provided in paragraph (b) of this section.

(b) The following equipment, carried in addition to the equipment

required under 46 CFR 199.175, must meet the following requirements:

(1) Cover, protecting. The cover must be of highly visible color

and capable of protecting the occupants against exposure. A cover is

not required for fully enclosed lifeboats.

(2) Ditty bag. The ditty bag must consist of a canvas bag or

equivalent and must contain a sailmaker's palm, needles, sail twine,

marline, and marlin spike, except that motor-propelled lifeboats need

not carry a ditty bag.

(3) Mast and sail. A unit, consisting of a standing lug sail

together with the necessary spars and rigging, must be provided in

accordance with table 1 to this section, except that motor-propelled

lifeboats need not carry a mast or sails. The sails must be of good

quality canvas, or other material acceptable to the Commandant, colored

Indian Orange (Cable No. 70072, Standard Color Card of America;

incorporated by reference, see Sec.  169.115). Rigging must consist of

galvanized wire rope not less than \3/16\-inch in diameter. The mast

and sail must be protected by a suitable cover.

[[Page 68307]]

Table 1 to Sec.   169.529

----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Length of lifeboat, feet                                                           Standing lug sail                                                                 Mast \1\                       Yard \1\

--------------------------------------------------------------------------------------------------------------------------------------------------------------             -------------------------------------------------------------

Luff and head     Leach length       Foot length     Clew to throat                 Commercial       Length                         Length

Area,          lengths     ------------------------------------------------------  Ounces per  designation ------------------  Diameter,  ------------------  Diameter,

Over--                       Not over--  square feet ------------------                                                       square yard     number                         inches                         inches

Feet    Inches    Feet    Inches    Feet    Inches    Feet    Inches                              Feet    Inches                 Feet    Inches

----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

17           58        5       11       12        1        8       10       10       10        14.35           10       11        2            3        6       11            2

17............................................           19           74        6        8       13        8       10        0       12        2        14.35           10       12        6            3        7        8            2

19............................................           21           93        7        5       15        1       11        2       13        8        14.35           10       13       10       3\1/2\        8        5       2\1/2\

21............................................           23          113        8        3       16       11       12        4       15        1        14.35           10       15        2       3\1/2\        9        3       2\1/2\

23............................................           25          135        9        0       18        6       13        6       16        6        14.35           10       16        6            4       10        0            3

25............................................           27          158        9        9       20        0       14        7       17       10        17.50            8       17       10            4       10        9            3

27............................................           29          181       10        5       21        5       15        7       19        1        17.50            8       19        2       4\1/2\       11        5       3\1/4\

29............................................           31          203       11        0       22        8       16        6       20        3        20.74            6       20        6       4\1/2\       12        0       3\1/4\

31 \2\........................................  ...........  ...........  .......  .......  .......  .......  .......  .......  .......  .......  ...........  ...........  .......  .......  ...........  .......  .......  ...........

----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

\1\ Mast lengths measured from heel to center of upper halyard sheave. Mast diameters measured at thwart. Mast and yard shall be of clear-grained spruce, fir, or equivalent.

\2\ Subject to special consideration.

[[Page 68308]]

PART 184--VESSEL CONTROL AND MISCELLANEOUS SYSTEMS AND EQUIPMENT

0

21. The authority citation for part 184 is revised to read as follows:

Authority:  46 U.S.C. 2103, 3306; E.O. 12234, 45 FR 58801, 3

CFR, 1980 Comp., p. 277; DHS Delegation 00170.1, Revision No. 01.2.

0

22. Revise Sec.  184.710 to read as follows:

Sec.  184.710  First-aid kits.

A vessel must carry either a first-aid kit that meets the

requirements in 46 CFR 199.175(b)(10) or a kit with equivalent contents

and instructions. For equivalent kits, the contents must be stowed in a

suitable, watertight container that is marked ``First-Aid Kit''. A

first-aid kit must be easily visible and readily available to the crew.

PART 199--LIFESAVING SYSTEMS FOR CERTAIN INSPECTED VESSELS

0

23. The authority citation for part 199 is revised to read as follows:

Authority: 46 U.S.C. 2103, 3103, 3306, and 3703; and DHS

Delegation 00170.1, Revision No. 01.2, paragraph (II)(92)(b).

0

24. Revise Sec.  199.05 to read as follows:

Sec.  199.05  Incorporation by reference.

Certain material is incorporated by reference in this part with the

approval of the Director of the Federal Register under 5 U.S.C. 552(a)

and 1 CFR part 51. All approved incorporation by reference (IBR)

material is available for inspection at the Coast Guard Headquarters

and at the National Archives and Records Administration (NARA). Contact

the Coast Guard at: Commandant (CG-ENG-4), U.S. Coast Guard Stop 7509,

2703 Martin Luther King Jr. Avenue SE, Washington, DC 20593-7509, email

typeapproval@uscg.mil or visit https://www.dco.uscg.mil/CG-ENG-4/. It

is also available for inspection at the National Archives and Records

Administration (NARA). For information on the availability of this

material at NARA, email: fr.inspection@nara.gov or go to

www.archives.gov/federal-register/cfr/ibr-locations.html. The material

may be obtained from the following source(s):

(a) ASTM International (ASTM). 100 Barr Harbor Drive, P.O. Box

C700, West Conshohocken, PA 19428-2959; phone: (610) 832 9500; email

service@astm.org; web: www.astm.org.

(1) ASTM D 93-97, Standard Test Methods for Flash-Point by Pensky-

Martens Closed Cup Tester, approved July 10, 1997; IBR approved for

Sec. Sec.  199.261; 199.290.

(2) ASTM F1003-02 (Reapproved 2007), Standard Specification for

Searchlights on Motor Lifeboats, approved May 1, 2007; IBR approved for

Sec.  199.175.

(3) ASTM F1014-02 (Reapproved 2007), Standard Specification for

Flashlights on Vessels, approved May 1, 2007; IBR approved for Sec.

199.175.

(b) International Maritime Organization (IMO). Publications

Section, 4 Albert Embankment, London, SE1 7SR, United Kingdom; phone:

+44 (0)20 7735 7611; email: info@imo.org; web: www.imo.org.

(1) IBC Code, International Code for the Construction and Equipment

of Ships Carrying Dangerous Chemicals in Bulk, 2016 edition, copyright

2016, Chapter 2 Ship survival capability and location of cargo tanks;

IBR approved for Sec.  199.280.

(2) IBC Code, International Code for the Construction and Equipment

of Ships Carrying Dangerous Chemicals in Bulk, 2016 edition, copyright

2016, Chapter 17 Summary of minimum requirements; IBR approved for

Sec.  199.30.

(3) MSC Circular 699, Revised Guidelines for Passenger Safety

Instructions, issued July 17, 1995, IBR approved for Sec.  199.217.

(4) Resolution A.520(13), Code of Practice for the Evaluation,

Testing and Acceptance of Prototype Novel Life-saving Appliances and

Arrangements, adopted November 17, 1983; IBR approved for Sec.  199.40.

(5) Resolution A.657(16), Instructions for Action in Survival

Craft, adopted October 19, 1989; IBR approved for Sec.  199.175.

(6) Resolution A.658(16), Use and Fitting of Retro-reflective

Materials on Life-saving Appliances, adopted October 19, 1989; IBR

approved for Sec. Sec.  199.70; 199.176.

(7) Resolution A.760(18), Symbols Related to Life-saving Appliances

and Arrangements, adopted November 4, 1993, IBR approved for Sec. Sec.

199.70; 199.90.

(8) Resolution MSC.370(93), Amendments to the International Code

for the Construction and Equipment of Ships Carrying Liquefied Gases in

Bulk, (IGC Code), adopted May 22, 2014; IBR approved for Sec. Sec.

199.30; 199.280.

(c) International Standard Organization (ISO). Chemin de Blandonnet

8, CP 401, 1214 Vernier, Geneva, Switzerland; phone: +41 22 749 01 11;

email: central@iso.org; web: www.iso.org.

(1) ISO 17339:2018(E), Ships and marine technology--Life saving and

fire protection--Sea anchors for survival craft and rescue boats,

Second edition, July 2018; IBR approved for Sec.  199.175.

(2) ISO 18813:2006(E), Ships and marine technology--Survival

equipment for survival craft and rescue boats, First edition, April 1,

2006; IBR approved for Sec.  199.175.

(3) ISO 25862:2009(E), Ships and marine technology--Marine magnetic

compasses, binnacles and azimuth reading devices, First edition, May

15, 2009; IBR approved for Sec.  199.175.

Sec.  199.30   [Amended]

0

25. Amend Sec.  199.30 in the definition for ``Toxic vapor or gas'' as

follows:

0

a. Remove the text ``IBC Code'' and add, in its place, the text ``IBC

Code; incorporated by reference, see Sec.  199.05''; and

0

b. Remove the text ``IGC Code'' and add, in its place, the text ``IGC

Code; incorporated by reference, see Sec.  199.05''.

0

26. Amend Sec.  199.175 as follows:

0

a. In paragraph (a)(4), remove the word ``and'';

0

b. Redesignate paragraph (a)(5) as paragraph (a)(6);

0

c. Add new paragraph (a)(5);

0

d. In the introductory text to paragraph (b), remove the text ``table

199.175 of this section'' and add, in its place, the text ``table 1 to

this section'';

0

e. Revise the introductory text to paragraph (b)(2), paragraphs (b)(5),

(6), (9) through (13), (16), (17), and (19), and (b)(27)(i);

0

f. In paragraph (b)(28)(i), remove the text ``F 1003'' and add, in its

place, the text ``F1003'';

0

g. Revise paragraph (b)(40) introductory text;

0

h. Redesignate paragraphs (b)(40)(i) and (ii) as paragraphs

(b)(40)(iii) and (iv);

0

i. Add new paragraphs (b)(40)(i) and (ii);

0

j. In newly-redesignated paragraph (b)(40)(iv), remove the words

``reverse osmosis'' and add, in their place, the text ``reverse-

osmosis'';

0

k. Add paragraph (c) immediately before table 199.175;

0

l. Designate table 199.175 as table 1 to Sec.  199.175;

0

m. In newly-designated table 1 to Sec.  199.175, revise entries 5 and

17; and

0

n. Add footnote 11 to the footnotes following table 1 to Sec.  199.175.

The revisions and additions read as follows:

Sec.  199.175   Survival craft and rescue boat equipment.

(a) * * *

(5) Must be marked with either the Coast Guard approval number or

the standard that the product meets, as applicable; and

* * * * *

[[Page 68309]]

(b) * * *

(2) Bilge pump. The bilge pump must meet the requirements in ISO

18813:2006(E) paragraph 4.3 (incorporated by reference, see Sec.

199.05) and must be installed in a ready-to-use condition.

* * * * *

(5) Can opener. A can opener must meet the requirements in ISO

18813:2006(E) paragraph 4.43 (incorporated by reference, see Sec.

199.05). A can opener may be in a jackknife meeting the requirements in

paragraph (b)(16) of this section.

(6) Compass. The compass and its mounting arrangement must meet the

requirements in ISO 18813:2006(E) paragraph 4.6 (incorporated by

reference, see Sec.  199.05).

(i) In a totally enclosed lifeboat, the compass must be permanently

fitted at the steering position; in any other boat it must be provided

with a binnacle, if necessary, to protect it from the weather, and with

suitable mounting arrangements.

(ii) The compass must be tested in accordance with the provisions

in ISO 25862:2009(E) Annex H (incorporated by reference, see Sec.

199.05) by an independent laboratory accepted by the Coast Guard in

accordance with part 159, subpart 159.010, of this chapter.

* * * * *

(9) Fire extinguisher. The fire extinguisher must be approved under

approval series 162.028. The fire extinguisher must have a rating of a

40-B:C. Two 10-B:C extinguishers may be carried in place of a 40-B:C

extinguisher. Extinguishers with larger numerical ratings or multiple

letter designations may be used instead.

(10) First-aid kit. Each first-aid kit must meet the requirements

in ISO 18813:2006(E) paragraph 4.12 (incorporated by reference, see

Sec.  199.05).

(i) A first-aid kit may be considered acceptable if it meets all of

the requirements of ISO 18813:2006(E) paragraph 4.12, except that it

does not contain the burn preparations. It must be clearly marked on

the first-aid kit that it does not include the burn preparations.

(ii) The active ingredients in medicinal products must conform to

over-the-counter (OTC) drug regulations set out in 21 CFR part 330.

(11) Fishing kit. The fishing kit must meet the requirements in ISO

18813:2006(E) paragraph 4.13 (incorporated by reference, see Sec.

199.05).

(12) Flashlight. The flashlight must be a type I or type III that

is constructed and marked in accordance with ASTM F1014 (incorporated

by reference, see Sec.  199.05). One spare set of batteries and one

spare bulb, stored in a watertight container, must be provided for each

flashlight.

(13) Hatchet. The hatchet must be suitable for cutting a rope

towline or painter in an emergency and must not require assembly or

unfolding.

(i) The hatchet must be at least 14 inches in length and have a

cutting edge of approximately 3\1/4\ inches in length, with a hardened

steel or equivalent alloy head.

(ii) The hatchet must be provided a lanyard at least 3 feet in

length.

(iii) The hatchet must be stowed in brackets near the release

mechanism and, if more than one hatchet is carried, the hatchets must

be stowed at opposite ends of the boat.

* * * * *

(16) Jackknife. The jackknife must consist of a one-bladed knife

fitted with a can opener and attached to the boat by its lanyard. The

jackknife must meet the requirements in ISO 18813:2006(E) paragraph

4.19 (incorporated by reference, see Sec.  199.05).

(17) Knife. The knife must be of the non-folding type with a

buoyant handle as follows:

(i) The knife for a rigid liferaft must be secured to the raft by a

lanyard and stowed in a pocket on the exterior of the canopy near the

point where the painter is attached to the liferaft. If an approved

jackknife is substituted for the second knife required on a liferaft

equipped for 13 or more persons, the jackknife must also be secured to

the liferaft by a lanyard.

(ii) The knife in an inflatable or rigid-inflatable rescue boat

must be of a type designed to minimize the possibility of damage to the

fabric portions of the hull.

(iii) Any knife may be replaced with a jackknife meeting the

requirements in paragraph (b)(16) of this section.

* * * * *

(19) Mirror. The signalling mirror must meet the requirements in

ISO 18813:2006(E) paragraph 4.23 (incorporated by reference, see Sec.

199.05).

* * * * *

(27) * * *

(i) The sea anchor for a lifeboat, rescue boat, and rigid liferaft

must meet the requirements in ISO 17339:2018(E) (incorporated by

reference, see Sec.  199.05).

* * * * *

(40) Water. The water must meet the requirements in ISO

18813:2006(E) paragraph 4.46 (incorporated by reference, see Sec.

199.05).

(i) The water must meet the U.S. Public Health Service ``Drinking

Water Standards'' in 40 CFR part 141 to suitably protect the container

against corrosion. After treatment and packing, the water must be free

from organic matter, sediment, and odor. It must have a pH between 7.0

and 9.0 as determined by means of a standard pH meter using glass

electrodes. Water quality must be verified by the local municipality or

independent laboratory accepted by the Coast Guard in accordance with

part 159, subpart 159.010, of this chapter.

(ii) Containers of emergency drinking water must be tested in

accordance with the provisions in ISO 18813:2006(E) by an independent

laboratory accepted by the Coast Guard in accordance with part 159,

subpart 159.010, of this chapter.

* * * * *

(c) Any Coast Guard-approved equipment on board before December 14,

2022 may remain on board as long as it remains in good and serviceable

condition.

Table 1 to Sec.   199.175--Survival Craft Equipment

--------------------------------------------------------------------------------------------------------------------------------------------------------

International voyage                         Short international voyage

-----------------------------------------------------------------------------------------------

Item No.                       Item                               Rigid                                           Rigid

Lifeboat        liferaft       Rescue boat      Lifeboat        liferaft       Rescue boat

(SOLAS A pack)                                  (SOLAS B pack)

--------------------------------------------------------------------------------------------------------------------------------------------------------

* * * * * * *

5.................................  Can opener \11\.....               3               3  ..............               3  ..............  ..............

[[Page 68310]]

* * * * * * *

17................................  Knife \1\ \4\ \11\..               1               1               1               1               1               1

* * * * * * *

--------------------------------------------------------------------------------------------------------------------------------------------------------

Notes:

\1\ Each liferaft equipped for 13 persons or more must carry two of these items.

* * * * *

\4\ A hatchet counts towards this requirement in rigid rescue boats.

* * * * *

\11\ One (1) jackknife may replace one (1) can opener and one (1) knife.

Sec.  199.280   [Amended]

0

27. Amend Sec.  199.280 in paragraphs (e)(2) and (3) by removing the

words ``in Bulk'' and adding, in their place, the text ``in Bulk

(incorporated by reference, see Sec.  199.05)''.

Dated: October 26, 2022.

W.R. Arguin,

Rear Admiral, U.S. Coast Guard, Assistant Commandant for Prevention

Policy.

[FR Doc. 2022-23666 Filed 11-10-22; 8:45 am]

BILLING CODE 9110-04-P